Z88BAS.TXT -- BY CHRIS FENN

---------------------------

Here is a list of Z88 BBC BASIC Commands Statements and functions, I've

commented on some to help clarify. Abbreviations are listed in

brackets, they can be typed in as such to save memory.

COMMANDS:

--------

AUTO [AU.] line numbering

DELETE [DEL.]

LIST [L.]

LISTO controls the appearance of a

listed program (4 options)

LOAD [LO.]

NEW

OLD recover program after using

the "NEW" command.

RENUMBER [REN.]

TRACE [TR.]

STATEMENTS:

-----------

BPUT# [BP.#] output a byte.

CALL [CA.] call machine code

with paremeters

CHAIN [CH.]

CLEAR [CL.]

CLOSE# [CL.#]

CLS

DATA [D.]

DEF define function or procedure.

DIM (supports integer variables)

ELSE [EL.]

END

ENDPROC

FN

FOR [F.]

GOSUB [GOS.]

GOTO [G.]

IF

INPUT [I.]

INPUT LINE

INPUT# input from file.

LET

LOCAL [LOC.]

NEXT [N.]

ON

ON ERROR

OPT assembler options with four

different options.

OSCLI operating system command.

Allows string expression to be passed.

PRINT [P.] also controls field width,

sci. notation,decimal places etc.

PRINT# [P.#] write to file

PROC (PROCEDURE)

PUT output to port

READ

REM

REPEAT [REP.]

REPORT [REPO.] report error

RESTORE [RES.]

RETURN [R.]

RUN does not clear A%-Z%

SAVE [SA.]

SPC print spaces

STEP [S.]

STOP

TAB TAB(X) or TAB(X,Y)

THEN [TH.]

TO

UNTIL [U.]

VDU [V.] output bytes to screen

WIDTH [W.]

FUNCTIONS:

----------

ABS

ACS arc cosine

ASC ASCII value.

ASN arc sine

ATN arc tangent

BGET# [B.#] byte from data file.

CHR$

COS cosine

COUNT [COU.] returns the number of chars. sent to

output stream since the last new line.

DEG degrees

EOF#

ERL error line.

ERR error code.

EVAL [EV.] evaluate string. example:A=10:B=20

PRINT EVAL ("A+B")/would return 30.

EXP exponent

EXT# extent of file. Total length of file whose

channel number is its argument.

FALSE [FA.]

GET/GET$ wait for key.

HIMEM [H.] first byte of free memory.

INKEY/INKEY$

INSTR

INT

LEFT$

LEN

LN natural logarithem

LOG logarithem

LOMEM [LOM.] lower memory bound.

MID$

OPENIN [OP.] open file for output.

OPENOUT open file for output.

OPENUP [OPENU.] open file for update.

PAGE [PA.] program area. Can be altered to

allow several progs. to be in memory at once.

PI

POS horizontal cursor position.

PTR# file pointer.

RAD radians.

RIGHT$

RND with optional paremeters.

SGN

SIN sine

SQR

STR$

STRING$ repeats strings. A$=STRING$(2,"Z88")

returns "Z88Z88"

TAN [T.] tangent

TIME [TI.] elapsed time clock

TIME$ EXAMP:Sunday April 3,1988,10:17:30 AM

TOP returns the value of the first free location

after the end of the current program.

TRUE

USR call machine code at specified address.Passes

the least sig.bytes of the INT. variables A%,B%,C%

D%,E%,H%,L% and F% into corresponding registers of

the Z80 on entry.

VAL value of string

VPOS [VP.] vertical  cursor position

OPERATORS:

----------

+,*,^,-,/

DIV integer divide

MOD modulo

RELATIONAL OPERATORS:

---------------------

<,<=,<>,=,>,>=

LOGICAL OPERATORS:

------------------

AND [A.]

EOR

NOT

OR

The equivalent of PEEK & POKE on the Z88 is handled by the use of

indirection operators using VECTORS. (VEC)

INDIRECTION OPERATORS include:

? byte indirection

! word indirection

$ string indirection

For you math people. Here is the output of a program I found in

TI PPC NOTES, it was converted to run on a Tandy Model 100.

Hopefully I translated it correctly to run on the Z88.

it checks the math operations of the computer.

RADIX= 2

PRECISION= 32

FPWIDTH= 4.2949673E9

CLOSEST RELATIVE SEPARATION FOUND IS UlpOne= 2.32830644E-10

ADD/SUBTRACT HAS A GUARD DIGIT AS IT SHOULD.

OTHER NOTES:

------------

1/3 returns .333333333           1=1 returns -1

Empty loop to 10000 takes 13.5 secs. to execute.

Empty INTEGER loop to 10000 takes 12.5 secs. to execute.

I've found it to be roughly as fast as a 4.77 MZ PC. but the Z88 is

more accurate.

I've had pretty good luck loading M/S BASIC programs & running some

programs unchanged. The W/P in the Z88 with its fast global search &

replace makes it much easier to convert programs when you have to.

You can route output to the printer at anytime by pressing the

square key (Alt.),+,& P. Square -P turns it off. You can send the

output to a RAM file by pressing the square key +S, to turn it off

would be square key-S.

The screen size in Z88 BBC BASIC is 8 lines by 94 columns.

I will later upload screen codes to turn on BOLD,UNDERLINE,FLASH etc.

(not supplied in the manual)

Graphic commands are not supported from BASIC,such as PLOT,LINE,DRAW

but I'm sure someone will be able to access the ROM routines.

Sound (not much) is in the form of PRINT CHR$(7) (bell code).

The only special codes you need to tack on to an ASCII file to load

into BASIC are: #B & .J, the #B routes the stream to the BASIC area,

the .J tells it to "JAM" any commands while it's loading. You may also

include "NEW" to the sequence to delete the old program first.

The Z88 includes CLI files (Command Line Interpreter). Using special

codes you can build batch/macro files that can act on all the features

of the Z88, including unattended operations.

A few examples:"#"=the square key,"|"=the diamond key,.T> creates

output file etc.

You can have many BASIC programs ACTIVE at the same time (memory

providing) switching between them by pressing square key"B",or by

pressing the index key and choosin��which one you want. Actually you

can have many documents,programs and worksheets ACTIVE switching

between them in a like manner.

I am no expert on BASIC but if there is something you would like

me to try on the Z88 I'll try to accomodate.

-------------------

C.FENN [LOGON LOGAN]

Member of the TIMEX/SINCLAIR USERS GROUP of Las Vegas,&

Member of the Las Vegas PC USERS GROUP.

#

FN

FOR [F.]

GOSU