THE 'RESET' RESTART

This is the normal 'reset'. Addresses 1019 and 101A contain zero

so the returnaddress is 0, the 'restart'-routine in the Spectrum

ROM.

0000 RESET     DI

LD   SP,+1019          Stack 0000 and  return  to

JP   1748,PAGE_OUT     the Spectrum ROM.

DEFB +FF

THE 'SHADOW_ROM ERROR' RESTART

The Discovery-ROM error handler. It takes a one-byte immediate

error code.

0008 SH_ERROR  JP   0168,CONTSIG

THE 'ENTRY_1' ROUTINE

This is the entry point when coming from a RST 0008-instruction

when the Spectrum ROM is paged in. When a RST  0008  is  encoun-

tered,  the  instruction  'LD  HL,(CH_ADD)'  is  executed,   the

Discovery ROM is  paged  in,  and  the  execution  continues  at

address 000B.

000B ENTRY_1   POP  HL

PUSH AF

JP   007A,ENTRY_2      Jump forward.

THE 'CALL A SPECTRUM ROM SUBROUTINE' RESTART

This restart calls a subroutine in the Spectrum ROM. The address

of the subroutine should be put immediately after the  RST  0010

(or CALL 0010) instruction.

0010 CAL_SPEC  EX   (SP),HL           Pass HL to  IY  and  fetch

POP  IY                the returnaddress in HL.

PUSH DE                Store the old value of DE.

LD   E,(HL)            Read the required  address

INC  HL                in DE. (continues forward)

JR   0067,CAL_SPEC2

THE 'RUNNING OR CHECKING SYNTAX ?' RESTART

Returns the zero flag set if the Spectrum is currently  checking

syntax rather than running a statement.

0018 TEST_RUN  BIT  7,(FLAGS)

RET

DEFB +FF

THE 'PRINT THE COPYRIGHT MESSAGE' ROUTINE

001E (C)_MSG   RST  0008,SH_ERROR     Report

DEFB +47               (c) Dave Corney 1984-86

THE 'NEXT CHARACTER' RESTART

Identical to the Spectrum RST 0020. Gets the next character from

the current BASIC-line.

0020 NEXT_CHAR RST  0010,CAL_SPEC     Call Spectrum ROM.

DEFW +0020,NEXT_CHAR

RET

DEFB +FF, +FF, +FF     Unused locations.

DEFB +FF

THE 'FLOATING POINT CALCULATOR' RESTART

Identical to the Spectrum ROM RST 0028,  except  that  a  length

byte should be inserted as first offset (nr. of offsets + 2).

0028 FP_CALC   POP  HL                Fetch the returnaddress in

LD   C,(HL)            HL and the lengthbyte in C

LD   B,+00

PUSH HL                Restack returnaddress

JP   0648,FP_CALC_2

THE 'LOOKUP' RESTART

This is the most important restart. It gives access to  all  the

tables which the Discovery uses. It should be called  by loading

the table-entry in the B-register, and placing a 1-byte immedia-

te table-number after the RST 0030.

0030 LOOKUP    POP  HL                Read the  table-number  in

LD   C,(HL)            the C-register and restore

INC  HL                the Program Counter.

PUSH HL

JP   05B1,LOOKUP_2     Jump to continue.

THE 'MASKABLE INTERRUPT' ROUTINE

It behaves exactly the same as the Spectrum's RST 0038.

0038 MASK_INT  PUSH IY                Call the Spectrum ROM, but

RST  0010,CAL_SPEC     also preserve the IY-regis

DEFW 0038,MASK_INT     ter.

POP  IY

RET

DEFB +FF, +FF, +FF     Unused locations.

DEFB +FF, +FF, +FF

DEFB +FF, +FF

THE 'KEY_INT' ROUTINE

This routine initialises the IC 6116's RAM  and  continues  with

the KEY_INT subroutine in the Spectrum ROM at address +004A.

0048 KEY_INT   PUSH BC                Prepare the stack for  the

PUSH HL                KEY_INT subroutine.

PUSH DE

PUSH BC

PUSH AF

LD   A,(3001)

RRCA                   Is  IC  6116  present  and

CALL NC,1554,INIT_RAM2 initialised? Try to initia

lise the RAM if not.

RST  0030,LOOKUP       Lookup the address to page

DEFB +0E               out and/or return.

CALL 0087,CALL_JP      Perform that operation.

POP  AF                Restore the stack.

POP  BC

POP  DE

LD   HL,+0049          The address in  the  Spec-

EX   (SP),HL           trum ROM. Store it.

JP   1748,PAGE_OUT     Make an indirect jump  via

the PAGE_OUT subroutine.

THE 'NON-MASKABLE INTERRUPT' ROUTINE

This is the address the program is forced to call during a NMI.

In the Spectrum ROM  the  routine  contains  a  'bug',  but  the

Discovery uses it to call the hardware directly.  On  entry  the

HL-reisterpair contains the address of the appropriate  service-

routine (load, save or format).

0065 INT_ALL   RST  0038,MASK_INT     This address is called  if

both interrupttypes should

be taken care of.

0066 NMI       JP   (HL)

THE 'CALL A SPECTRUM ROM ROUTINE (2)' ROUTINE

The continuation of the CAL_SPEC restart at 0010. The  stack  is

set up to make sure that the program resumes  its  execution  at

the address following the DEFW-instruction. The  return  address

is stacked, followed by the PAGE_IN address and the ROM  address

to be called.

0067 CAL_SPEC2 LD   D,(HL)            Take higher part of DEFW.

INC  HL                Move to the return address

EX   (SP),HL           Stack return address.

EX   DE,HL             Pass the ROM address to HL

PUSH HL                and stack it too.

LD   HL,+1708          Exchange with PAGE_IN.

EX   (SP),HL

PUSH IY

EX   (SP),HL           Exchange again.

LD   IY,+5C3A          Restore IY.

JP   1748,PAGE_OUT     Call the required  routine

indirectly.

THE 'ENTRY_2' ROUTINE

The HL register-pair points to the address of the error-code  or

hook-code (possibly in the Spectrum ROM). It must be fetched and

the hook-code table is searched. Finally a jump if made  to  the

correct routine.

007A ENTRY_2   RST  0010,CAL_SPEC     Fetch err-nr or hook-code.

DEFW +007B,TEMP_PTR2+3

INC  HL                HL := returnaddress.

EX   (SP),HL           Stack HL; HL := AF.

PUSH HL                Stack AF.

PUSH DE                Store DE temporarely.

LD   B,+FF             Signal: no subtables.

RST  0030,LOOKUP       Lookup the startaddress in

DEFB +00               the hook-code table.

POP  DE

POP  BC                BC := AF.

0087 CALL_JP   JP   (HL)              Jump to the routine.

THE 'VERSION' HOOKCODE

The version number of this Discovery ROM is put on the  calcula-

tor stack.

0088 VERSION   RST  0028,FP_CALC      Call the calculator.

DEFB +09               (lengthbyte)

DEFB +34,stk-data      2.2

DEFB +F2, exponent +82

DEFB +0C, +CC, +CC, +CD

DEFB +38,end-calc      2.2

JR   009B,RET_SPEC1    Jump to page out.

THE 'LENGTH' HOOKCODE

The length of the current channel is calculated. The same as USR

432d.

0093 LENGTH_1  LD   IX,(CURCHL)       Fetch the startaddress.

CALL 03D9,LEN_CHAN     Calculate the length.

POP  HL                Unstack unwanted addresses

009B RET_SPEC1 POP  HL

JR   00CB,RET_SPEC2    Return to Spectrum ROM.

THE 'INITIALISE RAM' HOOKCODE

The IC 6116 is initialised. The same as USR 14070d.

009E INIT_RAM  DI                     No interrupts allowed.

CALL 1554,INIT_RAM2    Initialise IC 6116.

EI

JR   00CB,RET_SPEC2    Return to Spectrum ROM.

THE 'CHARACTER I/O' HOOKCODE

The A register is used for input or output.  DE  points  to  the

appropriate address in the channel descriptor, so the offset  is

0 for output and 2 for input. The offset is used as an index  in

the subtable for that channel, and a jump is made to the correct

routine.

00A5 CHAR_IO   RES  3,(TV-FLAG)       Signal: mode not changed.

PUSH BC                BC contains AF.

LD   HL,(CURCHL)       HL := current channel.

PUSH HL                Copy HL to IX.

POP  IX

LD   A,(IX+04)         A := channel name.

OR   +20               Force to lower case.

EX   DE,HL             Switch pointers.

SCF

SBC  HL,DE             HL := HL - DE - 1.

LD   B,L               Output if B=0, input if 2.

RST  0030,LOOKUP       Lookup the startaddress in

DEFB +04               channel-info table.

POP  AF                Restore AF.

CALL 0087,CALL_JP      Call the current routine.

THE 'INPUT EXTRA CHARACTERS' ROUTINE

If the CHAR_IO routine has been called  by  the  INPUT  command,

then continue to read or write the rest of the  string  until  a

<CR> is encountered. Else, the routine was used  by  the  INKEY$

#stream command and no extra characters may be read.

00C0 INPUT_EX  POP  HL                Unstack unwanted addresses

POP  HL

EXX

PUSH AF

BIT  5,(FLAG-X)        INPUT or INKEY$ mode?

JR   NZ,00CE,INPUT_EX2 INPUT? Then continue.

POP  AF

00CB RET_SPEC2 JP   1748,PAGE_OUT     Jump to Spectrum ROM.

00CE INPUT_EX2 POP  AF                Read or write successfull?

JR   NC,00CB,RET_SPEC2 Exit if not.

CP   +0D               Is it a <CR>?

JR   Z,00DD,INPUT_CR   Step if so.

RST  0010,CAL_SPEC     Call ADD_CHAR  to  process

DEFW +0F85,ADD_CHAR    this character.

CALL 0623,NEAT_RET     Jump to ED_LOOP,  enter  a

DEFW +0F38,ED_LOOP     loop.

00DD INPUT_CR  CALL 0623,NEAT_RET     Jump to  ED_ENTER,  finish

DEFW 1025,ED_ENTER     the loop.

THE 'MDRIVE' HOOKCODE

If the microdrive system variables are not present already, then

they are created. However, no extra channels may be created.

00E2 M_DRIVE   LD   HL,(CHANS)        HL := start of channels.

LD   DE,+5CB6          DE := start of MD maps.

AND  A                 Test  if  the   microdrive

SBC  HL,DE             system variables are  pre-

JR   NZ,009B,RET_SPEC1 sent. Exit if so.

00ED M_DRIVE2  EX   DE,HL             HL := start of MD maps - 1

DEC  HL

LD   BC,+003A          Reserve 58 bytes.

RST  0010,CAL_SPEC     Make the room.

DEFW +1655,MAKE_ROOM

JR   009B,RET_SPEC1    Jump to Spectrum ROM.

THE 'HOOKCODE ERROR' ROUTINE

If it is not a Spectrum error, then report i is given.

00F7 HOOKERROR CP   +1B               Is it not a Spectrum error

JR   C,00FD,HOOKERR2   then skip.

00FB REPORT_i  RST  0008,SH_ERROR     Report 'Hookcode error'.

DEFB +31

00FD HOOKERR2  LD   (ERR-NR),A        Store the error-code.

THE 'BASIC ERROR' HOOKCODE

If it is an error generated by an extended command,  then  clean

up the memory and the stack and jump to the correct  error-hand-

ling routine.

0100 BASIC_ERR LD   HL,(CH-ADD)       Store the address  of  the

LD   (X-PTR),HL        current character.

BIT  7,(ERR-NR)        Did an error occur?

LD   (ERR-NR),A        (Store the error-code)

JR   Z,016D,CONTSIG_2  then continue signalling.

BIT  7,(SUBPPC)        Statementnr > 127?

JR   NZ,016D,CONTSIG_2 then signal the error.

0115 BAS_ERR2  LD   DE,(WORKSP)       DE := end of E-LINE + 1

AND  A

SBC  HL,DE             Current character past end

ADD  HL,DE             of the line?

JR   NC,016D,CONTSIG_2 then signal the error.

LD   DE,(E-LINE)

AND  A

SBC  HL,DE             Current character in edit-

JR   C,0131,BAS_ERR4   buffer? step if not.

0128 BAS_ERR3  RST  0010,CAL_SPEC     Get the line-number in the

DEFW +19FB,E_LINE_NO   edit-buffer.

LD   HL,(CH-ADD)       Prepare HL and step.

DEC  HL

JR   013E,BAS_ERR5

0131 BAS_ERR4  LD   HL,(PPC)          Fetch current line-number.

BIT  7,H               Give an error if past  the

JR   NZ,016D,CONTSIG_2 program area.

RST  0010,CAL_SPEC     Get  the  startaddress  of

DEFW +196E,LINE_ADDR   this BASIC line.

INC  HL                Make  HL  point  one  byte

INC  HL                before the first character

INC  HL                of the line.

013E BAS_ERR5  LD   D,(SUBPPC)        Find the  startaddress  of

LD   E,+00             the current statement.

RST  0010,CAL_SPEC

DEFW +198B,EACH_STMT

RST  0018,TEST_RUN     Running or checking syntax

JR   NZ,014C,BAS_ERR6  Step if running.

RST  0010,CAL_SPEC     Remove the hidden floating

DEFW +11A7,REMOVE_FP   point forms in the line.

014C BAS_ERR6  RST  0010,CAL_SPEC     Clear the working areas.

DEFW +16BF,SET_WORK

RST  0020,NEXT_CHAR    Fetch the keyword in A.

LD   B,+FF             Signal: no subtables.

RST  0030,LOOKUP       Lookup the startaddress of

DEFB +02               the service routine in the

keyword table.

PUSH HL                Store it  while  preparing

RST  0020,NEXT_CHAR    CH-ADD to point to the 1st

POP  HL                parameter.

CALL 0087,CALL_JP      Call the service routine.

Open the 'S' channel and continue with the next statement.

015A OPEN_'S'  LD   A,+FE             Open the 'S' channel.

RST  0010,CAL_SPEC

DEFW +1601,CHAN_OPEN

LD   (ERR-NR),+FF      Reset ERR-NR.

CALL 0623,NEAT_RET     Return to the Spectrum ROM

DEFW +1B76,STMT_RET    continue the program.

THE 'CONTINUE SIGNALLING OF AN ERROR' ROUTINE

All errors can be signalled. The error-code is fetched and set.

0168 CONTSIG   POP  HL                HL points  to  the  error-

LD   A,(HL)            code. Fetch it  and  store

LD   (ERR-NR),A        it.

THE 'CONTSIG_2' ROUTINE

Enter  here  when  coming  from  the  service  routines  of  the

different commands. On entry ERR-NR holds the code of the  error

to be reported. When an error occurs, a direct jump is  made  to

this address, so no error-code has to be  fetched.  A  check  is

made whether it was an edit-error or a runtime-error. If it  was

an edit-error then a jump is made to the address that is pointed

to by ERR-SP. A third check is  made  for  an  error-routine  at

address 5B1D, that is only present in 128K mode. This way  BASIC

extensions such as Beta-Basic should still be compatible.

016D CONTSIG_2 RES  3,(TVFLAG)        Signal: mode not changed.

RST  0010,CAL_SPEC     Clear the calculator stack

DEFW +16C5,SET_STK

0174 CONTSIG_3 LD   SP,(ERR-SP)       Restore the stackpointer.

POP  DE                Fetch the error-address.

LD   HL,+107F          This is ED_ERROR.

AND  A                 Test if it  was  an  edit-

SBC  HL,DE             error.

JR   NZ,0187,CONTSIG_4 Skip if not.

POP  HL                Fetch the error-address.

LD   (ERR-SP),HL       Prepare  to   check   this

JR   0174,CONTSIG_3    address and loop back.

0187 CONTSIG_4 PUSH DE                Restack the error-address.

LD   HL,+1303          This is MAIN_4.

AND  A                 Test if it was a  runtime-

SBC  HL,DE             error.

JR   Z,019E,CONTSIG_5  Jump if so.

BIT  4,(FLAGS)         Test if in 128K mode.

JR   Z,01BF,CONTSIG_6  Step out if not.

LD   HL,+5B1D          Test for an  error-routine

AND  A                 from this address.

SBC  HL,DE

JR   NZ,01BF,CONTSIG_6 Step out if not.

019E CONTSIG_5 LD   A,(ERR-NR)        Fetch the error-code.

PUSH AF                Store it temporarely.

CP   +57               Was it 'RAM corrupt'?

CALL NZ,05FB,CL_T_CHNS Tidy the chan  area  first

POP  AF                if not. Restore error-code

INC  A                 Is it a Spectrum error?

CP   +1C

JR   C,01BF,CONTSIG_6  Step out if so.

CALL 062F,CD_TO_STK    Copy code to stack:

DEFW +1303,MAIN_4      print error-number.

DEFW +0041             (length)

CALL 0656,EXEC_STK     Execute the code.

DEC  A                 A = error-code.

CALL 05EB,P_ERR_MSG    Print the error-message.

LD   HL,+1349          Prepare the  returnaddress

EX   (SP),HL           print  comma,  line-   and

statement number.

01BF CONTSIG_6 JP   1748,PAGE_OUT     Return to the Spectrum ROM

THE 'AUTO_RUN' HOOKCODE

If there is no BASIC program in memory AND the only character in

the edit-buffer is 'RUN' then the  file  'run'  is  loaded  from

drive 1.

01C2 AUTO_RUN  LD   HL,(CHANS)        Standard channels take  21

LD   BC,+0019          bytes and  an  edit-buffer

ADD  HL,BC             with 1 command 3 bytes.

LD   BC,(WORKSP)       Test if this configuration

SBC  HL,BC             is true.

JR   NZ,016D,CONTSIG_2 Otherwise report  'Program

finished'.

01D1 AUTO_RUN2 DEC  BC                BC := endmarker.

DEC  BC                BC := <CR>.

DEC  BC                BC := 'RUN'.

LD   A,(BC)            Get command-code.

CP   +F7               Was it 'RUN'?

JR   NZ,016D,CONTSIG_2 Exit if not.

01D9 AUTO_RUN3 LD   HL,(E-LINE)       HL := start of edit-buffer

PUSH HL                Stack HL.

LD   BC,+0009          Reserve 9 extra bytes.

PUSH BC                Stack this number too.

RST  0010,CAL_SPEC     Make the required room  in

DEFW +1655,MAKE_ROOM   the edit-buffer.

POP  BC                Restore 9.

INC  BC                Include the byte 'RUN'.

POP  DE                DE := start of edit-buffer

LD   HL,+01F6          Start of the LOAD command.

LDIR                   Move it.

DEC  (NS-PPC)          Correct command pointer.

LD   (T-ADDR),+01      Signal 'LOAD'.

01F3 REP_NONS  RST  0010,CAL_SPEC     Force Spectrum ROM to  re-

DEFW +1C8A,REPORT_C    port 'Nonsense in BASIC'.

01F6 LOAD_MSG  DEFB +EF, +2A, +BC     LOAD *SGN PI;"run"

DEFB +A7, +3B, +22

DEFB +72, +75, +6E

DEFB +22

THE 'OPEN #' COMMAND ROUTINE

On entry (CH-ADD)  points  to  the  first  character  after  the

'OPEN #' token.

0200 OPEN_#    RST  0010,CAL_SPEC     Get a numerical value from

DEFW +1C82,EXPT_1NUM   the BASIC line.

CALL 0525,TEST_SEP     Seperator after it?

JP   NZ,01F3,REP_NONS  Report the error if not.

CALL 053A,CHK_CHAN     Find a channel + next char

CP   +BF               Is it 'IN'?

JR   NZ,0215,#_NOT_IN  Step if not.

'IN' takes no parameters.

0210 #_IN      RST  0020,NEXT_CHAR    Get the next character.

LD   B,+02             Signal: input.

JR   0258,#_CONT_1     Step forward.

0215 #_NOT_IN  CP   +DF               Is it 'OUT'?

LD   B,+05             Signal: output, create.

JR   Z,0221,#_OUT_EXP  Step if so.

CP   +89               Is it 'EXP'?

JR   NZ,022D,#_NOT_EXP Step if not.

LD   B,+01             Signal: output.

'OUT'  and  'EXP'  can  take  one  numerical  parameter.  If  no

parameter follows, the default 0 is assumed.

0221 #_OUT_EXP RST  0020,NEXT_CHAR    Get the next character.

RST  0010,CAL_SPEC     End of statement reached?

DEFW +2048,PR_ST_END   (so, no parameter?)

JR   Z,0258,#_CONT_1   Step if so.

PUSH BC                Store OPEN parameters.

RST  0010,CAL_SPEC     Get a numerical value.

DEFW +1C82,EXPT_1NUM

JR   025C,#_CONT_2     Step forward.

022D #_NOT_EXP CP   +A5               Is it 'RND'?

JR   NZ,0256,#_NO_PAR  Step if not.

If 'RND' takes only parameter,  then  the  file  should  already

exist, else it should be created.

0231 #_RND_1   RST  0020,NEXT_CHAR    Get the next character.

RST  0010,CAL_SPEC     Get the (first) parameter.

DEFW +1C82,EXPT_1NUM

CALL 0525,TEST_SEP     Another following?

JR   Z,0241,#_RND_2    Then step.

CALL 0532,USE_M1       Use -1 as second parameter

LD   +03               Signal: input, output.

JR   0246,#_RND_3      Step forward.

0241 #_RND_2   RST  0010,CAL_SPEC     Get the second parameter.

DEFW +1C82,EXPT_1NUM

LD   B,+07             Signal: in, out, create.

0246 #_RND_3   PUSH BC                Store OPEN parameters.

CALL 050E,CHK_END      Test for end of statement.

RST  0028,FP_CALC      stream, P1, P2

DEFB +07               (lengthbyte)

DEFB +01,exchange      stream, P2, P1

DEFB +C0,st-mem-0      (mem-0 holds P1)

DEFB +04,multiply      stream, P2*P1  (filesize)

DEFB +E0,get-mem-0     stream, filesize, P1

DEFB +38,end-calc      stream, filesize, P1

RST  0010,CAL_SPEC     BC := P1 (record-length)

DEFW +1E99,FIND_INT2

JR   0262,#_CONT_3     Step forward.

0256 #_NO_PAR  LD   B,+00             Signal: no I/O, no create.

0258 #_CONT_1  PUSH BC                Store OPEN parameters.

CALL 0532,USE_M1       Use -1 as default.

025C #_CONT_2  CALL 050E,CHK_END      Test for end of statement.

LD   BC,+0000          BC := record-length

BC contains the record-length (0 if not specified), the calcula-

tor stack holds the  stream  number  (at  the  bottom)  and  the

required filesize at the top (-1 for 'IN').

0262 #_CONT_3  PUSH BC                Store the record-length.

RST  0028,FP_CALC      stream, filesize

DEFB +05               (lengthbyte)

DEFB +C0,st-mem-0      (mem-0 holds the filesize)

DEFB +02,delete        stream

DEFB +38,end-calc      stream

CALL 062F,CD_TO_STK    Copy code to stack:

DEFW +1736,OPEN        OPEN # command.

DEFW +0021             (length)

DEC  HL                point to relative  address

DEC  HL                1754.

LD   (HL),+C9          Put a RET in place.

CALL 0656,EXEC_STK     Execute the code.

JR   Z,027A,#_CONT_4   If the stream  was already

RST  0008,SH_ERROR     open and not K,  S  or  P,

DEFB +2A               then report the error.

027A #_CONT_4  PUSH HL                Store  channel-address  in

the stream-info table.

RST  0028,FP_CALC      -

DEFB +04               (lengthbyte)

DEFB +E0,get-mem-0     filesize

DEFB +38,end-calc      filesize

POP  DE                DE := address  in  stream-

POP  HL                info table, HL :=  record-

POP  AF                length, AF := OPEN flags.

PUSH DE                Store DE again.

CALL 06A9,OPEN_CHAN    Now open the channel.

POP  HL

LD   BC,+5C18          BC := start of streams.

AND  A                 Is HL an internal channel?

SBC  HL,BC

ADD  HL,BC

JR   C,0294,#_CONT_5   Then skip.

RES  5,(IX+04)         Make channel permanent.

0294 #_CONT_5  EX   DE,HL             DE := address  in  stream-

info table

CALL 080C,GETOFFSET    HL := offset of channel.

EX   DE,HL             Swith pointers again.

LD   (HL),E            Store the  offset  in  the

INC  HL                stream-info table.

LD   (HL),D

RET                    Finished.

THE 'LOAD, SAVE, VERIFY AND MERGE' COMMAND ROUTINES

The A register holds the  first  character  after  the  keyword,

(T-ADDR) holds 0 for SAVE, 1 for LOAD, 2 for VERIFY  and  3  for

MERGE. The length of the file is the length of  the  data  +  7,

since the first 7 bytes are used to store header apart from  the

filename. In case of SAVE, the  channel  should  be  opened  for

output and create, in  all  other  cases  just  for  input.  The

routine uses large parts of the  tape-routine  in  the  Spectrum

ROM, so sufficient room for the stack is needed.

029D LD_SA_ETC CP   +2A               Is it '*'?

JP   NZ,016D,CONTSIG_2 Signal the error if not.

RST  0018,TEST_RUN     Run-time or checking?

JR   Z,02AB,LD_SA_2    Skip if checking.

LD   BC,+0022          Make 34 bytes room, for  2

RST  0010,CAL_SPEC     headers.

DEFW +0030,BC_SPACES

02AB LD_SA_2   PUSH IX                Store 128K register.

PUSH DE                Store start of the room.

RST  0020,NEXT_CHAR    Point to the next char.

CALL 053A,CHK_CHAN     Find a channel + next char

CALL 062F,CD_TO_STK    Copy code to stack:

DEFW +0652,SA_DATA     test parameters.

DEFW +0109             (length)

LD   IX,+0071          IX := start of code + 113.

ADD  IX,DE

LD   (IX-26),+C9       Put  a  RET  at   relative

address 069D.

LD   A,+C3             1FC3 = UNSTACK_Z.

CALL 0319,STOREADDR    Store  this   address   in

INC  IX                stead of 1BEE  (CHECK_END)

LD   A,+1F             in order to keep the stack

CALL 0319,STOREADDR    also when checking syntax.

POP  IX                IX := start of headers.

CALL 0656,EXEC_STK     Execute the code.

02D4 LD_SA_3   EX   (SP),IX           Stack header-start,  fetch

PUSH HL                128K register.

CALL 050E,CHK_END      Store start for LOAD, SAVE

POP  HL                etc while testing for  the

end of the statement.

LD   (K-CUR),HL        Store HL, since  K-CUR  is

POP  IX                also moved when creating a

PUSH HL                channel.

LD   B,(IX+0C)         BC := length of datablock.

LD   C,(IX+0B)

RST  0010,CAL_SPEC     Stack BC.

DEFW +2D2B,STACK_BC

RST  0028,FP_CALC      len data

DEFB +09               (lengthbyte)

DEFB +34,stk-data      len data,7 (len file-info)

DEFB +40

DEFB +B0, exponent +00

DEFB +00, +07, (+00, +00)

DEFB +0F,addition      len data + len file-info

DEFB +38,end-calc      file-size

LD   A,(T-ADDR)        Fetch the command type.

AND  A                 Was it 'SAVE'?

PUSH AF                (store the command type)

LD   A,+05             Signal: output, create.

JR   Z,02FE,LD_SA_4    Step if 'SAVE'.

LD   A,+02             Signal: input.

02FE LD_SA_4   LD   HL,+0000          HL := record-length.

CALL 06A9,OPEN_CHAN    Now open the channel.

POP  AF                Restore command type.

POP  DE                DE := start of data.

LD   HL,(STK-END)      Has the startaddress  been

SBC  HL,DE             shifted due to  the  crea-

LD   HL,(WORKSP)       tion of a channel?

EX   DE,HL             Then use the  saved  value

JR   C,0314,LD_SA_5    instead.

LD   HL,(K-CUR)

0314 LD_SA_5   CALL 06F4,SAVE_CHAN    Now execute the command.

JR   0368,CLOSE_CHN    And close the channel.

Store the address of UNSTACK_Z rather than  CHECK_END  into  the

correct locations in the stackcode, otherwise the stack will  be

severely corrupted when checking  syntax,  which  will  cause  a

system crash. The location are (relative) 069A, 06AE, 06FA, 0716

and 072F.

0319 STOREADDR LD   (IX-29),A

LD   (IX-15),A

LD   (IX+37),A

LD   (IX+58),A

LD   (IX+6C),A

RET

THE 'MOVE' COMMAND ROUTINE

The first channel is checked and  opened  for  input,  then  the

second channel is checked and opened for output and create. Then

the input-channel is moved to the output-channel, until the  end

of the input-file has been reached. Finally the two channels are

closed.  If  an  error  occurs  in  checking  or  opening    the

output-channel, then the input-channel has already been created.

Eg. MOVE 1;"name" TO "x"

0329 MOVE      CALL 053A,CHK_CHAN     Find a channel + next char

CP   +CC               Is it 'TO'?

JP   NZ,01F3,REP_NONS  Report the error if not.

RST  0018,TEST_RUN     Run-time or checking?

JR   Z,0341,MOVE_2     Skip if checking.

CALL 0532,USE_M1       Use -1.

LD   +02               Signal: input.

LD   HL,+0000          HL := record-length.

CALL 06A9,OPEN_CHAN    Open the channel.

PUSH IX                Store input-channel.

0341 MOVE_2    RST  0020,NEXT_CHAR    Get the next character.

CALL 053A,CHK_CHAN     Find a channel + next char

CALL 050E,CHK_END      test for end of statement.

POP  IX                IX := start of input-chan

PUSH IX

CALL 03D9,LEN_CHAN     Fetch channel-length.

LD   A,+05             Signal: output, create.

LD   HL,+0000          HL := record-length.

CALL 06A9,OPEN_CHAN    Open the channel.

RST  0010,CAL_SPEC     Clear  workspace  and  the

DEFW +16BF,SET_WORK    calculator stack.

POP  HL                HL := start of input-chan

IX := start of output-chan

CALL 06BF,MOVE_CHAN    Move the channel.

LD   (K-CUR),HL        Store start of input-chan

CALL 0368,CLOSE_CHN    Close the output-channel.

LD   IX,(K-CUR)        Prepare to close the input

channel.

THE 'CLOSE A CHANNEL' ROUTINE

On entry the IX registerpair holds the start of the channel.  If

a change is made to the buffer then it is written  to  disk.  No

check is made whether the file is still on the drive or an other

disk has been inserted.

0368 CLOSE_CHN LD   B,+06             Signal: close.

CALL 03DB,CALL_CHAN    Close the channel.

THE 'CLEAR A CHANNEL' ROUTINE

On entry the IX registerpair holds the start of the channel. The

channel is removed from memory, the space is reclaimed  and  the

relevant pointers are adjusted. The buffer  is  NOT  written  to

disk.

036D CLEAR_CHN CALL 080C,GETOFFSET    HL := offset of channel.

LD   BC +0014          The internal channels  (K,

AND  A                 S, R and P) take 20 bytes.

SBC  HL,BC             Is HL an internal channel?

RET  C                 Finished if it is.

ADD  HL,BC             Restore HL.

PUSH HL                Store the offset.

PUSH IX                Pass  the  start  of   the

POP  HL                Channel to HL.

LD   C,(IX+05)         Fetch channel-length in BC

LD   B,(IX+06)

PUSH BC                Keep a copy on the stack.

RST  0010,CAL_SPEC     Reclaim the space.

DEFW +19E8,RECLAIM_2

CALL 062F,CD_TO_STK    Copy code to stack:

DEFW +1663,POINTERS-1  Adjust pointers (start-1!)

DEFW +0023             (length)

LD   H,D               Change the first  two  in-

LD   L,E               structions in the code.

LD   (HL),+42          LD   B,D

INC  HL

LD   (HL),+48          LD   C,E

INC  HL

INC  HL

INC  HL

LD   (HL),+16          Start at 5C16,  the  start

INC  HL                of the stream-info table.

INC  HL

INC  HL

LD   (HL),+10          And consider all 16 chans.

POP  BC                BC := channel-length.

LD   A,B               BC := - BC.

CPL

LD   B,A

LD   A,C

CPL

LD   C,A

INC  BC

POP  HL                Restore the offset.

JP   0656,EXEC_STK     Execute the code and exit.

THE 'POINT' COMMAND ROUTINE

On entry the A register holds  the  first  character  after  the

keyword. The parameters are passed to the calculator stack and a

jump is made to the required routine.

03AA POINT     CP   +23               Is it a '#'?

JP   NZ,01F3,REP_NONS  Report the error if not.

RST  0020,NEXT_CHAR    Get the next character.

RST  0010,CAL_SPEC     Read the stream number.

DEFW +1C82,EXPT_1NUM

CALL 0525,TEST_SEP     Test for a seperator.

JP   NZ,01F3,REP_NONS  Report the error if absent

RST  0010,CAL_SPEC     Read the position.

DEFW +1C82,EXPT_1NUM

CALL 050E,CHK_END      Test for end of statement.

RST  0028,FP_CALC      stream, position

DEFB +04               (lengthbyte)

DEFB +01,exchange      position, stream

DEFB +38,end-calc      position, stream

RST  0010,CAL_SPEC     Fetch the  stream-data  in

DEFW +171E,STR_DATA    the BC registerpair.

LD   A,B               Stream not initialized?

OR   C

JR   NZ,03CD,POINT_2   Skip if initialised.

03CA REPORT_O  RST  0010,CAL_SPEC     Report 'Invalid stream'.

DEFW +160E,REPORT_O

03CD POINT_2   LD   IX,(CHANS)        IX := start of channels.

ADD  IX,BC             Add the offset.

DEC  IX                Form the startaddress.

03D5 POS_CHAN  LD   B,+0A             Signal: position

JR   03DB,CALL_CHAN    And skip.

03D9 LEN_CHAN  LD   B,+08             Signal: length

THE 'CALL A CHANNEL' ROUTINE

On entry the B register contains 0 for save, 2 for load,  4  for

open, 6 for close, 8 for length and 10 for position. A  contains

the OPEN flag. Table 4 is searched with  the  channel-identifier

in A and a jump is made to the apropriate routine.

03DB CALL_CHAN PUSH AF                Store the OPEN flags.

LD   A,(IX+04)         Get the channel-name.

OR   +20               Force to lower case.

RST  0030,LOOKUP       Lookup the startaddress in

DEFB +04               the channel-info table.

POP  AF                Restore the OPEN flags.

JP   (HL)              Jump to the routine.

THE 'CLEAR' COMMAND ROUTINE

On entry the A register holds  the  first  character  after  the

keyword. A test is made for 'CLEAR #', 'CLEAR DATA'  and  'CLEAR

LINE'.

03E5 CLEAR     CP   +CA               Is it 'LINE'?

JR   NZ,0427,CL_N_LINE Step if not.

The command 'CLEAR LINE <line 1>, <line 2>' is  to  be  used  to

remove all BASIC lines from <line 1> to <line 2> inclusive.

03E9 CLEAR_LN  RST  0010,CAL_SPEC     Pass the line  numbers  to

DEFW +1C79,NEXT_2NUM   the calculator stack.

CALL 050E,CHK_END      Test for end of statement.

CALL 04A7,LINE_STRT    HL := last line address.

JR   NZ,03F8,CLEAR_LN2 Skip if in program area.

RST  0010,CAL_SPEC     Otherwise  point  to   the

DEFW +19B8,NEXT_ONE    variables area.

EX   DE,HL

03F8 CLEAR_LN2 PUSH HL                Store last line address.

CALL 04A7,LINE_STRT    HL := first line address.

POP  BC                BC := last line address.

SBC  HL,BC             Last line before first? Or

RET  NC                both equal? Then exit.

ADD  HL,BC             Restore HL.

EX   DE,HL             DE := first line address.

LD   HL,(CH-ADD)       Test if DE <= CH-ADD <= BC

CALL 04B6,CHK_LINE

JR   C,0413,CLEAR_LN3  Skip if not.

LD   H,D               The current line should be

LD   L,E               removed too. Make the com-

LD   (NXTLIN),HL       mand pointer point to  the

DEC  HL                first command in the first

LD   (CH-ADD),HL       line after the block.

0413 CLEAR_LN4 LD   HL,(DATADD)       Test if DE <= DATADD <= BC

CALL 04B6,CHK_LINE

JR   C,0421,CLEAR_LN5  Skip if not.

DEC  DE                Reinitialize  it  to   the

LD   (DATADD),DE       first line after the block

INC  DE                (Restore DE)

0421 CLEAR_LN5 LD   H,B               HL := last line address.

LD   L,C

RST  0010,CAL_SPEC     Delete all line from DE to

DEFW +19E5,RECLAIM_1   HL inclusive.

RET                    Finished.

0427 CL_N_LINE CP   +E4               Is it 'DATA'?

JR   NZ,045D,CL_N_DATA Step if not.

The command 'CLEAR DATA <variable>' removes the BASIC  variable.

<variable> may be either a simple variable or an array, in which

case the name should have () after it,  eg.  'CLEAR  DATA  A()'.

These commands are particularly useful when overlaying parts  of

BASIC programs from disk or ram disk.

042B CLEAR_DT  RST  0020,NEXT_CHAR    Get the next character.

RST  0010,CAL_SPEC     Find the  startaddress  of

DEFW +28B2,LOOK_VARS   variable in the VARS area.

PUSH HL                Store end of var-name.

LD   A,C               Fetch discriminator in A.

RLA                    Now toggle bit  5  without

AND  +C1               disturbing the carry  flag

XOR  +40               (set if  found,  reset  if

SRL  A                 not).

PUSH AF                Store the result.

Bit 5 and 6 determine the var-type.

00 = numerical variable with name-length > 1.

10 = simple numerical/string variable or FOR/NEXT variable.

11 = string array

01 = numerical array

JR   Z,0449,CLEAR_DT3  Jump if var-name > 1 char.

043B CLEAR_DT2 RST  0010,CAL_SPEC     Fetch  current   character

DEFW +0018,GET_CHAR    again.

CP   +28               Is it a '('? (ie. array?)

JR   NZ,0449,CLEAR_DT3 Step if not.

RST  0020,NEXT_CHAR    But if it is,  then  check

CP   +29               that the next character is

the finishing ')'.

JP   NZ,01F3,REP_NONS  Report the error if not.

RST  0020,NEXT_CHAR    Point to next character.

0449 CLEAR_DT3 CALL 050E,CHK_END      Test for end of statement.

POP  AF                Restore discriminator.

POP  HL                Restore pointer to var.

RET  C                 Exit if var. doesn't exist

JR   NZ,0456,CLEAR_DT5 Step if var-name = 1 char.

0451 CLEAR_DT4 DEC  HL                Find the  first  character

BIT  7,(HL)            of the var-name.

JR   Z,0451,CLEAR_DT4

0456 CLEAR_DT5 RST  0010,CAL_SPEC     Point to the next variable

DEFW +19B8,NEXT_ONE    in the variables area.

RST  0010,CAL_SPEC     And  erase  the  requested

DEFW +19E8,RECLAIM_2   variable.

RET                    Finished.

045D CL_N_DATA CP   +23               Is it '#'?

JP   NZ,016D,CONTSIG_2 Report the error if not.

The command 'CLEAR #<stream>' is used to clear the channel  that

is attached to  <stream>.  If  <stream>  is  omitted,  then  all

channels are cleared.

0462 CLEAR_#   RST  0020,NEXT_CHAR    Get the next character.

RST  0010,CAL_SPEC     At the end of  the  state-

DEFW +2048,PR_ST_END   ment (ie. no <stream>?)

JR   Z,0483,CLEAR_ALL  Then clear all channels.

RST  0010,CAL_SPEC     Expect a number: <stream>.

DEFW +1C82,EXPT_1NUM

CALL 050E,CHK_END      Test for end of statement.

RST  0010,CAL_SPEC     BC := offset in streamdata

DEFW +171E,STR_DATA

LD   A,B               Channel closed already?

OR   C

RET  Z                 Finished if so.

LD   IX,(CHANS)        IX := startaddress of  the

ADD  IX,BC             channel.

DEC  IX

PUSH HL                Store  start  in   stream-

CALL 036D,CLEAR_CHN    table and clear the  chan-

JP   1730,OPEN_S       nel. Open the 'S' channel.

0483 CLEAR_ALL CALL 050E,CHK_END      Test for end of statement.

THE 'WIPE CHANNELS FROM MEMORY' ROUTINE

All the channels are cleared,  but  the  internal  channels  are

reinitialized again.

0486 WIPE_CHAN LD   HL,(CHANS)        Point  past  the  internal

LD   DE,+0014          channels.

ADD  HL,DE

EX   DE,HL             DE := start of channels.

LD   HL,(PROG)         HL := end of channels.

DEC  HL

RST  0010,CAL_SPEC     Reclaim the space between.

DEFW +19E5,RECLAIM_1

CALL 062F,CD_TO_STK    Copy code to stack:

DEFW +127C             Initalise internal chans.

DEFW +000C             (length)

CALL 0656,EXEC_STK     Execute the code.

XOR  A                 Clear streams 4-15.

LD   B,+18             12 streams (so 24 bytes)

04A2 WIPE_CHN2 LD   (DE),A

INC  DE

DJNZ 04A2,WIPE_CHN2

RET

THE 'GET LINE STARTADDRESS' SUBROUTINE

This subroutine is used to find the startaddress of the line  of

which the line-number is on the calculator stack.

04A7 LINE_STRT RST  0010,CAL_SPEC     Fetch the line-number into

DEFW +1E99,FIND_INT2   the BC registerpair.

LD   A,B               Jump if it is over 16,384.

CP   +40

JP   NC,0983,REPORT_B

LD   H,B               HL := line-number.

LD   L,C

RST  0010,CAL_SPEC     Find the  startaddress  of

DEFW +196E,LINE_ADDR   the  line  (or  the  first

RET                    line  after  if  the  line

doesn't exist).

THE 'CHECK LINE ADDRESS' SUBROUTINE

On entry the HL registerpair holds the line startaddress, DE the

lower edge to be tested and BC the higher edge.  The  subroutine

returns carry reset if DE <= HL <= BC.

04B6 CHK_LINE  AND  A                 Return the  carry  set  if

SBC  HL,DE             HL < DE.

RET  C

ADD  HL,DE             (correct HL)

SBC  HL,BC             Return the  carry  set  if

CCF                    HL > BC. Carry reset means

RET                    DE <= HL <= BC.

THE 'CAT' COMMAND ROUTINE

The requested channel for output is opened and a jump is made to

the CALL_UTIL routine.

04BF CAT       RST  0018,TEST_RUN     Run-time or checking?

JR   Z,04C9,CAT_2      Skip if checking syntax.

PUSH AF                Store  current   character

LD   A,+02             while opening #2.

RST  0010,CAL_SPEC

DEFW +1601,CHAN_OPEN

POP  AF

04C9 CAT_2     RST  0010,CAL_SPEC     Consider   a   '#<stream>'

DEFW +2070,STR_ALTER   after the keyword.

JR   C,04D4,CAT_3      Skip if no other stream.

CALL 0525,TEST_SEP     Test for a seperator.

JP   NZ,01F3,REP_NONS  Report the error if absent

04D4 CAT_3     LD   B,+04             Signal: CAT and handle it.

JR   04DE,PREP_UTIL

THE 'ERASE' COMMAND ROUTINE

A direct jump is made to the CALL_UTIL routine.

04D8 ERASE     LD   B,+00             Signal: ERASE and jump.

JR   04DE,PREP_UTIL

THE 'FORMAT' COMMAND ROUTNE

Continue with the CALL_UTIL routine.

04DC FORMAT    LD   B,+06             Signal: FORMAT

THE 'PREPARE UTILITY' ROUTINE

A channel is fetched from the BASIC line.

04DE PREP_UTIL PUSH BC                Store command offset.

CALL 053A,CHK_CHAN     Check the channel.

CALL 050E,CHK_END      Test for end of statement.

RST  0010,CAL_SPEC     Fetch the parameters  from

DEFW +2BF1,STK_FETCH   the calculator stack.

POP  HL                H := command offset.

THE 'CALL UTILITY' ROUTINE

On entry the H register holds 0 for ERASE, 4 for CAT and  6  for

FORMAT, the DE registerpair points to  the  parameter-block  and

the BC registerpair holds the length of that block. Table  6  is

searched for the handling routine.

04E9 CALL_UTIL PUSH DE                Store the start and length

PUSH BC                of the parameter-block.

LD   A,(DE)            A := channel-name.

LD   B,H               B := command offset.

RST  0030,LOOKUP       Lookup the startaddress in

DEFB +06               the channel_table_2 table.

POP  BC                Restore pointers and  make

POP  DE                a direct jump to the hand-

JP   (HL)              ling routine.

THE 'CLS #' COMMAND ROUTINE

On entry the A register holds  the  first  character  after  the

keyword. The colour system variables  are  all  reset  to  their

default values and the screen is cleared.

04F2 CLS_#     CP   +23               Is it '#'?

JP   NZ,016D,CONTSIG_2 Report the error if not.

RST  0020,NEXT_CHAR    Get the next character.

CALL 050E,CHK_END      Test for end of statement.

LD   A,+07             Make the border white.

RST  0010,CAL_SPEC

DEFW +229B,BORDER+7

LD   (ATTR-P),A        Set PAPER 7,  INK,  BRIGHT

XOR  A                 and FLASH 0.

LD   (MASK-P),A        No transparant colours.

LD   (P-FLAG),A        OVER and INVERSE 0.

RST  0010,CAL_SPEC     Clear the screen.

DEFW +0D6B,CLS

RET

THE 'CHECK END OF STATEMENT' SUBROUTINE

The possible characters for end-of-statement are ':'  and  <CR>.

When running, a return to the calling routine is made, otherwise

the next statement is considered.

050E CHK_END   RST  0010,CAL_SPEC     Read the character into A.

DEFW +0018,GET_CHAR

CP   +3A               Is it ':'?

JR   Z,051A,CHK_END2   Jump if so.

CP   +0D               Is it <CR>?

JP   NZ,01F3,REP_NONS  Report the error if not.

051A CHK_END2  RST  0018,TEST_RUN     Run-time or checking?

RET  NZ                Return if run-time.

LD   (ERR-NR),+FF      Signal: no errors yet.

CALL 0623,NEAT_RET     Consider a next statement.

DEFW +1BF4,STMT_NEXT

THE 'TEST FOR A SEPERATOR' SUBROUTINE

Possible seperators are ',' and ';'. On exit the  zero  flag  is

set if a seperator has been found.

0525 TEST_SEP  RST  0010,CAL_SPEC     Read the character into A.

DEFW +0018,GET_CHAR

CP   +3B               Is it a ';'?

JR   Z,052F,TEST_SEP2  Jump if so.

CP   +2C               Is it a ','?

RET  NZ                Return zero reset if not.

052F TEST_SEP2 RST  0020,NEXT_CHAR    Get the next character.

CP   A                 Return zero set.

RET

THE 'USE MINUS ONE' SUBROUTINE

During run-time, the value -1  is  put  as  last  entry  on  the

calculator stack.

0532 USE_M1    RST  0018,TEST_RUN     Return   immediately    if

RET  Z                 just checking syntax.

RST  0028,FP_CALC      Call the calculator.

DEFB +05               (lengthbyte)

DEFB +A1,stk-one       1

DEFB +1B,negate        -1

DEFB +38,end-calc      -1

RET

THE 'CHECK THE CHANNEL' ROUTINE

On entry the A register contains  the  first  character  of  the

channel  identifier.  During  run-time,  a  parameter-block   is

created in the workspace. DE holds the  startaddress,  BC  holds

the length and both registerpairs are stored on  the  calculator

stack.

053A CHK_CHAN  CP   +23               Is it a '#'?

JR   NZ,054A,CHK_CHAN3 Step if not.

RST  0018,TEST_RUN     Run-time or checking?

JR   Z,0544,CHK_CHAN2  Skip if checking syntax.

RST  0010,CAL_SPEC     Store the '#'.

DEFW +35D0,chr$+7

0544 CHK_CHAN2 RST  0020,NEXT_CHAR    Get the next character.

RST  0010,CAL_SPEC     Expect   a   number,   the

DEFW +1C82,EXPT_1NUM   stream number.

JR   0575,CHK_CHAN5    Step forward.

054A CHK_CHAN3 RST  0010,CAL_SPEC     Scan the  next  expression

DEFW +24FB,SCANNING    and store it on the calcu-

lator stack.

BIT  6,(FLAGS)         Numerical or string?

JR   Z,0560,STRING_1   Step if is was a string.

RST  0018,TEST_RUN     Run-time or checking?

JR   Z,0568,STRING_2   Step if checking syntax.

XOR  A                 Default = '' or 'm'

RST  0010,CAL_SPEC     Store the ''.

DEFW +35D0,chr$+7

RST  0028,FP_CALC      drive-nr, ''

DEFB +04               (lengthbyte)

DEFB +01,exchange      '', drive-nr

DEFB +38,end-calc      '', drive-nr

JR   0568,STRING_2     Step forward.

0560 STRING_1  CALL 0525,TEST_SEP     Test for a seperator.

JR   NZ,0572,CHK_CHAN4 Stack 0 twice if not found

RST  0010,CAL_SPEC     Expect   a   number   (the

DEFW +1C82,EXPT_1NUM   status number)

0568 STRING_2  CALL 0525,TEST_SEP     Test for a seperator.

JR   NZ,0575,CHK_CHAN5 Stack 0 once if not found.

RST  0010,CAL_SPEC     Expect a string; the file-

DEFW +1C8C,EXPT_EXP    name if using "m";1;"name"

JR   0578,CHK_CHAN6    Step forward.

0572 CHK_CHAN4 RST  0010,CAL_SPEC     Stack zero or empty string

DEFW +1CE6,USE_ZERO

0575 CHK_CHAN5 RST  0010,CAL_SPEC     Stack zero or empty string

DEFW +1CE6,USE_ZERO

0578 CHK_CHAN6 RST  0018,TEST_RUN     Run-time or checking?

JR   Z,0582,CHK_CHAN7  Skip if checking syntax.

LD   B,+02             Signal: not page-out

RST  0030,LOOKUP       Lookup the exit table.

DEFB +0E

CALL 0087,CALL_JP      Call the exit routine.

RST  0010,CAL_SPEC     Fetch current character.

DEFW +0018,GET_CHAR

RET

THE 'EXIT' ROUTINE

On entry, the  calculator  stack  contains  the  device-name,  a

number (the drive-nr for 'm', 'd', 'CAT'  and  'ram  disk';  the

status for 'j', 't' and 'b'; the stream number for  '#')  and  a

file-name (if present). The device-name should  be  1  character

long, otherwise an error will be generated.  The  three  entries

are chained together and stored in the workspace. The DE and  BC

registerpairs are set up correctly and stored on the  calculator

stack.

0586 EXIT      RST  0028,FP_CALC      dev-name, numb, file-name

DEFB +06               (lengthbyte)

DEFB +C0,st-mem-0      (mem-0 holds file-name)

DEFB +02,delete        dev-name, numb

DEFB +01,exchange      numb, dev-name

DEFB +38,end-calc      numb, dev-name

RST  0010,CAL_SPEC     Fetch parameters  of   the

DEFW +2BF1,STK_FETCH   device-name. DE := start,

DEC  BC                BC := length - 1

LD   A,B               Device-name 1 character?

OR   C

JR   Z,0596,EXIT_2     Then skip.

0594 REPORT_2A RST  0008,SH_ERROR     Report   'Invalid   device

DEFB +29               name'.

0596 EXIT_2    LD   A,(DE)            A := device-name.

SET  5,A               Force to lower case.

PUSH AF                Store channel-identifier.

RST  0010,CAL_SPEC     Fetch the number from  the

DEFW +1E94,FIND_INT1   calculator stack into A.

PUSH AF                Store status/drive-nr too.

LD   BC,+0002          Create 2 spaces  into  the

RST  0010,CAL_SPEC     workspace.

DEFW +0030,BC_SPACES

POP  AF                Store status/drive-nr into

LD   (HL),A            the second byte.

POP  AF                Store the  channel-identi-

LD   (DE),A            fier in the first byte.

RST  0010,CAL_SPEC     Store the string of  these

DEFW +2AB6,STK_STORE   2 bytes on the stack.

RST  0028,FP_CALC      string

DEFB +05               (lengthbyte)

DEFB +E0,get-mem-0     string, file-name

DEFB +17,strs-add      string + file-name

DEFB +38,end-calc      string + file-name

RET

THE 'LOOKUP (2)' ROUTINE

The continuation of RST 30. On entry the C register contains the

table number, the A register the value to search  for  in  table

'C', and B contains the offset in  table  'A'.  If  IC  6116  is

present, then the search is made using the tables copied in  it,

otherwise the ROM tables are used.

05B1 LOOKUP_2  LD   HL,+3001          IC 6116 present?

BIT  1,(HL)

LD   HL,+1FFF          Point to the tables in ROM

JR   Z,05BD,LOOKUP_3   Not present? Then skip.

INC  HL                Point to the tables in RAM

INC  HL

05BD LOOKUP_3  LD   D,(HL)            Read the startaddress into

INC  HL                the DE registerpair.

LD   E,(HL)

EX   DE,HL             HL := start of tables.

INC  C                 Simply  return  with  this

RET  Z                 address if C = 255.

PUSH BC                Stack B (offsetnumber)

LD   B,+00

ADD  HL,BC             HL := entry 'C'.

POP  BC                Restore B.

LD   D,(HL)            Read the  startaddress  of

DEC  HL                table 'C' into DE.

LD   E,(HL)            (= (HL - 1)).

EX   DE,HL             HL := start of table 'C'.

LD   C,A               Search for value 'A'.

05CD LOOKUP_4  LD   A,(HL)            Read a value from table.

INC  HL                Skip the address.

INC  HL

AND  A                 End of table (0=endmarker)

JR   Z,05D9,LOOKUP_5   Then use that address.

CP   C                 Is it value 'A'?

JR   Z,05D9,LOOKUP_5   Then use that address.

INC  HL                Else point to the new byte

JR   05CD,LOOKUP_4     and try to match that one.

05D9 LOOKUP_5  LD   A,C               Restore A.

LD   C,B               Offset number into C.

LD   D,(HL)            Read the  startaddress  of

DEC  HL                the subtable 'A'.

LD   E,(HL)

EX   DE,HL             HL := start of subtable

INC  C                 simply  return  with  this

RET  Z                 address if C = 255.  (also

used when a tables has  no

subtables)

PUSH BC                Store B.

LD   B,+00

ADD  HL,BC             HL := entry 'C'.

POP  BC                Restore B.

LD   D,(HL)            Read the  startaddress  of

DEC  HL                the handling routine  into

LD   E,(HL)            the DE registerpair.

EX   DE,HL             Now HL holds that address.

RET

THE 'PRINT AN ERROR-MESSAGE' SUBROUTINE

On entry the A register holds the message-number.  The  required

message is printed.

05EB P_ERR_MSG LD   B,+FF             Signal: no subtables.

RST  0030,LOOKUP       Find the  startaddress  in

DEFB +0C               the error-message table.

THE 'PRINT A MESSAGE AT HL' SUBROUTINE

HL points to the start of the message to be  printed.  The  last

byte of the message has it's 7th bit set.

05EF PRINT_MSG LD   A,(HL)            Read a character.

AND  +7F               Mask out bit 7.

RST  0010,CAL_SPEC     Print the character.

DEFW +0010,PRINT_A_1

BIT  7,(HL)            Last character printed?

INC  HL                (point to next character)

JR   Z,05EF,PRINT_MSG  Loop back if not.

RET

THE 'CLEAR ALL TEMPORARY CHANNELS' SUBROUTINE

All the channels  that  have  a  lower  case  channel  name  are

temporary, ie.  not  created  with  an  OPEN  #  command.  These

channels are removed from momory.

05FB CL_T_CHNS LD   HL,(CHANS)        Make HL  point  after  the

LD   BC,+0014          internal  channels   (they

0601 CL_T_NXT  ADD  HL,BC             take 20 bytes).

PUSH HL                Pass the result to IX.

POP  IX

LD   DE,(PROG)         DE := end of channel area.

DEC  DE

AND  A

SBC  HL,DE             Last channel checked?

RET  Z                 Finished if so.

JP   NC,1246,REPORT_k  Report  'RAM  corrupt'  if

even past channel area.

ADD  HL,DE             Correct HL.

LD   C,(IX+05)         BC := length of channel.

LD   B,(IX+06)

BIT  5,(IX+04)         Lower case channel name?

JR   Z,0601,CL_T_NXT   Check next channel if not.

CALL 036D,CLEAR_CHN    Clear the channel.

JR   05FB,CL_T_CHNS    Find the next channel.

THE 'NEAT RETURN' ROUTINE

Fetch the two bytes immediately after  the  CALL  and  form  the

jump-address. Restore the stackpointer and jump to the  address.

It leaves no trace on the stack.

0623 NEAT_RET  POP  HL                Fetch 'old' return address

LD   E,(HL)            Read the  next  two  bytes

INC  HL                (the jump-address) into DE

LD   D,(HL)

LD   SP,(ERR-SP)       Restore the stackpointer.

PUSH DE                Make an indirect jump  via

JP   1748,PAGE_OUT     the PAGE_OUT routine.

THE 'COPY CODE TO THE STACK' SUBROUTINE

The four bytes following the CALL are fetched and stored in  the

DE and BC registerpairs respectively. DE is the startaddress and

BC the length of the routine in the Spectrum ROM to be copied to

the stack. The first two bytes that are stacked contain the last

address + 1 of the code, so the room that is needed is BC  +  2.

On exit HL holds the last address of the code in the  stack,  DE

the first and BC the length.

062F CD_TO_STK POP  HL                Fetch 'old' return address

LD   E,(HL)            DE := startaddress of  the

INC  HL                code to be copied.

LD   D,(HL)

INC  HL

LD   C,(HL)            BC := length of the code.

INC  HL

LD   B,(HL)

INC  HL

PUSH HL                Store 'new' return address

PUSH BC                Store the length.

PUSH DE                And the startaddress.

CALL 0679,MK_RM_STK    Make room in the stack.

POP  HL                HL := startaddress of code

PUSH DE                DE = startaddress in stack

RST  0010,CAL_SPEC     LDIR from the Spectrum ROM

DEFW +33C3,MOVE_FP+3

POP  HL                DE := start of code in stk

EX   DE,HL             HL := end of code in stack

DEC  HL

POP  BC                BC := length of the code.

RET

THE 'FLOATING POINT CALCULATOR (2)' ROUTINE

The continuation of RST 0028,FP_CALC. On entry HL points to  the

address of the first byte following the RST  instruction  (which

is the lengthbyte) and the BC registerpair holds the  length  of

the space to be created in the stack.  The  RST  instruction  in

stored in the stack, followed by BC - 2  databytes.  The  return

address is stacked and the code is executed.

0648 FP_CALC_2 CALL 0679,MK_RM_STK    Make room in the stack.

POP  HL                HL := 'old' return address

PUSH DE                DE = start of code in stk.

DEC  BC                BC := length of data.

DEC  HL                Point to RST instruction.

LDI                    Stack it.

INC  HL                Skip the lengthbyte.

LDIR                   Move the databytes.

POP  DE                DE := start of code in stk

PUSH HL                Stack 'new' return address

THE 'EXECUTE CODE IN THE STACK' SUBROUTINE

On entry DE holds the first address in  the  stack  and  BC  the

length of the code. The last address of the code is fetched  and

a RET instruction is placed there. So, the value of BC should be

one more than the length of the actual routine. The  routine  is

CALLed and the stackroom reclaimed.

0656 EXEC_STK  PUSH HL                HL = end of code in stack.

PUSH DE                DE = start of code in stk.

PUSH BC                BC = length of the code.

EX   DE,HL

DEC  HL

LD   D,(HL)            DE := last address + 1.

DEC  HL

LD   E,(HL)

EX   DE,HL

DEC  HL                HL := last address.

LD   (HL),+C9          Put RET in place.

INC  HL                HL := last address + 1.

POP  DE                DE := length of the code.

POP  BC                BC := start of the code.

EX   (SP),HL           Store HL before the start.

PUSH BC

RST  0010,CAL_SPEC     CALL the code while in the

DEFW +1C1D,JUMP_C_R+7  Spectrum ROM.

POP  BC                BC := start of code.

EX   (SP),HL           HL := last address + 1.

PUSH DE                Store DE and AF (result of

PUSH AF                CALLed routine).

LD   D,B               DE := start of code.

LD   E,C

DEC  DE                DE := first roomaddress of

DEC  DE                the stack.

CALL 069B,FR_RM_STK    Free the room.

POP  AF                Restore AF and DE.

POP  DE

RET                    Finished.

THE 'MAKE ROOM IN THE STACK' SUBROUTINE

On entry BC holds the length of the code  to  be  moved  to  the

stack. A test is made for room  and  the  stackpointer  made  to

point to a position BC + 2 bytes lower. The extra  2  bytes  are

used to store the last address + 1 of the code (= 'old' value of

the stackpointer). The last 6 values on the stack are moved also

so some registerpairs/return addresses  can  be  stacked  before

calling this subroutine. On exit BC holds the length and DE  the

startaddress of the code in the stack.

0679 MK_RM_STK RST  0010,CAL_SPEC     Test for room (and  report

DEFW +1F05,TEST_ROOM   'Out of memory' if not).

LD   HL,+0000          Fetch the current value of

ADD  HL,SP             the stackpointer into HL.

LD   D,H               Make a copy in DE.

LD   E,L

SCF

SBC  HL,BC             HL := HL - BC - 1

DEC  HL                HL := HL - BC - 2, the new

LD   SP,HL             value for the stackpointer

PUSH DE                Store 'old' stackbottom.

PUSH BC                Store length.

LD   BC,+000C          Move  12  bytes  (6  stack

EX   DE,HL             entries) down the stack.

LDIR

EX   DE,HL

LD   (HL),E            Store last address + 1  as

INC  HL                first  2  bytes  into  the

LD   (HL),D            created room.

INC  HL

EX   DE,HL             DE := start of room in stk

POP  BC                BC := length of room.

POP  HL                HL := 'old' stackbottom.

RET                    Finished.

THE 'FREE THE ROOM IN THE STACK' SUBROUTINE

On entry DE should point to the start and HL to the last address

+ 1 of the room in the stack. The  stackpointer  and  stack  are

moved up in memory again.

0698 FR_RM_STK EX   DE,HL             HL := startaddress of room

DEC  DE                DE := last address of room

PUSH HL                (these 2 bytes should  not

SCF                    be moved)

SBC  HL,SP             HL := HL - SP - 1

DEC  HL                HL := HL - SP - 2 = length

LD   B,H               of room to free.

LD   C,L               Copy the length to BC.

POP  HL                HL := last address of  the

DEC  DE                current stack.

LDDR                   Move up the stack.

INC  DE                DE := 'new' stackbottom.

EX   DE,HL             Move it to HL.

LD   SP,HL             Restore the stackpointer.

RET

THE 'OPEN A CHANNEL' SUBROUTINE

On entry the calculator stack  holds  a  pointer  to  a  channel

parameter block and the length of the required  channel.  The  A

register holds the OPEN flags, bit 0 set for output, bit  1  set

for input and bit 2 set for create. HL holds  the  record-length

of the channel. Table 4 is searched and a jump is made. On  exit

IX holds the startaddress of the channel.

06A9 OPEN_CHAN PUSH HL                Store the record-length.

PUSH AF                And the OPEN flags.

RST  0028,FP_CALC      pointer, length

DEFB +05               (lengthbyte)

DEFB +C1,st-mem-1      (mem-1 holds the length)

DEFB +02,delete        pointer

DEFB +38,end-calc      pointer

RST  0010,CAL_SPEC     Fetch  the  parameters  of

DEFW +2BF1,STK_FETCH   the pointer.

PUSH BC                BC = length of parameters.

PUSH DE                DE = start of parameters.

LD   A,(DE)            Read the channel-name in A

LD   B,+04             Signal: OPEN.

RST  0030,LOOKUP       Lookup the startaddress in

DEFB +04               the channel table.

POP  DE                DE := start of parameters.

POP  BC                BC := length of parameters

POP  AF                AF := OPEN flags.

EX   (SP),HL           HL := record-length.

RET                    Jump to the handling  rou-

tine.

THE 'MOVE A CHANNEL' ROUTINE

On entry HL points to the start of the input-channel and  IX  to

the output-channel. A buffer as large as possible is created  in

the workspace. The buffer is filled from  the  input-buffer  and

emptied  into  the  output-buffer  until  the   end    of    the

input-channel has been reached. Then the buffer is reclaimed.

06BF MOVE_CHAN PUSH IX                IX = output-channel.

PUSH HL                HL = input-channel.

RST  0010,CAL_SPEC     BC := free memory.

DEFW +1F1A,FREE_MEM

LD   HL,+FF00          Make buffer  as  large  as

06C8 MOVE_CHN2 SBC  HL,BC             possible, with an overhead

JR   Z,06C8,MOVE_CHN2  of 256 bytes, unless  this

makes the buffer 0 bytes.

LD   B,H               BC := free space.

LD   C,L

RST  0010,CAL_SPEC     Create BC  spaces  in  the

DEFW +0030,BC_SPACES   workspace.  DE  :=   first

byte in the created room.

POP  IX                IX := input-channel.

06D3 MOVE_CHN3 PUSH DE                Store start of buffer.

PUSH BC                Store bufferlength.

CALL 07CF,LOAD_BYTS    Load the  bytes  into  the

POP  HL                buffer. HL := bufferlength

POP  DE                DE := start of buffer.

EX   (SP),IX           Exchange input- and output

PUSH DE                channel. Store  start  and

PUSH HL                length of the buffer.

PUSH AF                Store EOF flag.

AND  A                 BC = unloaded bytes  (when

SBC  HL,BC             EOF). HL := loaded bytes.

LD   B,H               BC := bytes to save.

LD   C,L

CALL 0711,SAVE_BYTS    Write  buffer  to  output-

POP  AF                channel. Restore EOF flag.

POP  BC                BC := bufferlength.

POP  DE                DE := start of buffer.

EX   (SP),IX           Exchange input- and output

JR   Z,06D3,MOVE_CHN3  channel and  repeat  while

not EOF.

EX   DE,HL             Switch pointers  over  and

RST  0010,CAL_SPEC     reclaim the buffer-space.

DEFW +19E8,RECLAIM_2

POP  HL                HL := input-channel.

RET                    IX = output-channel.

THE 'SAVE OR LOAD A CHANNEL' ROUTINE

On entry the A register should hold 0 for SAVE, 1  for  LOAD,  2

for VERIFY or 3 for MERGE. IX should point to the start  of  the

channel, HL to the position in memory from where the data should

be loaded or saved. A header should have  been  set  up  in  the

workspace which is identical to the header used  by  a  cassette

load or save with the exception that a filename must  be  given.

DE should point to the start of this header. Depending on  A,  a

different part of this routine is used.

06F4 SAVE_CHAN PUSH HL                HL = startaddress of data.

LD   BC,+0001          Signal: 1 byte.

LD   (T-ADDR),A        Store request.

AND  A                 Was it 'SAVE'?

JR   NZ,0727,LD_VER_ME Step if not.

First the file-type is saved,  the  file-name  is  skipped,  and

the length- and start-bytes are  saved.  Finally  the  described

block is saved.

06FC SAVE_CTRL CALL 0711,SAVE_BYTS    Save file-type (1 byte).

EX   DE,HL             HL := start of header.

LD   C,+0A             Skip  the  file-name   (10

ADD  HL,BC             bytes).

PUSH HL                HL points to the length.

EX   DE,HL             Restore pointers.

LD   C,+06             Save the  file-parameters.

CALL 0711,SAVE_BYTS    (6 bytes)

POP  HL                HL points to the length.

LD   C,(HL)            BC := file-length.

INC  HL

LD   B,(HL)

POP  DE                DE := startaddress of data

BC bytes are saved from address DE onward.

0711 SAVE_BYTS EXX                    Use alternate registers.

LD   B,+00             Signal: save.

CALL 07EE,I/O_ADDR     Find I/O address in HL.

0717 SAVE_BYT2 LD   A,B

OR   C                 Last byte saved?

RET  Z                 Exit if so.

LD   A,(DE)            Get byte to be saved.

EXX                    Use alternate registers.

PUSH HL                Save  I/O  address   while

CALL 0087,CALL_JP      CALLing it.

POP  HL

EXX                    Use normal registers.

INC  DE                Increase startaddress.

DEC  BC                Decrease length.

JP   0717,SAVE_BYT2    Loop back.

The file-type is verified and stored in the 2nd header. The next

6 bytes (the file-parameters)  are  stored  in  the  appropriate

places in the 2nd header and a jump is made to the LOAD,  VERIFY

or MERGE routine. A special case is a bytes-file. The header has

to be verified first and cannot be MERGEd.

0727 LD_VER_ME CALL 07AB,VER_BYTES    Check the file-types.

JR   Z,072E,L/V/M_2    Skip if they match.

072C REPORT_m  RST  0008,SH_ERROR     Report 'Wrong file type'.

DEFB +35

072E L/V/M_2   EX   DE,HL             HL := start of header.

DEC  HL

LD   A,(HL)            Fetch the file-type into A

LD   C,+11             Skip 17 bytes  (ie.  point

ADD  HL,BC             to the 2nd header).

LD   (HL),A            Copy the file-type.

PUSH HL                Store start of 2nd header.

PUSH AF                Store file-type.

LD   C,+0B             Skip 11 bytes  (ie.  point

ADD  HL,BC             to the file-parameters).

LD   C,+06             Now load the  file-parame-

EX   DE,HL             ters  into  their  correct

CALL 0785,TRY_LOAD     places in the 2nd header.

POP  AF                Restore file-type.

EX   (SP),IX           IX := start of 2nd header.

POP  HL                HL := start of channel.

EX   (SP),HL           HL := startaddress,  stack

PUSH HL                start of channel and start

address.

CP   +03               File-type = CODE?

JR   Z,0753,VERI_CTRL  Use VERIFY routine if so.

LD   A,(T-ADDR)        Fetch request into A.

DEC  A                 Is it 'LOAD'?

JR   Z,076D,LOAD_CTRL  Step if so.

DEC  A                 Is it 'VERIFY'?

JR   NZ,0792,MERG_CTRL Step if not.

The header is verified and a jump is made into the LOAD-routine.

0753 VERI_CTRL CALL 062F,CD_TO_STK    Copy code to stack:

DEFW +07CB,VE_CONTRL   VERIFY control.

DEFW +003D             (length)

DEC  HL

LD   (HL),+E1          Place POP HL  at  relative

DEC  HL                address 0806.

DEC  HL

DEC  HL

LD   (HL),+C9          Place  RET   at   relative

DEC  HL                address 0803.

LD   (HL),+3C          And INC A at address 0802.

POP  HL                HL := startaddress of data

CALL 0656,EXEC_STK     Execute the code.

JR   Z,077E,LOAD_CTR2  Skip if no  error  in  the

076B REPORT_j  RET  0008,SH_ERROR     header.  Otherwise  report

DEFB +32               'File size error'.

The current BASIC program  and/or  variables  are  cleared  when

loading a program or an array. Depending on the carry  flag  the

data is loaded or verified.

076D LOAD_CTRL CALL 062F,CD_TO_STK    Copy code to stack:

DEFW +0808,LD_CONTRL   LOAD control.

DEFW +00AC             (length)

LD   HL,+0068

ADD  HL,DE             HL := start of code + 104.

LD   (HL),+C9          Place  RET   at   relative

address 0870.

POP  HL                HL := startaddress of data

CALL 0656,EXEC_STK     Execute the code.

077E LOAD_CTR2 LD   B,D               IX := startaddress of data

LD   C,E               BC := length of data

EX   (SP),IX           IX := start of channel

POP  DE                DE := startaddress of data

JR   NC,078C,TRY_VERI  Step if verifying.

0785 TRY_LOAD  CALL 07C7,LOAD_BYTS    Load the data-block.

RET  Z                 Exit if no errors.

0789 REPORT_8  RST  0010,CAL_SPEC     Report 'End of file'.

DEFW +15E4,REPORT_8

078C TRY_VERI  CALL 07A8,VERI_BYTS    Verify the data-block.

RET  Z                 Exit if no errors.

0790 REPORT_k  RST  0008,SH_ERROR     Report 'Verification   has

DEFB +34               failed'.

Room is created to hold the data to be MERGEd and  the  data  is

loaded into it. The program and its variables  are  merged  with

the current program. The Spectrum ROM routine is  used,  so  all

the tape-protections still work.

0792 MERG_CTRL CALL 062F,CD_TO_STK    Copy code to stack:

DEFW +08B6,ME_CONTRL   MERGE control.

DEFW +000E             (length)

POP  HL                HL := startaddress of data

CALL 0656,EXEC_STK     Execute the code.

LD   B,D               BC := length of data.

LD   C,E

POP  IX                IX := start of channel.

PUSH HL                HL = startaddress of data.

EX   DE,HL             DE := startaddress of data

CALL 0785,TRY_LOAD     Load bytes in the buffer.

POP  HL                HL := startaddress of data

RST  0010,CAL_SPEC     Now merge the program  and

DEFW +08CE,ME_NEW_LP-6 its variables.

RET

The disk-load-address is fetched and the routine called BC times

verifying each byte and returning if  there  is  a  verification

failure. On exit the zero flag is set when the end of  file  has

been reached, reset if an error occured.

07AB VERI_BYTS EXX                    Use alternate registers.

LD   B,+02             Signal: load.

CALL 07EE,I/O_ADDR     Find the I/O address.

07B1 VERI_BYT2 LD   A,B

OR   C                 All bytes loaded?

RET  Z                 Exit if so.

07B4 VERI_BYT3 EXX                    Use alternate registers.

PUSH HL                Store  the   I/O   address

CALL 0087,CALL_JP      while loading a byte.

POP  HL

EXX                    Use normal registers.

JR   NC,07C6,VER_B_ERR Jump if not succesful.

EX   DE,HL

CP   (HL)              The actual verification.

EX   DE,HL

RET  NZ                Return if they mismatch.

INC  DE                Increase startaddress.

DEC  BC                Decrease length.

JP   07B1,VERI_BYT3    Loop back.

07C6 VER_B_ERR JR   NZ,0789,REPORT_8  Report EOF if that is true

CALL 07FF,EOF_PRSSD    <EOF> presssed?

JR   C,07B4,VERI_BYT2  Try to re-load if not.

JR   0789,REPORT_8     Report the error.

The disk-load-address is fetched and the routine  is  called  BC

times. The bytes are stored from address DE onward. On exit  the

zero flag is set when the end of  the  file  has  been  reached,

reset in case of an error.

07CF LOAD_BYTS EXX                    Use alternate registers.

LD   B,+02             Signal: load.

CALL 07EE,I/O_ADDR     Find the I/O address.

07D5 LOAD_BYT2 LD   A,B

OR   C                 All bytes loaded?

RET  Z                 Exit if so.

07D8 LOAD_BYT3 EXX                    Use alternate registers.

PUSH HL                Store  the   I/O   address

CALL 0087,CALL_JP      while loading a byte.

POP  HL

EXX                    Use normal registers.

JR   NC,07E7,LD_B_ERR  Jump if not successful.

LD   (HL),A            The actual loading.

INC  DE                Increase startaddress.

DEC  BC                Decrease length.

JP   07D5,LOAD_BYT2    Loop back.

07E7 LD_B_ERR  RET  NZ                Handle EOF in the  calling

routine.

CALL 07FF,EOF_PRSSD    <EOF> pressed?

JR   C,07D8,LOAD_BYT3  Try to re-load if not.

RET                    Handle EOF in the  calling

routine.

THE 'FIND THE I/O ADDRESS' SUBROUTINE

On entry B holds 0 for save, 2 for load. The  channel  table  is

searched and on exit HL holds the I/O address.

07EE I/O_ADDR  PUSH BC                Store entry-number.

PUSH IX                HL := start of channel.

POP  HL

RST  0010,CAL_SPEC     Set the appropriate  flags

DEFW +1615,CHAN_FLAG   for this channel.

LD   A,(IX+04)         Fetch channel-name in A.

OR   +20               Force to lower case.

POP  BC                Restore entry-number.

RST  0030,LOOKUP       Lookup the startaddress in

DEFB +04               the channel table.

EXX                    Switch  registers   before

RET                    returning.

THE 'TEST IF END OF FILE PRESSED' SUBROUTINE

A test is made whether the keys <CAPS  SHIFT>  and  <ENTER>  are

being pressed together. The carry flag is returned reset if so.

07FF EOF_PRSSD LD   A,+BF             First test <ENTER>.

AND  A

IN   A,(+FE)

RRCA

RET  C                 Return if not pressed.

LD   A,+FE             Now test <CAPS SHIFT>.

IN   A,(+FE)

RRCA                   This resets the  carry  if

RET                    it is pressed.

THE 'GET THE OFFSET OF A CHANNEL' SUBROUTINE

The startaddress of the channel  area  is  subtracted  from  the

start of the current channel and increased by one. This way  the

first channel does not have an offset of 0, since an offset of 0

means that the channel has not been opened yet.

080C GETOFFSET PUSH IX                HL  := start  of   current

POP  HL                channel.

LD   BC,(CHANS)        BC := start of chan area.

AND  A

SBC  HL,BC

INC  HL                HL := offset of channel.

RET

THE 'MAKE A CHANNEL IN MEMORY' SUBROUTINE

On entry A holds the OPEN flags: bit 0 SET for output, bit 1 SET

for input and bit 2 SET for create. The  other  bits  should  be

cleared. DE points to the start of the parameter  block  in  the

workspace and BC contains the length. HL holds the length of the

channel to be created. The  start  of  the  parameter  block  is

stored in the system variable X-PTR, the space is created and if

the parameter block is after the created channel  area  (and  is

moved), this system variable will still point  to  it.  The  I/O

addresses, the device-name, the length of the  channel  and  (if

present) the file-name are inserted into the channel  area.  The

routine continues with a test for the correct channel info.

0818 MAKE_CHAN PUSH BC                BC = length of parameters.

PUSH AF                A = OPEN flags.

LD   (X-PTR),DE        DE = start of parameters.

LD   B,H               Copy the channel length to

LD   C,L               BC.

PUSH BC                And store it.

LD   HL,(PROG)         HL := start of the channel

DEC  HL

RST  0010,CAL_SPEC     Make the room.

DEFW +1655,MAKE_ROOM

POP  BC                BC := channel length.

POP  AF                A := OPEN flags.

INC  HL                Step into the created room

PUSH HL                IX := start of channel.

POP  IX

PUSH BC                Store the channel length.

LD   (HL),+20          Position a 'space'.

DEC  BC                Now copy this 'space' into

LD   D,H               all locations of the crea-

LD   E,L               ted channel.

INC  DE

PUSH HL                (Store channel start)

LDIR

POP  HL                (Restore channel start)

AND  A                 No I/O allowed?

JR   NZ,083E,MAKE_CHN3 Skip if allowed.

083C MAKE_CHN2 LD   A,+03             Signal: input, output.

083E MAKE_CHN3 CALL 0888,SET_ADDRS    Insert address.

EX   DE,HL             DE := location of  device-

name in channel.

LD   HL,(X-PTR)        HL := start of parameters.

LDI                    Move the device-name.

EX   DE,HL             Switch pointers again.

POP  BC                BC := channel-length.

CALL 0897,STORE_BC     Insert BC in descriptor.

EX   (SP),HL           HL := length of parameters

PUSH HL                Store position in descrip-

tor and length of parm.

INC  HL                Prepare to insert  5  more

INC  HL                bytes (file-name +  number

INC  HL                + 7 (7 is  the  number  of

INC  HL                bytes already inserted  in

INC  HL                the channel descriptor)).

AND  A

SBC  HL,BC             Enough room for the bytes?

POP  BC                BC := length of parameters

POP  HL                HL := position in descript

PUSH BC                Store length again.

DEC  BC                Exclude the device-name.

PUSH DE                DE = start of parm. + 1.

EX   DE,HL             Switch pointers.

JR   NC,0860,MAKE_CHN4 Move the file-name and the

LDIR                   number if present.

0860 MAKE_CHN4 POP  DE                DE := start of parameters.

DEC  DE

POP  BC                BC := length of parameters

THE 'TEST THE CHANNEL PARAMETERS' ROUTINE

On entry DE points to the start of the parameter  block  and  BC

holds the length. Two immediate data-bytes are used;  the  first

to check if  the  parameter  after  the  OPEN-command  is  valid

(drive-number for 'm', 'd' and 'CAT', status for 't' and 'j' and

stream-number for '#'), the second  to  check  if  a  string  of

characters may be passed on to the routine (file-name for  'm').

On exit the carry flag is SET for an error.  If  SET,  then  the

zero flag is set for an error in the first byte,  RESET  for  an

error in the second byte.

0863 TEST_C    EX   DE,HL             HL := start of parameters.

POP  DE                Unstack return address.

PUSH BC                BC = length of parameters.

INC  HL

LD   C,(HL)            C := drive/status.

LD   B,+00

LD   A,(DE)            A := first byte after CALL

INC  DE                Point to the second byte.

CALL 0878,TEST_C_2     Check the first byte.

BIT  0,B               SET the zero flag.

POP  BC                BC := length of parameters

LD   A,(DE)            A := second byte.

INC  DE                Point to the 'new'  return

PUSH DE                address and stack it.

RET  C                 Return if an error.

DEC  BC                BC := BC - 2.

DEC  BC                (length of file-name)

0878 TEST_C_2  AND  A

JP   P,0881,TEST_C_3   Skip if A < 128.

CPL                    A := 255 - A.

CP   +7F               A = 127 (so, A was 128?)

RET  Z                 Exit without error if so.

DEC  C

0881 TEST_C_3  CP   C                 A < C?

RET  C                 Exit with error if so.

LD   A,B               B = 0 (length of file-name

AND  A                 < 256 characters)?

RET  Z                 Exit without error if so.

SCF                    Signal: error.

RET

Store the I/O addresses in the channel descriptor pointed to  by

IX. A has bit 0 set for output and bit 1 for input. Depending on

these bytes, the addresses  #0008  (DO_I/O  hookcode)  or  #15C4

('Invalid I/O device') are used.

0888 SETADDRS  PUSH IX                HL := start of the channel

POP  HL

CALL 088E,SETADDRS2    Repeat twice:

088E SETADDRS2 LD   BC,+0008          Prepare for a DO_I/O.

RRCA                   Rotate bits of A right.

JR   C,0897,STORE_BC   Skip if bit 0 was set.

LD   BC,+15C4          Prepare for error-message.

Store BC at address HL and update HL two addresses.

0897 STORE_BC  LD   (HL),C

INC  HL

LD   (HL),B

INC  HL

RET

The 'K', 'S' and 'P' subtables of table #04. This table is  only

used by the MOVE-command.

089C TAB_4:KSP DEFW +08E4,WRITE_KSP   Write.

DEFW +08C6,READ_KSP    Read.

DEFW +08A8,OPEN_KSP    Open.

DEFW +001C             Close (return).

DEFW +0532,USE_M1      Length (use -1).

DEFW +08CC,POSN_KSP    Position.

THE 'OPEN THE 'K', 'S' OR 'P' CHANNEL' SUBROUTINE

On entry DE points to the start of the parameter  block  and  BC

holds the length. Depending on the device-name,  the  offset  is

formed in the BC register and IX is made to point to  the  start

of the channel.

08A8 OPEN_KSP  LD   A,(DE)            A := device-name.

PUSH AF                Keep a copy on the stack.

CALL 0863,TEST_C       Test for channel info.

DEFB +00               No extra parameters.

DEFB +00               No file-name.

JR   C,0902,REPORT_A   Report the error if found.

LD   IX,(CHANS)        IX := start of chan area.

POP  AF                A := device-name.

OR   +20               Force to lower case.

CP   +70               Is it 'p'?

08BA OPEN_'s'  RET  C                 Exit if it is 's'.

08BB OPEN_'k'  LD   BC,+0005          Signal: 'k'.

JR   NZ,08C3,OPEN_KP   Skip if it is 'k'.

08C0 OPEN_'p'  LD   BC,+000F          Signal: 'p'.

08C3 OPEN_KP   ADD  IX,BC             IX := start of channel.

RET

THE 'READ FROM THE 'K', 'S' OR 'P' CHANNEL' SUBROUTINE

Call the Spectrum ROM immediately.

08C6 READ_KSP  EXX

RST  0010,CAL_SPEC

DEFW +15E6,INPUT_AD    Call INPUT_AD.

EXX

RET

THE 'POSITION THE 'K', 'S' OR 'P' CHANNEL' SUBROUTINE

If printing to the screen ('K' or 'S'), then the position  value

is fetched  and  used  as  the  X-coordinate  in  a  'PRINT  AT'

instruction.

08CC POSN_KSP  PUSH IX                HL := start of channel.

POP  HL

RST  0010,CAL_SPEC     Set the appropriate  chan-

DEFW +1615,CHAN_FLAG   nel flags.

BIT  1,(FLAGS)         Printer or screen?

JP   NZ,0BF3,REPORT_J  Report an error if printer

RST  0010,CAL_SPEC     A := position value.

DEFW +1E94,FIND_INT1

DEC  A

LD   B,A               B := X-coordinate.

LD   C,+00             C := Y-coordinate.

RST  0010,CAL_SPEC     Position the cursor.

DEFW +0A9B             (The 'PRINT AT' routine)

RET

THE 'WRITE TO THE 'K', 'S' OR 'P' CHANNEL' SUBROUTINE

Call the Specrum ROM immediately.

08E4 WRITE_KSP EXX

RST  0010,CAL_SPEC

DEFW +15F2,PRINT_A_2   Call PRINT_A_2.

EXX

RET

The '#' subtable of table #04.

08EA TAB_4:#   DEFW +0905,WRITE_#     Write.

DEFW +0911,READ_#      Read.

DEFW +08F6,OPEN_#      Open.

DEFW +001C             Close (return).

DEFW +091C,LENGTH_#    Length.

DEFW +0926,POSN_#      Position.

The '#' channel is 8 bytes long:

bytes 0 and 1 : the output address

bytes 2 and 3 : the input address

byte  4       : the device-name (upper case when permanent)

bytes 5 and 6 : the channel length.

byte  7       : the stream being pointed to.

THE 'OPEN THE '#' CHANNEL' SUBROUTINE

The channel descriptor is made and checked for validity.

08F6 OPEN_#    LD   HL,+0008          Reserve 8 bytes.

CALL 0818,MAKE_CHAN    Make the channel.

DEFB +0F               Stream number <= 15.

DEFB +00               No file-name.

RET  NC                Exit if no error occured.

JP   Z,03CA,REPORT_O   'Invalid stream' if >15.

0902 REPORT_A  RST  0010,CAL_SPEC     Report 'Invalid argument'.

DEFW +34E7,REPORT_A

THE 'WRITE TO THE '#' CHANNEL' SUBROUTINE

On entry IX points to the start  of  the  channel.  The  output-

channel is fetched and a call made to the output routine in  the

Spectrum ROM.

0905 WRITE_#   PUSH IX                Store start of channel.

PUSH AF                Store output character.

CALL 0930,CHK_LOOP     IX := output-channel.

POP  AF                Restore output character.

CALL 08E4,WRITE_KSP    Process the character.

JR   0919,#_END        Step forward.

THE 'READ FROM THE '#' CHANNEL' SUBROUTINE

On entry IX points to the  start  of  the  channel.  The  input-

channel is fetched and a call is made to the  input  routine  in

the Spectrum ROM.

0911 READ_#    PUSH IX                Store start of channel.

CALL 0930,CHK_LOOP     IX := input-channel.

CALL 08C6,READ_KSP     Get a character in A.

0919 #_END     POP  IX                Restore start of channel.

RET

THE 'LENGTH OF THE '#' CHANNEL' SUBROUTINE

The output-channel is fetched and the length calculated.

091C LENGTH_#  PUSH IX                Store start of channel.

CALL 0930,CHK_LOOP     IX := output-channel.

CALL 03D9,LEN_CHAN     Calculate the length.

JR   0919,#_END        Step back.

THE 'POSITION THE '#' CHANNEL' SUBROUINE

The output-channel is fetched and the channel is positioned.

0926 POSN_#    PUSH IX                Store start of channel.

CALL 0930,CHK_LOOP     IX := output-channel.

CALL 03D5,POS_CHAN     Position the channel.

JR   0919,#_END        Step back.

THE 'CHECK FOR AN INFINITE LOOP' SUBROUTINE

On entry IX points to the start of the '#' channel.  A  test  is

made for an infinite loop. When after 15  times  no  channel  is

found that is not a  '#'  stream,  then  the  'Wally'  error  is

printed. On exit IX points to the requested channel.

0930 CHK_LOOP  LD   B,+0F             Maximal repetition.

0932 CHK_LOOP2 LD   A,(IX+07)         Fetch pointed stream.

RST  0010,CAL_SPEC     Open the attached channel.

DEFW +1601,CHAN_OPEN

LD   IX,(CURCHL)       IX := start of this chan.

LD   A,(IX+04)         A := device-name.

OR   +20               Force to lower case.

CP   +23               Is it a '#'?

RET  NZ                Exit if not.

DJNZ 0932,CHK_LOOP2    Repeat the test B times.

RST  0008,SH_ERROR     Report 'Don't be a Wally'.

DEFB +48

The 'T' subtable of table #04.

0948 TAB_4:T   DEFW +0988,WRITE_T     Write.

DEFW +0A6B,READ_T      Read.

DEFW +096B,OPEN_T      Open.

DEFW +0986,CLOSE_T     Close.

DEFW +0532,USE_M1      Length (use -1).

DEFW +0BF3,REPORT_J    Position (error-message).

The 'T' channel is 10 bytes long:

bytes 0 and 1 : the output address.

bytes 2 and 3 : the input address.

byte  4       : the device-name (upper case when permanent).

bytes 5 and 6 : the channel-length.

byte  7       : bit 0 : SET for ZX PRINTER emulation.

bit 1 : SET for <CR> <LF> sequence.

bit 2 : SET for true backspacing.

bit 5 : SET for 'AT' character.

bit 6 : SET for 'TAB' character.

bit 7 : SET for a second parameter (AT and TAB)

byte  8       : last column (paper width).

byte  9       : current column.

The 'B' subtable of table #04.

0954 TAB_4:B   DEFW +0A63,WRITE_B     Write.

DEFW +0A71,READ_B      Read.

DEFW +0960,OPEN_B      Open.

DEFW +001C             Close (return).

DEFW +0532,USE_M1      Length (use -1).

DEFW +0BF3,REPORT_J    Position (error-message).

The 'B' channel is 7 bytes long:

bytes 0 and 1 : the output address.

bytes 2 and 3 : the input address.

byte  4       : the device-name (upper case when permanent).

bytes 5 and 6 : the channel-length.

THE 'OPEN THE 'B' CHANNEL' SUBROUTINE

The channel is created and filled  with  the  I/O  address,  the

device-name and length.

0960 OPEN_B    LD   HL,+0007          Reserve 7 bytes.

CALL 0818,MAKE_CHAN    Make the channel.

DEFB +00               No extra parameters.

DEFB +00               No file-name.

RET  NC                Exit if no error occured.

JR   0902,REPORT_A     Otherwise report the error

THE 'OPEN THE 'T' CHANNEL' SUBROUTINE

The channel is created and filled  with  the  I/O  address,  the

device-name and length. The line-length is checked and if it  is

less than 256, the len-length - 1 (or the  number  of  the  last

character in the line, starting with 0) is inserted as  byte  8.

Byte 9 is cleared.

096B OPEN_T    PUSH HL                HL = line-length.

LD   HL,+000A          Reserve 10 bytes.

CALL 0818,MAKE_CHAN    Make the channel.

DEFB +0F               Status <= 15.

DEFB +00               No file-name.

JR   C,0902,REPORT_A   Report an error if found.

POP  HL                HL := line-length.

LD   A,H               Report 'Out of range' when

AND  A                 HL is more than 255.

JR   NZ,0983,REPORT_B

DEC  L

LD   (IX+08),L         Store max-column.

LD   (IX+09),H         Clear current-column.

RET

0983 REPORT_B  CALL 0010,CAL_SPEC     Report  'Integer  out   of

DEFW +1E9F,REPORT_B    range'.

THE 'CLOSE THE 'T' CHANNEL' SUBROUTINE

Only a <CR> is sent to the printer.

0986 CLOSE_T   LD   A,+0D             Prepare for a <CR>.

THE 'WRITE TO THE 'T' CHANNEL' SUBROUTINE

On entry the A register holds the character to be printed and IX

the start of the channel. If the ZX PRINTER emulation is  to  be

used and the character does not belong to an 'AT'  or  a  'TAB',

then the address of the print routine will be the  last  address

in table #16 (A is set to zero), otherwise the table has  to  be

searched for the correct address.

0988 WRITE_T   LD   D,A               Store the character.

LD   A,(IX+07)         Char does not belong to  a

'TAB' or an 'AT'  and  the

AND  +F1               ZX  PRINTER  is   to    be

DEC  A                 emulated?

LD   A,D               (Restore A)

JR   Z,0993,WRITE_T_2  Then skip.

XOR  A                 Zero A.

0993 WRITE_T_2 PUSH DE                Store the character (in D)

LD   B,+FF             Signal: no subtables.

RST  0030,LOOKUP       Lookup the startaddress in

DEFB +16               the character table.

POP  AF                Restore the character.

PUSH IX                Store start of channel.

EX   (SP),HL           Stack the address.

LD   BC,+0007          HL := start of channel + 7

ADD  HL,BC

LD   C,(IX+08)         C := max-column.

RET                    Jump indirectly.

THE 'PRINT_OUT' SUBROUTINES

The print addresses for the  various  control-codes  (only  when

emulating the ZX PRINTER). Cursor right means just another space

'AT' and 'TAB' are stored in the channel descriptor (byte 7) and

PRINT_SECOND is  used  by  'INK',  'PAPER',  'FLASH',  'BRIGHT',

'INVERSE' and also for 'AT' and 'TAB', that use two bytes  each.

PRINT_COMMA  makes  the  current  column-number  the  next  16th

position and cursor-left only decreases the  column-number  when

not at the start of the line.

09A4 PR_CSRRIG SET  0,(FLAGS)         Signal: a leading space.

RET

09A9 PRINT_AT  SET  5,(HL)            Signal: 'AT'.

JR   09AF,PRINT_2ND    and a second character.

09AD PRINT_TAB SET  6,(HL)            Signal: 'TAB'.

09AF PRINT_2ND SET  7,(HL)            Signal: another character.

RET

09B2 PRINT_,   LD   A,(IX+09)         A := current column.

OR   +0F               A := next 16th column.

INC  A

INC  C                 No check on line-length?

JR   Z,09CE,TAB_LOOP   Then simply add the spaces

CP   C                 End of line reached?

JR   C,09CE,TAB_LOOP   Add the spaces if not.

XOR  A                 Jump to the start of a new

JR   09CE,TAB_LOOP     line and add the spaces.

THE 'PRINT OTHER CHARACTERS' SUBROUTINE

Enter here when not emulating the ZX PRINTER, or else  when  the

character that has to be printed is not a control character,  or

when still receiving characters from an 'AT' or a  'TAB'.  First

'TAB' is handled,  since  the  first  character  is  used  as  a

positioner, then 'AT' is handled, since the  first  charactr  is

not to be used. The second character holds the position for 'AT'

09C1 PR_OTHER  BIT  6,(HL)            No 'TAB'?

JR   Z,09DE,PR_OTHER2  Then skip.

09C5 TAB       RES  6,(HL)            Signal: no 'TAB'.

INC  C                 No check on line-length?

JR   Z,09CE,TAB_LOOP   Then simply add the spaces

09CA TAB_WRAP  SUB  C                 If tabbing beyond the line

JR   NC,09CA,TAB_WRAP  length, then subtract  the

ADD  A,C               line-length (a few times).

09CE TAB_LOOP  CP   (IX+09)           Return once  the  position

RET  Z                 to tab to has been reached

PUSH AF

CALL NC,0A47,PR_SPACE  If  before  the  position,

POP  AF                print a space.

PUSH AF

CALL C,0A1E,PR_NEWLIN  If after the position,  go

POP  AF                to the next line.

JR   09CE,TAB_LOOP     Repeat the process.

09DE PR_OTHER2 BIT  7,(HL)            No '2nd character'?

JR   Z,09E5,PR_OTHER3  Then skip.

09E2 2ND_CHAR  RES  7,(HL)            Signal: no '2nd character'

RET

09E5 PR_OTHER3 BIT  5,(HL)            No 'AT'?

JR   Z,0A1A,PR_OTHER4  Then skip.

09E9 AT        RES  5,(HL)            Signal: no 'AT'.

INC  C                 No check on line-length?

JR   Z,09F1,AT_2       Then skip the test.

CP   C                 Past the end of the line?

JR   NC,0983,REPORT_B  Then report 'Out of range'

09F1 AT_2      CP   (IX+09)           Return once  the  position

RET  Z                 to go to has been reached.

PUSH AF                Store position to go to.

JR   C,0A04,CHAR_LEFT  Step if moving backward.

CALL 0A4B,PR_SPACE2    Print a space (forward).

JR   0A17,AT_3         Step forward.

09FD PR_CRSLEF LD   A,(IX+09)         A := current column.

AND  A                 Return if already  at  the

RET  Z                 beginning of the line.

DEC  A                 A := column to go to.

PUSH AF                Store it.

0A04 CHAR_LEFT BIT  2,(IX+07)         'True backspacing'?

JR   NZ,0A0F,CHAR_LEF2 Then skip.

CALL 0A5D,RESET_COL    Go to start of the line.

JR   0A17,AT_3         Step forward.

0A0F CHAR_LEF2 LD   A,+08             A := <backspace>.

DEC  (IX+09)           Column := column - 1.

CALL 0A63,WRITE_B      Write the character.

0A17 AT_3      POP  AF                A := position to go to.

JR   09F1,AT_2         Repeat the process.

A test is made for <CR>, the only control character that can  be

used when not emulating the ZX PRINTER. The status-byte  (IX+07)

is tested whether just a <CR> or a <CR> <LF> sequence should  be

sent to the printer.

0A1A PR_OTHER4 CP   +0D               Is it a <CR>?

JR   NZ,0A2A,PR_OTHER5 Skip if not.

0A1E PR_NEWLIN CALL 0A5D,RESET_COL    Go to start of the line.

BIT  1,(IX+07)         Send a <LF> too?

RET  NZ                Exit if not.

LD   A,+0A             A := <LF>.

JR   0A63,WRITE_B      Write the character.

Graphics are replaced with a '?', tokens are expanded  (Spectrum

ROM routine) and the control characters (with  codes  less  than

32) are replaced with a  '?'  too  when  not  emulating  the  ZX

PRINTER.

0A2A PR_OTHER5 AND  A                 Graphics/tokens?

JP   P,0A36,PR_OTHER6  Skip if not.

SUB  +A5               Graphic character?

JR   C,0A43,PR_?       Then print a '?'.

RST  0010,CAL_SPEC     Expand  tokens  with   the

DEFW +0C10,PO_TOKENS   Spectrum ROM.

RET

0A36 PR_OTHER6 RES  0,(FLAGS)         Signal: no leading space.

CP   +20               Is it a space?

JR   Z,0A47,PR_SPACE   Then print a space.

JR   NC,0A4D,WRITECHAR Print the printables.

BIT  0,(HL)            Exit  when  not  emulating

RET  Z                 the ZX PRINTER.

0A43 PR_?      LD   A,+3F             A := '?'.

JR   0A4D,WRITECHAR    Print it.

0A47 PR_SPACE  SET  0,(FLAGS)         Signal: leading space.

LD   A,+20             A := ' '.

If the end of the line has been reached, a <CR> (with or without

a <LF>) is printed first, followed by the character.

0A4D WRITECHAR PUSH AF                Store char temporarely.

LD   A,(IX+08)         A := max-column.

CP   (IX+09)           End of the line reached?

CALL C,0A1E,PR_NEWLIN  Then go to the next line.

INC  (IX+09)           Column := column + 1.

POP  AF                Restore character to print

JR   0A63,WRITE_B      Write the character.

The current column is made zero and a <CR> is printed.

0A5D RESET_COL LD   (IX+09),+00       Zero current column.

LD   A,+0D             Print a <CR>.

THE 'WRITE TO THE 'B' CHANNEL' SUBROUTINE

On entry IX points to the start of the channel and A  holds  the

character to be printed.

0A63 WRITE_B   LD   B,+00             Signal: write.

0A65 GO_PRINT  LD   H,A               H := character to print.

LD   A,+81             Signal: printer port.

JP   0FE5,CAL_PHY      Write the character.

THE 'READ FROM THE 'T' CHANNEL' SUBROUTINE

Any character is read, but returned with bit 7 RESET.

0A6B READ_T    CALL 0A71,READ_B       Read a character.

RES  7,A               RESET bit 7.

RET

THE 'READ FROM THE 'B' CHANNEL' SUBROUTINE

Just read a character.

0A71 READ_B    LD   B,+02             Signal: read.

JR   0A65,GO_PRINT     Step back.

The 'm' subtable of table #04.

0A75 TAB_4:m   DEFW +0B6E,WRITE_CT    Write.

DEFW +0B7C,READ_CT     Read.

DEFW +0A81,OPEN_M      Open.

DEFW +0BF8,CLOSE_M     Close.

DEFW +0B81,LENGTH_M    Length.

DEFW +0BB8,POSN_M      Position.

The 'm' channel is 33 + the block-size long.

(the block-size can be 128, 256, 512 or 1024 bytes).

bytes  0 and  1 : the output address.

bytes  2 and  3 : the input address.

byte   4        : the device-name (upper case when permanent).

bytes  5 and  6 : the channel-length.

byte   7        : the drive-number.

bytes  8 to  17 : the file-name.

bytes 18 and 19 : the block-size.

bytes 20 and 21 : the record-size.

bytes 22 and 23 : the number of bytes in the last block.

bytes 24 and 25 : the location of the first block.

bytes 26 and 27 : the location of the last block.

bytes 28 and 29 : the location of the current block.

bytes 30 and 31 : the position in the current block.

byte  32        : bit 0 : SET if changes were made to the buffer

bytes 33 and up : the buffer.

THE 'OPEN THE 'M' CHANNEL' SUBROUTINE

On entry DE points to the start of the parameter block, BC holds

the length, HL contains the  record-length,  A  holds  the  OPEN

flags and mem-1  contains  the  required  length.  Depending  on

whether the file has  been  found  and  the  'create'-flag,  the

channel is opened or the report 'File not found' generated.

0A81 OPEN_M    PUSH HL                HL = record-length.

PUSH AF                A = OPEN flags.

CALL 0E38,CATSEARCH    Open 'CAT' and find file.

POP  DE                D := OPEN flags.

LD   A,D               A := OPEN flags.

BIT  2,D               'create' allowed?

JR   NZ,0A9D,CREATE    Then skip.

JR   C,0A95,FILEFOUND  Skip too if file was found

AND  A                 No 'I/O' nor 'create'?

LD   A,+05             Then signal: output,  cre-

JR   Z,0A9D,CREATE     ate and step forward.

0A93 REPORT_h  RST  0008,SH_ERROR     Report 'File not found'.

DEFB +30

The file has been found, so it should  not  be  created.  If  it

should be opened for output only, then continue with the  OUTPUT

part of the routine, else signal: input and go to the INPUT part

0A95 FILEFOUND CP   +01               'output' only?

JR   Z,0AF6,OUTPUT     Then go to the OUTPUT part

SET  1,A               Signal: input.

JR   0ADC,INPUT        Go to the INPUT part.

Test whether there is enough room on the disk  for  the  channel

and store in mem-1 the actual length  (in  bytes)  of  the  room

needed. BC holds the record-number of the last file + 1  or  the

number of a file with the same name in the catalog.  If  another

file with the same name exists on the disk, then  it  should  be

erased. A record for the 'CAT'-file containing  the  information

is created and stored on the disk.

0A9D CREATE    PUSH AF                Store the OPEN flags.

PUSH BC                Store the record-number of

the last file.

CALL 0E64,GET_FREE     Find free space on disk.

PUSH DE                DE = start of the gap.

RST  0010,CAL_SPEC     BC = length of the gap.

DEFW +2D2B,STACK_BC    Stack the length.

CALL 0F51,STCK_SIZE    Stack the block-size.

RST  0028,FP_CALC      lng, bls (len-gap, blcksz)

DEFB +1F               (lengthbyte)

DEFB +E1,get-mem-1     lng, bls, lnf (len-file)

DEFB +36,less-0        lng, bls, (0/1)  (lnf=-1?)

DEFB +00,jump-true

DEFB +10,TO 0ABE,HALF_SIZE

DEFB +E1,get-mem-1     lng, bls, lnf

DEFB +37,greater-0     lng, bls, (0/1)

DEFB +30,not           lng, bls, (1/0)   (lnf=0?)

DEFB +00,jump-true

DEFB +11,TO 0AC4,FULL_SIZE

DEFB +04,multiply      lng * bls

DEFB +E1,get-mem-1     lng * bls, lnf

DEFB +03,subtract      lng * bls - lnf

DEFB +36,less-0        (0/1)

DEFB +30,not              (does file fit in gap?)

DEFB +00,jump-true                   (Exit if so)

DEFB +TO 0AC7,CREATE_E

DEFB +A0,stk-zero      0        (signal: no room)

DEFB +33,jump                          (And exit)

DEFB +08,TO 0AC5,STORE_LEN

0ABE HALF_SIZE DEFB +01,exchange      bls, lng

DEFB +A1,stk-one       bls, lng, 1

DEFB +0F,addition      bls, lng + 1

DEFB +A2,stk-half      bls, lng + 1, 0.5

DEFB +04,multiply      bls, (lng + 1) / 2

DEFB +3A,truncate      bls, INT ((lng + 1) / 2)

0AC4 FULL_SIZE DEFB +04,multiply      actual length of file

0AC5 STORE_LEN DEFB +C1,stk-mem-1            (store it in mem-1)

DEFB +02,delete        -

0AC7 CREATE_E  DEFB +38,end-calc      -

POP  BC                BC := start of the gap.

CALL 0F48,POSN_BC      Position the catalog-file.

CALL 0EE3,READ_REC     Find the record.

LD   Hl,(5CA0)         HL := first free block  of

INC  HL                the new file.

INC  BC                BC := new record-number.

POP  DE                DE :=  last  record-number

in the 'CAT'-file.

CALL 0EA4,MAKE_FILE    Make a file-record in  the

CALL 0C63,PUT_BLOCK    catalog and write buffer.

POP  AF                Restore the OPEN flags.

On entry A holds the OPEN flags and IX points to  the  start  of

the channel. All the information  is  inserted  in  the  channel

descriptor. The first block -  1  is  inserted  as  the  current

block and the block-size as the position in the block,  so  when

the INKEYS#<stream> is used, the first byte in the  first  block

is fetched after it is loaded.

0ADC INPUT     CALL 0888,SETADDRS     Set I/O addresses.

POP  BC                BC := record-length.

CALL 0E18,STOREINFO    Store info in the  channel

descriptor.

LD   HL,(5C9E)         HL := first block location

DEC  HL                Point 1 block before it.

EX   DE,HL             Switch pointers.

LD   (HL),E            Store it.

INC  HL

LD   (HL),D

LD   C,(IX+12)         BC := block-size.

LD   B,(IX+13)

INC  HL                Store  the  block-size  as

LD   (HL),C            current  position  in  the

INC  HL                block.

LD   (HL),D

RET

On entry A holds the OPEN flags, BC the number of records in the

'CAT'-file. The new file-size  is  calculated  and  the  channel

descriptor is filled with the data.

0AF6 OUTPUT    PUSH AF                Store the OPEN flags.

PUSH BC                Store no. records in 'CAT'

LD   BC,(5C9C)         BC := number of  bytes  in

INC  BC                the last block.

RST  0010,CAL_SPEC     Stack this number.

DEFW +2D2B,STACK_BC

XOR  A                 A := 0.

RST  0010,CAL_SPEC     Store the 0 too.

DEFW +2D28,STACK_A

CALL 0F51,STCK_SIZE    Store the block-size too.

LD   BC,(5CA0)         BC := position last block.

RST  0010,CAL_SPEC     Store it too.

DEFW +2D2B,STACK_BC

LD   BC,(5C9E)         BC := position first block

PUSH BC                Keep a copy.

RST  0010,CAL_SPEC     Store it too.

DEFW +2D2B,STACK_BC

CALL 0BA9,CALC_LEN     Calculate the file-size.

Stack it (call it S).

CALL 0EE3,READ_REC     Find right file-record.

LD   HL,(5C9E)         HL := new first block

POP  BC                BC := previous first block

PUSH BC                    = new last block - 1.

AND  A

SBC  HL,BC             HL := last - first.

LD   B,H               Copy this to BC.

LD   C,L

RST  0010,CAL_SPEC     And stack it.

DEFW +2D2B,STACK_BC    (Call it B)

CALL 0F51,STCK_SIZE    Store the block-size too.

RST  0028,FP_CALC      S, B, block-size (say L)

DEFB +2B               (lengthbyte)

DEFB +04,multiply      S, B * L   (length of gap)

DEFB +C0,st-mem-0      (mem-0 holds B * L)

DEFB +02,delete        S

DEFB +31,duplicate     S, S

DEFB +A1,stk-one       S, S, 1

DEFB +0F,addition      S, S + 1     (also byte 0)

DEFB +01,exchange      S + 1, S

DEFB +E1,get-mem-1     S + 1, S, len

DEFB +36,less-0        S + 1, S, (0/1)  (len=-1?)

DEFB +00,jump-true            (then go to DOUBLE)

DEFB +12,TO 0B4A,DOUBLE

DEFB +E1,get-mem-1     S + 1, S, len

DEFB +37,greater-0     S + 1, S, (0/1)

DEFB +30,not           S + 1, S, (1/0)   (len=0?)

DEFB +00,jump-true         (then go to FULLSIZE2)

DEFB +15,TO 0B52,FULLSIZE2

DEFB +E1,get-mem-1     S + 1, S, len

DEFB +0F,addition      S + 1, S + len

DEFB +C1,st-mem-1      (mem-1 holds new file-len)

DEFB +E0,get-mem-0     S + 1, S + len, B * L

DEFB +03,subtract      S + 1, (S + len) - (B * L)

DEFB +37,greater-0     S + 1, (0/1)

DEFB +30,not           S + 1, (1/0) (enough room)

DEFB +00,jump-true                   (Exit if so)

DEFB +10,TO 0B56,OUTPUT_E

DEFB +A0,stk-zero      S + 1, 0 (Signal: no room)

DEFB +33,jump                          (And exit)

DEFB +0B,TO 0B54,STORE_RM

0B4A DOUBLE    DEFB +A2,stk-half      S + 1, S, 0.5

DEFB +05,division      S + 1, S * 2

DEFB +31,duplicate     S + 1, S * 2, S * 2

DEFB +E0,get-mem-0     S + 1, S * 2, S * 2, B * L

DEFB +03,subtract      S+1, S*2, (S*2)-(B*L)

DEFB +36,less-0        S + 1, S * 2, (0/1)

DEFB +00,jump-true                 (enough room?)

DEFB +03,TO 0B54,STORE_RM            (Exit if so)

0B52 FULLSIZE2 DEFB +02,delete        S + 1

DEFB +E1,get-mem-0     S + 1, B * L

0B54 STORE_RM  DEFB +C1,st-mem-1      (mem-1 holds new file-len)

DEFB +02,delete        S + 1

0B56 OUTPUT_E  DEFB +38,end-calc      S + 1

POP  HL                HL := last block position

POP  BC                BC := first block position

LD   D,B               Copy it to DE.

LD   E,C

CALL 0EA4,MAKE_FILE    Make a file-record in  the

CALL 0C63,PUT_BLOCK    catalog and write buffer.

POP  AF                A := OPEN flags.

POP  BC                BC := record-length.

PUSH AF

CALL 0E18,STOREINFO    Store info in the  channel

CALL 0BB8,POSN_M       descriptor  and   position

POP  AF                the 'CAT'-file.

JP   0888,SETADDRS     Set the I/O addresses.

THE 'WRITE TO THE 'M' CHANNEL' SUBROUTINE

On entry A holds the byte to be written  to  the  disk,  and  IX

points to the start of the channel. The byte is  stored  in  the

buffer at the correct position (possibly after loading the right

block from disk) and bit 0 of (IX+32d) is SET to  indicate  that

the buffer has been changed, so it has to  be  written  to  disk

when loading a new block or closing the channel.

0B6E WRITE_CT  PUSH AF                Store byte temporarely.

CALL 0F5B,ENDOFFILE    Find the correct position.

JP   NC,076B,REPORT_j  Report an error if found.

POP  AF                Restore   the   byte   and

LD   (HL),A            insert it into the buffer.

SET  0,(IX+20)         Signal: change made.

RET

THE 'READ FROM THE 'M' CHANNEL' SUBROUTINE

On entry IX points to the start of the channel. The current byte

is fetched (possibly after loading the right block from disk).

0B7C READ_CT   CALL 0F5B,ENDOFFILE    Find the correct position.

LD   A,(HL)            Read the byte.

RET

THE 'CALCULATE THE LENGTH OF THE 'M' CHANNEL' SUBROUTINE

The appropriate values from the channeldescriptor are stored  on

the calculator stack and the length is calculated.

0B81 LENGTH_M  LD   C,(IX+16)         BC := number of  bytes  in

LD   B,(IX+17)         the last block.

INC  BC                Include byte 0.

RST  0010,CAL_SPEC     Store this number.

DEFW +2D2B,STACK_BC    (Call it B)

LD   C,(IX+1E)         BC := position in  current

LD   B,(IX+1F)         block.

RST  0010,CAL_SPEC     Store it too.

DEFW +2D2B,STACK_BC    (Call it P)

CALL 0F51,STCK_SIZE    Stack block-size (say S)

LD   C,(IX+1A)         BC := last block

LD   B,(IX+1B)

RST  0010,CAL_SPEC     Stack it too.

DEFW +2D2B,STACK_BC    (Call it L)

LD   C,(IX+1C)         BC := current block.

LD   B,(IX+1D)

RST  0010,CAL_SPEC     Stack it too.

DEFW +2D2B,STACK_BC    (Call it F)

0BA9 CALC_LEN  RST  0028,FP_CALC      B, P, S, L, F

DEFB +0E               (lengthbyte)

DEFB +03,subtract      B, P, S, L - F

DEFB +04,multiply      B, P, S * (L - F)

DEFB +01,exchange      B, S * (L - F), P

DEFB +03,subtract      B, S * (L - F) - P

DEFB +0F,addition      B + S * (L - F) - P

(The remaining length, X)

DEFB +31,duplicate     X, X

DEFB +36,less-0        X, (0/1)

DEFB +30,not           X, (1/0)      (length>=0?)

DEFB +00,jump-true                   (Exit if so)

DEFB +02,TO 0BB6,CALC_L_E

DEFB +30,not           0              (length:=0)

0BB6 CALC_L_E  DEFB +38,end-calc      The remaining length.

RET

THE 'POSITION THE 'M' CHANNEL' SUBROUTINE

On entry  IX  points  to  the  start  of  the  channel  and  the

calculator stack holds the new  position  of  the  channel.  The

position is multiplied with the record-size, giving the position

in the file in bytes.

0BB8 POSN_M    BIT  3,(IX+02)         Input allowed?

JR   Z,0BF3,REPORT_n   Report the error if not.

LD   C,(IX+14)         BC := record-size.

LD   B,(IX+15)

LD   A,B               Record-size unspecified?

OR   C

JR   NZ,0BC9,POSN_M_2  Skip if not.

INC  BC                Use 1 as record-size.

0BC9 POSN_M_2  RST  0010,CAL_SPAC     Stack the record-size.

DEFW +2D2B,STACK_BC

RST  0028,FP_CALC      new-pos, rec-sz

DEFB +08               (lengthbyte)

DEFB +01,exchange      rec-sz, new-pos

DEFB +A2,stk-half      rec-sz, new-pos, 0.5

DEFB +03,subtract      rec-sz, new-pos - 0.5

DEFB +27,int           rec-sz, INT (new-pos - .5)

DEFB +04,multiply      rec-sz*INT(new-pos-0.5)

DEFB +38,end-calc      The position in bytes.

CALL 0F51,STCK_SIZE    Stack the block-size.

RST  0010,CAL_SPEC     The calculator stack holds

DEFW +36A0,n-mod-m     the  block-number  in  the

file and the  position  in

RST  0010,CAL_SPEC     this block.

DEFW +1E99,FIND_INT2   BC := block-number in file

LD   L,(IX+18)         HL := first block position

LD   H,(IX+19)

ADD  HL,BC             HL := current block pos.

LD   B,H               Copy this number to BC.

LD   C,L

CALL 0FAA,READ_NEXT    Load right block.

RST  0010,CAL_SPEC     BC  :=  position  in  this

DEFW +1E99,FIND_INT2   block.

LD   (IX+1E),C         Insert this  address  into

LD   (IX+1F),B         the channel descriptor.

RET

0BF3 REPORT_J  RST  0010,CAL_SPEC     Report 'Invalid I/O device

DEFW +15C4

0BF6 REPORT_n  RST  0008,SH_ERROR     Report 'Wrong disk'.

DEFB +36

THE 'CLOSE THE 'M' CHANNEL' SUBROUTINE

On entry IX points to the start of the current channel. If input

was allowed, then write the buffer to disk; else the channel was

opened with 'OUT' and the correct file-length should be  written

in the catalog-file.

0BF8 CLOSE_M   BIT  3,(IX+02)         Input was allowed?

JR   NZ,0C63,PUT_BLOCK Then write buffer to disk.

LD   E,(IX+1C)         DE := current block.

LD   D,(IX+1D)

A test is made whether the current position is the last position

in the file.

LD   L,(IX+16)         HL := number of  bytes  in

LD   H,(IX+17)         the last block.

LD   C,(IX+1E)         BC := current position  in

LD   B,(IX+1F)         the block.

DEC  BC

AND  A

SBC  HL,BC             BC at the end of the file?

JR   NZ,0C21,CLOSE_M_2 Skip if not.

LD   L,(IX+1A)         HL := last block.

LD   H,(IX+1B)

AND  A

SBC  HL,DE             Current = Last block?

JR   Z,0C63,PUT_BLOCK  Then write buffer to disk.

Not all the space available is  used  by  the  file.  Write  the

correct length in the file-record in the catalog.

0C21 CLOSE_M_2 PUSH IX                Store start of channel.

PUSH BC                Store current position.

PUSH DE                Store last block position.

LD   L,(IX+18)         HL := first block position

LD   H,(IX+19)

PUSH HL                Store it too.

LD   DE,+FFE5          This is -27d.

CALL 0FA1,HL:=IX+DE    HL := start of parameters.

EX   DE,HL             Pass it to DE.

LD   BC,+000C          BC := length of parameters

CALL 0E38,CATSEARCH    Open 'CAT'-file.

LD   (IX+04),+EF       Signal: 'CAT' channel.

JP   NC,0A93,REPORT_h  Report an error if found.

LD   HL,(5C9E)         HL := first block ('CAT')

POP  DE                DE := first block (descr.)

AND  A

SBC  HL,DE             Info on  disk  =  info  in

JR   NZ,0BF6,REPORT_n  descriptor? Report  'Wrong

disk' if not.

LD   HL,(5CA0)         HL := last block ('CAT')

POP  DE                DE := last block (descr.)

SBC  HL,DE             Repeat the test.

JR   C,0BF6,REPORT_n

LD   (5CA0),DE         Store DE.

POP  HL                Store the number of  bytes

LD   (5C9C),HL         in the last block too.

LD   H,B               HL := current record

LD   L,C

CALL 0F07,MAKE_FREE    Store record in 'CAT'-file

CALL 0368,CLOSE_CHN    Close the 'CAT'-file.

POP  IX                IX := start of 'm' channel

THE 'PUT A BLOCK ON DISK' SUBROUTINE

On entry IX points to the start of the channel. If changes  have

been made to the buffer then write it out on the disk.

0C63 PUT_BLOCK BIT  0,(IX+20)         Exit if  no  changes  were

RET  Z                 made to the buffer.

LD   BC,+0000          Signal:  write  an  entire

LD   DE,+0000          block.

CALL 0FA1,HL:=IX+DE    HL := IX + 33 = buffer

EX   DE,HL             Pass the bufferstart to DE

LD   L,(IX+1C)         HL := current block.

LD   H,(IX+1D)

LD   A,(IX+07)         A := drive-number.

CALL 0FE5,CAL_PHY      Write the block.

RES  0,(IX+20)         Signal: no changes yet.

RET

The 'm' and ' ' subtables of table #06. The only  difference  in

these two tables is the entry for erase, since the  error  'File

not found' should not be generated when using the 'm' identifier

0C83 TAB_6:m   DEFW +0CF7,ERASE_M     Erase.

DEFW +0594,REPORT_2A   Error 'Invalid I/O device'

DEFW +0C93,CAT_M       Catalog.

DEFW +0D12,FORMAT_M    Format.

0C8B TAB_6:' ' DEFW +0CFE,ERASE_' '   Erase.

DEFW +0594,REPORT_2A   Error 'Invalid I/O device'

DEFW +0C93,CAT_M       Catalog.

DEFW +0D12,FORMAT_M    Format.

THE 'CATALOG THE 'M' CHANNEL' SUBROUTINE

The 'CAT'-file is opened, the records  are  printed  (the  names

only), followed by the free space on the disk; rounded downwards

to the nearest K.

0C93 CAT_M     CALL 0DCC,OPEN_CAT     Open the 'CAT'-file.

CALL 0F45,POS_START    Position it at the start.

LD   HL,+0000          HL := free-space counter.

CALL 0CC8,PR_NAME      Print the disk-name.

CALL 0CEC,PRINT_CR     Print a <CR>.

0CA2 CAT_M_NXT CALL 0CC8,PR_NAME      Print the next file-name.

JR   NC,0CA2,CAT_M_NXT Repeat until end of 'CAT'.

LD   B,H               BC := free-space.

LD   C,L

RST  0010,CAL_SPEC     Stack it.

DEFW +2D2B,STACK_BC

CALL 0F51,STCK_SIZE    Also stack the block-size.

RST  0028,FP_CALC      free, size

DEFB +09               (lengthbyte)

DEFB +04,multiply      free * size

DEFB +34,stk-data      free * size, 1024  (is 1K)

DEFB +3B, exponent +8B

DEFB +00, (+00,+00,+00)

DEFB +05,division      (free * size) / 1024

DEFB +3A,truncate      TRUNC (free * size) / 1024

DEFB +38,end-calc      The free space on the disk

CALL 0CEC,PRINT_CR     Print a <CR>.

PUSH IX                Save start of the channel,

RST  0010,CAL_SPEC     while  printing  the  free

DEFW +2DE3,PRINT_FP    space on the disk.

POP  IX

CALL 0CEC,PRINT_CR     Again, print a <CR>.

JP   0368,CLOSE_CHN    Close the 'CAT'-file.

Print the name of the current block and move to the next record.

HL is used to  store  the  number  of  used  blocks.  First  the

start-block of the new file is added to HL, then the last  block

is subtracted from it. When the endmarker is reached,  only  the

first value is added. This is the number of blocks on the  disk,

so HL holds the number of blocks that have not been used.

0CC8 PR_NAME   PUSH HL                Store block counter.

CALL 0EE3,READ_REC     Read the next record.

POP  HL                Restore the block counter.

LD   DE,(5C9E)         DE := start-block of file.

ADD  HL,DE             Add it to HL.

LD   DE,(5CA0)         DE := end-block of file.

LD   A,D

OR   E                 Endmarker reached?

INC  A

SCF                    Signal: last record.

RET  Z                 Return if so.

SBC  HL,DE             Subtract (end-block + 1).

LD   DE,+5CA2          DE := start of file-name.

LD   A,(DE)            Read first character of  a

AND  A                 file-name. CHR$ 0? Then it

RET  Z                 is a hidden file, so exit.

LD   B,+0A             10 characters in a name.

0CE5 PR_NAME2  LD   A,(DE)            Read a character.

CALL 0CEE,PRINT_A      Print it.

INC  DE                Point to the next one.

DJNZ 0CE5,PR_NAME2     Loop back until done all.

0CEC PRINT_CR  LD   A,+0D             A := <CR>.

0CEE PRINT_A   PUSH IX                Save start of the channel,

RST  0010,CAL_SPEC     while printing CHR$ 'A'.

DEFW +15F2,PRINT_A_2

POP  IX

AND  A                 Signal: more records to go

RET

THE 'ERASE FROM THE 'M' CHANNEL' SUBROUTINE

The catalog is searched for the file and if it was  found,  then

the entry in the 'CAT'-file should  be  erased.  If  not  found,

simply return without an error-message.

0CF7 ERASE_M   CALL 0E38,CATSEARCH    Find the file-record.

JR   NC,0D0F,ERASE_M_E Exit if not found.

JR   0D04,ER_FOUND     Step to erase the entry.

THE 'ERASE FROM THE ' ' CHANNEL' SUBROUTINE

The catalog is searched for the file and if it was  found,  then

the entry in the 'CAT'-file should  be  erased.  If  not  found,

report 'File not found'.

0CFE ERASE_' ' CALL 0E38,CATSEARCH    Find the file-record.

JP   NC,0A93,REPORT_h  Report the error if absent

The number of records in the catalog is found in BC.

0D04 ER_FOUND  PUSH BC                Store the record-number.

0D05 ER_FOUND2 INC  BC                Point to the next record.

CALL 0E3D,FIND_FILE    Find the next record  with

JR   C,0D05,ER_FOUND2  the same name (or: just go

to the end of the catalog)

POP  HL                HL := record-number.

CALL 0F07,MAKE_FREE    Erase the entry.

0D0F ERASE_M_E JP   0368,CLOSE_CHN    Close the 'CAT'-file.

THE 'FORMAT THE 'M' CHANNEL' SUBROUTINE

On entry DE points to the start of the parameter  block  and  BC

holds the length. The disk is FORMATted and  the  'CAT'-file  is

written to the disk.

0D12 FORMAT_M  LD   HL,(PROG)         HL := start of new channel

DEC  HL

PUSH HL                Store it.

LD   HL,(ERR-SP)       Also store the address  of

PUSH HL                the error routine.

LD   (ERR-NR),+2F      Signal: 'Disk I/O error'.

CALL 0D6B,FORMATCHK    Disk FORMATted already?

POP  HL                Restore the error  routine

LD   (ERR-SP),HL       address.

LD   A,(ERR-NR)        Fetch error status.

CP   +2F               Still 'Disk I/O error'?

JP   NZ,016D,CONTSIG_2 Report new error if not.

POP  IX                IX := start of the channel

LD   A,(IX+07)         A := drive-number.

LD   B,+06             Signal: format.

CALL 0FE5,CAL_PHY      Format the disk.

Now create the two catalog entries. The first holds  the  start-

and end-block of the'CAT'-file, the number of bytes in the  last

block and the disk-name. The second  is  an  endmarker  and  has

+FFFF as it's 5th and 6th byte and the total number of blocks on

the disk as it's 3rd and 4th byte. On entry BC holds the  block-

size, DE the last block-number of  the  'CAT'-file  and  HL  the

total number of blocks on the disk that are  accessible  to  the

system. The extra block with block-number -1 is not counted.

0D38 FORMAT_M2 DEC  BC                BC := block-size - 1.

LD   (5C9C),BC         Store it.

LD   (5CA0),DE         DE = catalog-size.

PUSH HL                HL = number of blocks.

LD   DE,+FFE7          This is -25d.

CALL 0FA1,HL:=IX+DE    HL := disk-name in channel

LD   DE,+5CA2          DE := file-name in record.

LD   BC,+000A          Copy 10 characters.

LDIR

LD   (5C9E),BC         Zero the start-block.

INC  HL                HL points to the 2nd entry

CALL 0E2E,PUT_12FF     Put 12 FF's after HL.

CALL 0EF6,WRITE_REC    Write the record on disk.

POP  HL                HL := number of blocks.

LD   (5C9E),HL         Store it.

LD   HL,+FFFF          HL := endmarker.

LD   (5CA0),HL         Store it too.

CALL 0EF6,WRITE_REC    Write the record on disk.

JP   0368,CLOSE_CHN    Close the 'CAT'-file.

THE 'CHECK FOR A FORMATTED DISK' SUBROUTINE

The catalog is read and when this is impossible  then  the  disk

has not been FORMATted yet. (or has a different format).

0D6B FORMATCHK LD   HL,+1708          If an error  occurs,  then

PUSH HL                return to the calling rou-

LD   (ERR-SP),SP       tine, even  when  the  ROM

has been paged out.  (1708

is PAGE_IN)

LD   L,+80             Signal: find configuration

CALL 0DD4,MAKE_CAT     Make a 'CAT' channel.

CALL 0D82,CHECK_2      Try to read the catalog.

POP  HL                Clear the stack.

RET  Z                 Return if no errors.

LD   (ERR-NR),+0C      Else report 'BREAK -  CONT

RET                    repeats'.

0D82 CHECK_2   CALL 0F45,POS_START    Point to the first record.

CALL 0EE3,READ_REC     Read it.

LD   A,+FD             Open channel 'K'.

RST  0010,CAL_SPEC

DEFW +1601,OPEN_CHAN

LD   A,+E0             Print 'Destroy "'.

CALL 05EB,P_ERR_MSG

LD   DE,+5CA2          DE := start of disk-name

LD   B,+0A             10 characters in a name.

0D97 CHECK_3   LD   A,(DE)            Read a character.

CP   +20               Not a control character?

CALL NC,0CEE,PRINT_A   Then print it.

INC  DE                Point to the next one.

DJNZ 0D97,CHECK_3      Loop back until done all.

LD   A,+E1             Print '" ?' and wait for a

CALL 0DB1,PR_&_WAIT    key.

PUSH AF                Store the key.

LD   A,+E4             Signal: destroy question.

LD   B,+FF             Signal: no subtables.

RST  0030,LOOKUP       Lookup the answer  key  in

DEFB +0C               the error-message table.

POP  AF                Restore the key.

OR   +20               Force to lower case.

CP   (HL)              Test the key.

RET

Print the message in A and wait for a keypress. Return the value

of the key in A.

0DB1 PR_&_WAIT CALL 05EB,P_ERR_MSG    Print the message.

SET  5,(TVFLAG)        Signal: clear bottom lines

SET  3,(FLAGS)         Signal: 'L' mode.

RST  0010,CAL_SPEC     Wait for a key.

DEFW +15DE,WAIT_KEY

RET                    Return with the key in A.

The 'CAT' subtable of table #04.

0DC0 TAB_4:CAT DEFW +0B6E,WRITE_CT    Write.

DEFW +0B7C,READ_CT     Read.

DEFW +0DCC,OPEN_CAT    Open.

DEFW +0C63,PUT_BLOCK   Close.

DEFW +0B81,LENGTH_M    Length.

DEFW +0BB8,POSN_M      Position.

The 'CAT' channel is the same as the 'm'  channel,  except  that

the file-name is not initialised, so this channel  could  easily

be changed into a 'm' channel.

THE 'OPEN THE 'CAT' CHANNEL' SUBROUTINE

On entry DE points to the start of the parameter block, BC holds

the length and A holds the OPEN flags. The  channel  is  created

and the info is stored in it. Sometimes the space of  the  'CAT'

channel is used as the 'm' channel. Then the info in the channel

is changed. Therefor the parameter block should  be  allowed  to

hold a file-name. The  first  part  of  the  routine  adds  this

file-name to the parameter block.

0DCC OPEN_CAT  LD   HL,+000A          10 bytes in a file-name.

ADD  HL,BC             Add the parameter length.

LD   B,H               BC := length of the  'new'

LD   C,L               parameter block.

LD   L,+00             Signal:  read  the  format

from the disk.

0DD4 MAKE_CAT  PUSH BC                BC = length of parameters.

PUSH DE                DE = start of parameters.

PUSH HL                L = format check type.

CALL 0863,TEST_C       Test the channel info.

DEFB +81               A drive-number is needed.

DEFB +F6               File-name needed and <= 10

characters in it.

JR   NC,0DE5,MAKE_CAT2 Skip if no errors.

JR   NZ,0DE2,REPORT_F  Skip if error in file-name

0DE0 REPORT_c  RST  0008,SH_ERROR     Report   'Invalid    drive

DEFB +2B               number.

0DE2 REPORT_F  RST  0010,CAL_SPEC     Report 'Invalid filename'.

DEFW +1765,REPORT_F

0DE5 MAKE_CAT2 POP  BC                C := format check type.

POP  DE                DE := start of parameters.

PUSH DE                Store it again.

INC  DE                Point to the 2nd location.

LD   A,(DE)            A := drive-number.

LD   B,+04             Signal: enquire.

CALL 0FE5,CAL_PHY      Inquire the disk.

POP  DE                DE := start of parameters.

POP  HL                HL := length of parameters

XOR  A

PUSH BC                BC = block-size.

PUSH HL                Store length of parameters

LD   HL,+0021          HL := channel-length.

ADD  HL,BC

POP  BC                BC := length of parameters

CALL 0818,MAKE_CHAN    Make the 'CAT' channel.

DEFB +81               A drive-number is needed.

DEFB +F6               A file-name is needed with

<= 10 characters.

LD   DE,+FFF1          This is -15d.

CALL 0FA1,HL:=IX+DE    HL := channel start + 18d.

POP  BC                BC := block-size.

LD   (HL),C            Store it  in  the  channel

INC  HL                descriptor.

LD   (HL),B

CALL 0E2E,PUT_12FF     Put 12 FF's after it.

CALL 0EE3,READ_REC     Find the right file.

LD   HL,(5C9E)         HL := start-block of file.

LD   A,H               Start-block <>  0?  (start

OR   L                 of the 'CAT'-file)

JP   NZ,1304,REPORT_h  Then report the error.

LD   C,+10             Record-size of 'CAT' is 16

Store the record-size, the number of bytes in the last block and

the first- and end-block numbers in the descriptor.

0E18 MAKE_CAT3 LD   DE,+FFF3          This is -13d.

CALL 0FA1,HL:=IX+DE    HL := channel start + 20d.

LD   (HL),C            Store the record-size.

INC  HL

LD   (HL),B

INC  HL

EX   DE,HL             Move the number  of  bytes

LD   HL,+5C9C          in the last block and  the

LD   BC,+0006          first- and end-block  num-

LDIR                   bers to the descriptor.

LD   C,+02             Signal: point to record 2.

RET

THE 'STORE 12 FF'S AFTER HL' SUBROUTINE

HL is incremented and the next 12 bytes are made +FF.  The  13th

byte is made 0.

0E2E PUT_12FF  LD   B,+0C             Count 12 bytes.

0E30 PUT_12FF2 INC  HL                Increase address.

LD   (HL),+FF          Store the +FF in it.

DJNZ 0E30,PUT_12FF2    And repeat.

INC  HL                Clear the 13th byte.

LD   (HL),B

RET

THE 'SEARCH THE CATALOG FROM THE START' SUBROUTINE

The 'CAT'-file is opened and searched from  the  start  for  the

file-name.

0E38 CATSEARCH LD   L,+00             Signal:  read  the  format

from the disk.

CALL 0DD4,MAKE_CAT     Make the 'CAT' channel.

THE 'FIND THE FILE-RECORD' SUBROUTINE

On entry BC holds  the  record-number  in  the  'CAT'-file.  The

catalog is searched for a file whose name is  the  same  as  the

name that is stored at address (IX+08). On exit the  carry  flag

is SET when the file has been found. If found, then BC holds the

record-number of that file.

0E3D FIND_FILE PUSH BC                Store record-number.

0E3E FIND_FIL2 CALL 0EE3,READ_REC     Fetch the file-record.

LD   HL,(5CA0)         Is it the last record?

LD   A,H

AND  L                 (RESET the carry flag)

INC  A                 SET the zero flag if so.

POP  BC                BC := record-number.

INC  BC                Point to the next record.

RET  Z                 Return if it was the last.

PUSH BC                Store the record-number.

LD   DE,+FFE7          This is -25d.

CALL 0FA1,HL:=IX+DE    HL := channel start + 8.

(= start of file-name)

LD   BC,+000A          10 characters in a name.

LD   DE,+5CA2          DE := file-name in 'CAT'.

0E57 FIND_FIL3 LD   A,(DE)            Get a character.

CPI                    Does  it  match  with  the

JR   NZ,0E3E,FIND_FIL2 record? Loop back if not.

INC  DE                Point to the next one.

JP   PE,0E57,FIND_FIL3 Loop back until BC = 0.

POP  BC                BC := next record-number.

DEC  BC                BC := right record-number.

SCF                    Signal: found.

RET

THE 'GET FREE SPACE ON THE DISK' SUBROUTINE

On entry BC holds the record-number of a file, or when  no  such

file is present, the record-number of the endmarker. If BC holds

the record-number of the endmarker and the endmarker is  at  the

end of the catalog file, then there is no room in  the  catalog,

else a new file-record can be created, or  the  descripted  file

can be replaced. The file-name is skipped and the length of  the

gap between each two files is found. The largest gap  is  stored

and when two gaps of equal size are found, then the  second  gap

is used. On exit BC holds the length of the biggest gap  and  DE

holds the record-number of the start of the gap.

0E64 GET_FREE  CALL 0F48,POSN_BC      Position the 'CAT'-file.

LD   BC,+1001          16 bytes, to read.

0E6A GET_FREE2 PUSH BC

CALL 0B7C,READ_CT      Read a byte.

JR   NC,0EA2,REPORT_f  Report the error if at end

POP  BC

DJNZ 0E6A,GET_FREE2    Read next byte.

CALL 0F48,POSN_BC      Position the 'CAT'-file at

CALL 0EE3,READ_REC     BC (1) and read the record

LD   DE,+0000          DE := start-record of gap.

PUSH DE                Stack it.

On entry the stack holds the record-number of the file after the

gap and DE holds the length of the gap. The length of  the  next

gap is calculated and used as the next maximal gap.

0E7D FREE_LOOP PUSH DE                DE = length of max gap.

LD   HL,(5CA0)         HL := last block of file.

LD   A,H

OR   L

INC  A                 Endmarker of 'CAT'-file?

JR   Z,0E9D,FREE_EXIT  Then exit.

PUSH HL                Store last block position.

INC  BC                Increase record-number.

CALL 0EE3,READ_REC     Read the next record.

LD   HL,(5C9E)         HL := start of new file.

POP  DE                DE := end of previous file

AND  A

SBC  HL,DE             HL := length of gap.

POP  DE                DE := length of max gap.

SBC  HL,DE             New maximum found?

JR   C,0E7D,FREE_LOOP  Loop back if not.

0E97 NEW_MAX   ADD  HL,DE             Restore gap-length.

EX   DE,HL             Pass it to DE.

POP  HL                Delete 'old' max gap.

PUSH BC                BC = start of 'new' gap.

JR   0E7D,FREE_LOOP    Loop back.

0E9D FREE_EXIT POP  BC                BC := length of max gap.

DEC  BC

POP  DE                DE := start of the gap.

DEC  DE

RET

0EA2 REPORT_f  RST  0008,SH_ERROR     Report 'No room on disk'.

DEFB +2F

THE 'MAKE A FILE IN THE CATALOG' SUBROUTINE

On entry HL holds the  first  block-number,  DE  the  number  of

records in the 'CAT'-file, BC  the  record-number  of  the  file

after the gap, mem-0 the size of the gap in bytes and mem-1  the

required length of the file.

0EA4 MAKE_FILE PUSH BC                BC = record after gap.

PUSH DE                DE = no. records in 'CAT'.

LD   (5C9E),HL         HL = first block.

PUSH HL

CALL 0F51,STCK_SIZE    Stack the block-size.

RST  0028,FP_CALC      block-size (say B)

DEFB +0F               (lengthbyte)

DEFB +E1,get-mem-1     B, length (say L)

DEFB +37,greater-0     B, (0/1)  (file-size > 0?)

DEFB +00,jump-true              (then don't exit)

DEFB +04,TO 0EB6,MAKE_2

DEFB +38,end-calc      B

XOR  A                 Clear A and carry flag.

RET                    Jump indirectly to 0EBC.

0EB6 MAKE_2    DEFB +E1,get-mem-1     B, L

DEFB +A1,stk-one       B, L, 1

DEFB +03,subtract      B, L - 1     (also byte 0)

DEFB +01,exchange      L - 1, B

DEFB +32,n-mod-m       (file-size MOD block-size)

DEFB +38,end-calc

0EBC MAKE_3    JR   NC,0EA2,REPORT_f  Report error if size=0.

RST  0010,CAL_SPEC

DEFW +1E99,FIND_INT2   BC := no. blocks in file.

POP  HL                HL := first block of file.

ADD  HL,BC             HL := last block of file.

LD   (5CA0),HL         Store it.

RST  0010,CAL_SPEC     BC := no.  bytes  in  last

DEFW +1E99,FIND_INT2   block - 1.

LD   (5C9C),BC         Store it too.

LD   DE,+FFE7          This is -25d.

CALL 0FA1,HL:=IX+DE    HL := channel start + 8.

LD   DE,+5CA2          DE := start of file-name.

LD   BC,+000A          10 characters in a name.

LDIR                   Copy the file-name too.

POP  HL                HL := no. records in 'CAT'

POP  BC                BC := record-number of new

CALL 0F07,MAKE_FREE    file. Make room for it.

CALL 0F48,POSN_BC      Position the 'CAT'-file.

THE 'READ A FILE-RECORD FROM THE CATALOG' SUBROUTINE

On entry BC holds the record-number of the file to be read.  The

16 bytes of the current record are stored at address  5C9C,  and

the next record is made the current.

0EE3 READ_REC  PUSH BC                Store the record-number.

LD   HL,5C9C           HL := startaddress of data

LD   B,+10             16 bytes in a record.

0EE9 READ_REC2 PUSH BC                Store the counter.

PUSH HL                Store load-address.

CALL 0F3F,READ_CAT     Read a byte.

POP  HL                Retrieve load-address  and

LD   (HL),A            store the read byte.

INC  HL                Point to next address.

POP  BC                Restore counter.

DJNZ 0EE9,READ_REC2    Loop back until done all.

POP  BC                Restore the record-number.

RET

THE 'WRITE A FILE-RECORD TO THE CATALOG' SUBROUTINE

The file-record stored at address 5C9C to  5CAB  is  written  to

disk.

0EF6 WRITE_REC LD   HL,+5C9C          HL := startaddress of data

LD   B,+10             16 bytes in a record.

0EFB WRITE_RC2 PUSH BC                Store the counter.

LD   A,(HL)            Get a byte to write.

INC  HL                Point to next address.

PUSH HL                Store save-address.

CALL 0B6E,WRITE_CT     Write the byte to disk.

POP  HL                Restore save-address.

POP  BC                Restore counter.

DJNZ 0EFB,WRITE_RC2    Loop back until done all.

RET

THE 'MAKE OR FREE SPACE IN THE CATALOG' SUBROUTINE

The routine is called on three  occations,  the  first  when  an

entry should be deleted, the second  when  an  entry  should  be

created, and the third when  an  entry  should  be  replaced  by

another. When a file is  saved  with  a  name  that  is  already

present on the disk, the old file-record should be  deleted,  or

overwritten. When creating an entry, HL holds the  total  number

of file-records in the 'CAT'-file and BC  the  record-number  of

the file to be created. When deleting an  entry,  BC  holds  the

record-number of the endmarker and HL the record-number  of  the

file to be deleted. When a entry should be  replaced,  HL  holds

the record-number of the second file with the same name  and  BC

the record-number of the file to be created. In all three cases,

BC points to the record to be moved and the moving  is  finished

when BC reaches HL.

0F07 MAKE_FREE AND  A

SBC  HL,BC

ADD  HL,BC             If an entry should be cre-

JR   NC,0F0E,MK_FR_2   ated then skip.

DEC  BC

0F0E MK_FR_2   PUSH BC                BC = moving pointer.

PUSH HL                HL = end pointer.

The record is found and exchanged with the entry in memory.

0F10 MK_FR_3   CALL 0F48,POSN_BC      Position the 'CAT'-file.

0F13 MK_FR_4   PUSH BC                Store this position.

LD   DE,5C9C           DE := start in memory.

LD   B,+10             16 bytes in a record.

0F19 MK_FR_5   PUSH BC                Store the counter.

PUSH DE

CALL 0F3F,READ_CAT     Read a byte from the 'CAT'

POP  DE

LD   B,A               Switch this byte with  the

LD   A,(DE)            memory contents.

LD   (HL),A

LD   A,B

LD   (DE),A

INC  DE                Point to next byte in mem.

SET  0,(IX+20)         Signal: change made.

POP  BC                Restore the counter.

DJNZ 0F19,MK_FR_5      Loop back until done all.

POP  BC                BC := moving pointer.

POP  HL                HL := end pointer.

PUSH HL                Store end again.

AND  A

SBC  HL,BC             More records to go?

JR   Z,0F3C,MK_FR_8    Step if not.

JR   C,0F39,MK_FR_7    Step if not creating.

0F36 MK_FR_6   INC  BC                Move to  the  next  record

JR   0F13,MK_FR_4      and repeat.

0F39 MK_FR_7   DEC  BC                Move  to  previous  record

JR   0F10,MK_FR_3      and repeat.

0F3C MK_FR_8   POP  HL                HL = end pointer.

POP  BC                BC = moving pointer.

RET

THE 'READ FROM THE 'CAT' CHANNEL' SUBROUTINE

The pointers in the channel descriptor  are  used  to  determine

which byte is to be read. On exit the A register holds this byte

and HL the address in the buffer of that byte.

0F3F READ_CAT  CALL 0B7C,READ_CT      Read a byte.

RET  C                 Return if no error occured

RST  0008,SH_ERROR     Report 'Disk I/O error'.

DEFB +2F

THE 'POSITION THE 'CAT'-FILE AT THE START' SUBROUTINE

The 'start' is signalled and the file is positioned.

0F45 POS_START LD   BC,+0001          Record-number := 1.

THE 'POSITION THE 'CAT'-FILE AT BC' SUBROUTINE

On entry BC holds the record-number. This number is stacked  and

the channel is positioned.

0F48 POSN_BC   PUSH BC                BC = record-number.

RST  0010,CAL_SPEC     Move it to the  calculator

DEFW +2D2B,STACK_BC    stack as well.

CALL 0BB8,POSN_M       Position the 'm' channel.

POP  BC                BC := record-number.

RET

THE 'STACK THE BLOCK-SIZE' SUBROUTINE

The block-size is fetched from the channel descriptor and stored

on the calculator stack.

0F51 STCK_SIZE LD   C,(IX+12)         BC := block-size.

LD   B,(IX+13)

RST  0010,CAL_SPEC     Stack it.

DEFW +2D2B,STACK_BC

RET

THE 'END OF FILE TEST' SUBROUTINE

A test  is made whether the next  byte  may  be  read  from  the

channel and when so,  HL  is  returned  pointing  to  the  byte.

(possibly after saving the current block and/or loading the next

block).

0F5B ENDOFFILE LD   E,(IX+1E)         DE := position in  current

LD   D,(IX+1F)         block.

LD   C,(IX+1C)         BC := current block.

LD   B,(IX+1D)

LD   L,(IX+1A)         HL := last block.

LD   H,(IX+1B)

AND  A

SBC  HL,BC             Past the last block?

CCF                    (signal: error)

RET  NC                Then return.

JR   NZ,0F7F,EOF_3     Skip if current <> last.

0F74 EOF_2     LD   L,(IX+16)         HL := number of  bytes  in

LD   H,(IX+17)         the last block.

AND  A

SBC  HL,DE             Past last position in last

CCF                    block? (signal: error)

RET  NC                Then return.

0F7F EOF_3     PUSH DE                Store position.

LD   L,(IX+12)         HL := block-size.

LD   H,(IX+13)

SCF                    Include byte 0.

SBC  HL,DE             Past the end of the block?

JR   NC,0F94,EOF_5     Skip if not.

0F8B EOF_4     INC  BC                Load the next block.

CALL 0FAA,READ_NEXT

LD   DE,+0000          Position in block := 0.

POP  BC                Erase unwanted stack entry

PUSH DE                Store position.

0F94 EOF_5     CALL 0FA1,HL:=IX+DE    HL := address of byte.

POP  BC                BC := current position.

INC  BC                BC := new current position

LD   (IX+1E),C         Store it.

LD   (IX+1F),B

SCF                    Signal: succes.

RET

THE 'HL := IX + DE + 33' SUBROUTINE

On entry IX holds the start of the channel and DE a  value  that

has to be added to IX and stored in HL. On  exit  HL  holds  the

required  position  (depending  on  DE  only)  in  the   channel

descriptor. This routine is used to make HL point to the various

variables in the descriptor (when -34 < DE < 0) or to a location

in the buffer (when DE >= 0).

0FA1 HL:=IX+DE PUSH IX

POP  HL                HL := IX.

ADD  HL,DE             HL := IX + DE.

LD   DE,+0021          DE := 33.

ADD  HL,DE             HL := IX + DE + 33.

RET

THE 'READ NEXT BLOCK FROM DISK' SUBROUTINE

On entry BC holds the number of the block to be loaded. If BC is

not the current block and less than or equal to the  last  block

and input is allowed, then the block is loaded from disk.

0FAA READ_NEXT LD   L,(IX+1C)         HL := current block.

LD   H,(IX+1D)

AND  A

SBC  HL,BC             BC = current block?

RET  Z                 Ready if so.

0FB4 READ_NXT2 PUSH BC                Store the 'new' block num-

CALL 0C63,PUT_BLOCK    ber while saving the 'old'

POP  BC                current block on disk.

BIT  3,(IX+02)         Input allowed?

JR   Z,0FDE,RD_N_END   Skip if not.

0FBF READ_NXT3 LD   L,(IX+1A)         HL := last block.

LD   H,(IX+1B)

AND  A

SBC  HL,BC             BC = past last block?

JR   C,0FDE,RD_N_END   Skip if so.

0FCA READ_NXT4 PUSH BC                Store block-number.

LD   BC,+0200          Signal: read a whole block

LD   DE,+0000

CALL 0FA1,HL:=IX+DE    HL := start of buffer.

EX   DE,HL             Pass it to DE.

POP  HL                HL := block-number to load

PUSH HL                Keep a copy.

LD   A,(IX+07)         A := drive-number.

CALL 0FE5,CAL_PHY      Load the block.

0FDE RD_N_END  POP  BC                BC := block-number.

LD   (IX+1C),C         Store it.

LD   (IX+1D),B

RET

THE 'CALL A PHYSICAL DEVICE' SUBROUTINE

On entry A holds the drive-number or the device-number (+81  for

the printer port, +82 for the joystick port)  and  B  holds  the

offset (0 for write, 2 for read, 4 for inquire and 6 for format)

See the CAL_PHY subroutines for  more  information  on  how  the

registers should be set up.

0FE5 CAL_PHY   PUSH IX                Store  IX  while   calling

CALL 0FED,CAL_PHY_2    further.

POP  IX

RET

IX is made to hold the startaddress of  the  disk-info  for  the

correct  drive  and  HL  the  startaddress  of  the  appropriate

routine. A jump is made to that routine.

0FED CAL_PHY_2 PUSH HL                Save main registers.

PUSH DE

PUSH BC

LD   B,+FF             Signal: no subtables.

RST  0030,LOOKUP       Lookup the startaddress of

DEFB +0A               the disk-info in the table

PUSH HL

POP  IX                IX := start of disk-info.

POP  BC                B := the offset.

PUSH BC                Store it again.

RST  0030,LOOKUP       Lookup the startaddress in

DEFB +08               the device table.

POP  BC                Restore the main registers

POP  DE

EX   (SP),HL           Jump  indirectly  to   the

RET                    handling routine.

The 'CODE' subtable of table #04

0FFF TAB_4:COD DEFW +101E,WRITE_COD   Write.

DEFW +1014,READ_CODE   Read.

DEFW +100B,OPEN_CODE   Open.

DEFW +001C             Close (return).

DEFW +0532,USE_M1      Length (use -1).

DEFW +102D,POSN_CODE   Position.

The 'CODE' channel is 9 bytes long:

bytes 0 and 1 : the output address.

bytes 2 and 3 : the input address.

byte  4       : the device-name (upper case when permanent).

bytes 5 and 6 : the channel-length.

bytes 7 and 8 : the pointed address.

THE 'OPEN THE 'CODE' CHANNEL' SUBROUTINE

The space is created and the channel-info checked for validity.

100B OPEN_CODE LD   HL,+0009          Reserve 9 bytes.

CALL 0818,MAKE_CHAN    Make the channel.

DEFB +80               No check on extra parms.

DEFB +01               Filename of 1 char allowed

RET

THE 'READ FROM THE 'CODE' CHANNEL' SUBROUTINE

The pointer is fetched, the byte is read and  the  next  address

pointed to. On exit A holds the read byte.

1014 READ_CODE LD   B,(IX+08)         BC := pointed address.

LD   C,(IX+07)

LD   A,(BC)            The actual read.

SCF                    Signal: succes.

JR   1025,CODE_END     Step forward.

THE 'WRITE TO THE 'CODE' CHANNEL' SUBROUTINE

On entry A holds the byte to be written. The pointer is fetched,

the byte written and the pointer incremented.

101E WRITE_COD LD   B,(IX+08)         BC := pointed address.

LD   C,(IX+07)

LD   (BC),A            The actual write.

1025 CODE_END  INC  BC                Increment pointer.

1026 CODE_END2 LD   (IX+08),B         And store it again.

LD   (IX+07),C

RET

THE 'POSITION THE 'CODE' CHANNEL' SUBROUTINE

On entry the calculator stack holds the position. The  value  is

fetched and stored in the descriptor.

102D POSN_CODE RST  0010,CAL_SPEC     BC := (new) position.

DEFW +1E99,FIND_INT2

JR   1026,CODE_END2    Step back to store it.

The 'd' subtable of table #04

1032 TAB_4:D   DEFW +104E,WRITE_D     Write.

DEFW +1060,READ_D      Read.

DEFW +103E,OPEN_D      Open.

DEFW +1076,CLOSE_D     Close.

DEFW +106E,LENGTH_D    Length.

DEFW +0BF3,REPORT_J    Position (error-message).

The 'd' channel is 9 bytes long:

bytes 0 and 1 : the output address.

bytes 2 and 3 : the input address.

byte  4       : the device-name (upper case when permanent).

bytes 5 and 6 : the channel-length.

byte  7       : the output drive-number.

byte  8       : the input drive-number.

The 'd' channel is only used by the  MOVE  command,  however  it

should act as a normal channel; you should be able to  read  and

write to it. The MOVE-command first reads  the  bytes  from  the

input channel. This way the input drive-number is loaded as data

Next the data is stored  in  the  output  channel.  Finally  the

channel is closed.  The  close-routine  fetches  the  input-  an

output drive-numbers and moves the data from the input-drive  to

the output-drive, but only when the drives have the same format.

THE 'OPEN THE 'D' CHANNEL' SUBROUTINE

The channel is created and the parameters checked for validity.

103E OPEN_D    LD   HL,+0009          Reserve 9 bytes.

CALL 0818,MAKE_CHAN    Make the channel.

DEFB +81               A drive-number is needed.

DEFB +00               But no file-name allowed.

LD   (IX+08),+00       Signal: no input drive-num

ber present.

JP   C,0902,REPORT_A   Report 'Invalid  argument'

RET                    if an error occured.

THE 'WRITE TO THE 'D' CHANNEL' SUBROUTINE

On entry A holds the input drive-number. It is stored as byte  8

of the output channel. Only 1 byte is written. If  more  than  1

byte is written, then the channel  is  not  being  used  by  the

'MOVE' command, so an error should be reported.

104E WRITE_D   LD   B,(IX+08)         Has the input drive-number

INC  B                 already been specified?

DEC  B

JR   NZ,1059,WR_D_ERR  Then report the error.

LD   (IX+08),A         Store the drive-number.

RET

1059 WR_D_ERR  LD   (IX+07),+00       Clear the output drive-num

JP   0BF3,REPORT_J     ber  and  report  'Invalid

I/O device'.

THE 'READ FROM THE 'D' CHANNEL' SUBROUTINE

A is made to hold the input drive-number. If A = 0  then  it  is

the second byte read and the end of the input-file is signalled.

1060 READ_D    LD   A,(IX+07)         A := output drive-number.

AND  A                 Skip if no drive-number is

JR   Z,106C,RD_D_ERR   present.

LD   (IX+07),+00       Clear output drive-number.

SCF                    Signal: success.

RET

106C RD_D_ERR  INC  A                 A := 1; signal: no success

RET

THE 'LENGTH OF THE 'D' CHANNEL' SUBROUTINE

The length of the channel is stored on the calculator stack.  It

is always 2^31.

106E LENGTH_D  RST  0028,FP_CALC      -

DEFB +07               (lengthbyte)

DEFB +34,stk-data      2^31

DEFB +00

DEFB +50, exponent +A0

DEFB +00, (+00,+00,+00)

DEFB +38,end-calc      2^31

RET

THE 'CLOSE THE 'D' CHANNEL' SUBROUTINE

On entry IX holds the start of  the  channel.  If  this  is  the

channel that is used as the output channel, then  byte  7  holds

the output drive-number and byte 8  the  input  drive-number.  A

buffer is created in the workspace which is  made  as  large  as

possible. The buffer is filled with the data from the input disk

and emptied on the output disk. After all the  used  blocks  are

moved, the catalog-file is adapted to the new situation. All the

blocks are stored on consecutive block-numbers, so there is just

one gap; at the end. The pointers in the catalogfile still point

to the old positions of the files, so they have to  be  altered.

This is done at the end of the routine.

1076 CLOSE_D   LD   A,(IX+07)         A := output drive-number.

AND  A                 Exit if no drive-number is

RET  Z                 specified.

LD   B,A               Store the drive-number.

LD   A,(IX+08)         A := input drive-number.

AND  A                 Exit too if no  drive-num-

RET  Z                 ber is present here.

CP   B                 input disk = output disk?

JR   Z,10B5,CLOSE_DS   Then skip.

1084 CLOSE_D2  PUSH BC                Store output drive-number.

LD   BC,+0400          Signal: inquire.

CALL 0FE5,CAL_PHY      Inquire input-disk.

POP  AF                A := output drive-number.

PUSH IX                IX = input-channel start.

PUSH AF                A = output drive-number.

PUSH BC                BC = block-size input-disk

PUSH HL                HL = no. blocks input-disk

LD   BC,+0400          Signal: inquire.

CALL 0FE5,CAL_PHY      Inquire output-disk.

POP  DE                DE := no. blocks input.

AND  A                 Test if the input and out-

SBC  HL,DE             put disks  have  the  same

109B REPORT_n2 JP   NZ,0BF6,REPORT_n  no. blocks. Report  'Wrong

disk' if not.

POP  HL                HL := block-size input.

SBC  HL,BC             Same block-size?

JR   NZ,109B,REPORT_n2 Report 'Wrong disk' if not

10A3 CLOSE_D3  LD   DE,+FFE5          This is -27.

CALL 11AD,CONV_CAT     Open the 'CAT'-file of the

output disk.

CALL 0D82,CHECK_2      Test if the disk has  been

formatted already.

JP   NZ,1545,REP_BREAK Jump out if not.

CALL 0368,CLOSE_CHN    Close the 'CAT'-file.

POP  BC

10B5 CLOSE_DS  POP  IX                IX := input-channel start.

PUSH IX

PUSH BC

10B8 CLOSE_D4  LD   DE,+FFE6          This is -26.

CALL 11AD,CONV_CAT     Open the 'CAT'-file of the

the input-disk.

RST  0010,CAL_SPEC     Clear the workspace.

DEFW +16BF,SET_WORK

RST  0010,CAL_SPEC     BC := free memory.

DEFW +1F1A,FREE_MEM

LD   HL,+FF00          As large as possible.

SBC  HL,BC             HL := bytes free in memory

LD   B,H               Copy it to BC.

LD   C,L

LD   E,(IX+12)         DE := block-size.

LD   D,(IX+13)

SBC  HL,DE             Enough room for a block?

JR   NC,10D7,CLOSE_D5  Then skip.

LD   B,+FF             Try with less room.

10D7 CLOSE_D5  RST  0010,CAL_SPEC     Make the room.

DEFW +0030,BC_SPACES

LD   HL,+FFFF          Store startblock on disk.

PUSH HL                (-1)

The blocks on the disk that are used are loaded into  the  work-

space until there are no more blocks, or the workspace is full.

10DE LOAD_LOOP LD   DE,(WORKSP)       DE := start of workspace.

LD   BC,+0000          BC := blocks loaded (=0)

10E5 LOAD_LP2  PUSH BC

CALL 1176,AT_END?      Load a block if available.

POP  BC

JR   C,10FF,SAVE_LOOP  Step if not.

INC  BC                Include block in counter.

PUSH DE                Store current position.

LD   L,(IX+12)         HL := block-size.

LD   H,(IX+13)

ADD  HL,DE             HL := end of next block.

LD   DE,(STKBOT)       DE := end of workspace.

AND  A                 Enough  room  for  another

SBC  HL,DE             block?

POP  DE                (Restore current position)

JR   C,10E5,LOAD_LP2   Then loop back.

The data is saved on  consecutive  blocks  on  the  output-disk.

There are BC blocks loaded in the workspace.

10FF SAVE_LOOP LD   DE,(WORKSP)       DE := start of workspace.

POP  HL                HL := startblock on disk.

1104 SAVE_LP2  LD   A,B               No (more) blocks to save?

OR   C

JR   Z,1122,MOVE_LOOP  Then step out.

DEC  BC                One block less to go.

POP  AF                A := output drive-number.

PUSH AF

INC  HL                HL := new startblock disk.

PUSH HL                Store it.

PUSH BC                BC = no. blocks to go.

PUSH DE                DE = position in workspace

LD   BC,+0000          Signal: write a full block

CALL 0FE5,CAL_PHY      Save the block.

POP  HL                HL := pos. in workspace.

LD   C,(IX+12)         BC := block-size.

LD   B,(IX+13)

ADD  HL,BC             HL := start of next block.

EX   DE,HL             Pass it to DE.

POP  BC                BC := no. blocks to go.

POP  HL                HL := startblock on disk.

JR   1104,SAVE_LP2     Loop back for the next one

Data is moved from the input- to the output-disk. If the end  of

the catalog-file has not yet been reached, then there  is  still

data to be moved.

1122 MOVE_LOOP POP  AF                A := output drive-number.

PUSH AF

PUSH HL                HL = last block saved.

LD   HL,(5CA0)

LD   A,H

AND  L

INC  A                 Last block in the catalog?

JR   NZ,10DE,LOAD_LOOP Loop back if not.

Now the input channel can be closed. The records in  the  'CAT'-

file on the output-disk still have to be made to  point  to  the

right positions on the disk. All the data in the files  take  up

one big space without gaps, but the records are still  the  same

as they were on the input-disk, possibly with gaps between them,

so the catalog still has to be compressed.

112D CAT_LOOP  POP  HL                Clear the stack.

POP  AF

CALL 0368,CLOSE_CHN    Close the input channel.

POP  IX                IX := start of new  output

PUSH IX                channel.

LD   DE,+FFE5          This is -27.

CALL 11AD,CONV_CAT     Create a catalog channel.

LD   DE,+0000          DE := gap-length (0).

LD   BC,+0002          BC := record-number (2).

Ajust the start-block.

1142 AJST_STRT LD   HL,(5C9E)         HL := old start-block.

ADD  HL,DE             Add gap-length.

LD   (5C9E),HL         Store new start-block.

Ajust the end-block.

1149 AJUST_END LD   HL,(5CA0)         HL := old end-block.

ADD  HL,DE             Add gap-length.

LD   (5CA0),HL         Store new end-block.

PUSH HL                Save end of old block.

PUSH BC                Save record-number.

DEC  BC                Correct record-number.

CALL 0F48,POSN_BC      Position the 'CAT'-file.

CALL 0EF6,WRITE_REC    Write record to disk.

CALL 0EE3,READ_REC     Read the next record.

POP  BC                BC := record-number.

INC  BC                Point to next record.

Form the next gap-length.

115E NEXT_GAP  POP  HL                HL := old end-block.

LD   DE,(5C9E)         DE := new start-block.

AND  A

SBC  HL,DE             HL := - gap-length.

INC  HL                HL := 1 - gap-length.

EX   DE,HL             Pass it to DE.

LD   HL,(5CA0)         HL := end-block.

LD   A,H

AND  L

INC  A                 Last block done?

JR   NZ,1142,AJST_STRT Loop back if not.

1170 MOVE_EXIT CALL 0368,CLOSE_CHN    Close the 'CAT' channel.

POP  IX                Clear the stack.

RET

THE 'AT-END TEST AND LOAD THE NEXT BLOCK' SUBROUTINE

Addresses #5C9C to #5CAB are used  to  hold  the  current  file-

record. Address #5C9E holds the number of  blocks  that  can  be

loaded into the buffer. After loading that block, the  value  on

this address is increased. When the last block has been  loaded,

the next file-record should be loaded from the catalog.

1176 AT_END?   LD   BC,(5C9E)         BC := current block.

LD   HL,(5CA0)         HL := last block.

AND  A

SBC  HL,BC             Current block <> last?

JR   NC,1191,LOAD_NEXT Step if so.

PUSH DE                DE = startaddress to load.

CALL 0EE3,READ_REC     Find next file-record.

POP  DE

LD   HL,(5CA0)         HL := last block number.

LD   A,H

AND  L

INC  A                 Is it the end-marker?

JR   NZ,1176,AT_END?   Load the  first  block  of

the next file if not.

SCF                    Signal: no success.

RET

The current block is loaded and  the  next  block  is  made  the

current block.

1191 LOAD_NEXT LD   H,B               HL := current block.

LD   L,C

INC  BC                BC := next block.

LD   (5C9E),BC         Store it in the descriptor

LD   BC,+0200          Signal: load a full block.

LD   A,(IX+07)         A := drive-number.

PUSH DE                DE = startaddress to load.

CALL 0FE5,CAL_PHY      Load the block.

POP  HL                HL := startaddress to load

LD   C,(IX+12)         BC := block-size.

LD   B,(IX+13)

ADD  HL,BC             HL := new startaddress.

EX   DE,HL             Pass it to DE.

AND  A                 Signal: success.

RET

THE 'OPEN A 'CAT'-FILE ON THE CURRENT 'M' CHANNEL' SUBROUTINE

Byte 7 or 8 of the channel-descriptor  holds  the  drive-number.

This byte is used as the parameter block for the creation of the

'CAT'-file (Together with the  previous  which  has  no  meaning

here). On entry DE holds the displacement in the descriptor (-27

or -26), so the first CALL makes HL point to the right byte -1.

11AB CONV_CAT  CALL 0FA1,HL:=IX+DE    HL := parameter start.

EX   DE,HL             It is needed in DE.

LD   BC,+0002          BC := parameter length.

CALL 0DCC,OPEN_CAT     Create the 'CAT'-file.

LD   (IX+04),+EF       Device-name := 'CAT'.

RET

The 'j' subtable of table #04

11BC TAB_4:J   DEFW +0BF3,REPORT_J    Write (error-message).

DEFW +11D4,READ_J      Read.

DEFW +11C8,OPEN_J      Open.

DEFW +001C             Close (return).

DEFW +0BF3,REPORT_J    Length (error-message).

DEFW +0BF3,REPORT_J    Position (error-message).

The 'j' channel is 8 bytes long:

bytes 0 and 1 : the output address.

bytes 2 and 3 : the input address.

byte  4       : the device-name (upper case when permanent).

bytes 5 and 6 : the channel-length.

byte  7       : the joystick-number : 1 for QWERT and 67890.

2 for QWERT and 12345.

THE 'OPEN THE 'J' CHANNEL' SUBROUTINE

The channel is created.

11C8 OPEN_J    LD   HL,+0008          Reserve 8 bytes.

CALL 0818,MAKE_CHAN    Make the channel.

DEFB +FE               Status 1 or 2.

DEFB +00               No file-name allowed.

JP   C,0902,REPORT_A   Report an error if found.

RET

THE 'READ FROM THE 'J' CHANNEL' SUBROUTINE

The registers are setup to read from  the  joystick-port  and  a

jump is made to CAL_PHY.

11D4 READ_J    LD   B,+02             Signal: read.

LD   H,(IX+07)         H := status.

LD   A,+82             Signal: joystick port.

11D9 JP_PHY    JP   0FE5,CAL_PHY      Read the port.

THE 'FORMAT THE 'J' CHANNEL' SUBROUTINE

On entry DE points to the parameter block. The registers are set

up to format the 'j' channel and a jump is made to CAL_PHY.

11DE FORMAT_J  PUSH DE                DE = parameter start.

CALL 0863,TEST_C       Test the parameters.

DEFB +0F               status < 16.

DEFB +00               No file-name allowed.

JP   C,0902,REPORT_A   Report an error if found.

POP  HL                HL := parameter start.

INC  HL

LD   H,(HL)            H := status.

LD   B,+06             Signal: format.

JR   11D9,JP_PHY       Format the port.

The joystick port subtabe of table #08.

11EE TAB_8:J   DEFW +1246,REPORT_k    Write (error-message).

DEFW +11F6,READ_J      Read.

DEFW +001C             Inquire (return).

DEFW +122F,FORMAT_J    Format.

THE 'READ FROM THE 'J' CHANNEL' SUBROUTINE

On entry H holds the status-number of the 'j' channel.  H  is  1

for the QWERT-keys and interface 2 port 1, H is 2 for the QWERT-

keys and interface 2 port 2. Depending on whether the  joystick-

port was formatted yet (bit 7 of address #3000 SET), kempston or

interface 2 is read.

11F6 READ_J    LD   A,(3000)

AND  +80               Joystick-port formatted?

JR   NZ,1201,READ_J_F  Step if so.

11FD READ_J_NF IN   A,(+1F)           Read the kempston port.

SCF                    Signal: success.

RET

1201 READ_J_F  LD   A,H               Use interface 2.

RLCA                   Bit 0 or 1 is SET; move it

RLCA                   to bit 3 or 4 respectively

RLCA

OR   +E3               A holds +E7 (for H = 1) or

+F3 (for H = 2), so port 1

or 2 + QWERT-keys is used.

IN   A,(+FE)           Read the port.

CPL                    A bit is SET for pressed.

AND  +1F               Keep only bits 0 - 5.

DEC  H                 H = 2?

JR   NZ,1223,PORT_2    Then handle port 2.

The bits are moved to their positions according to port 1.

(move ...RLDUF to ...FUDRL)

120F PORT_1    SRL  A                 'Fire' pressed?

JR   NC,1215,PORT_1_NF Skip if not.

OR   +40               Include 'fire'.

1215 PORT_1_NF SRL  A                 'Up' pressed?

JR   NC,121B,PORT_1_NU Skip if not.

OR   +10               Include 'up'.

121B PORT_1_NU SRL  A                 'Down' pressed?

JR   NC,1221,READ_J_E  Skip if not.

OR   +04               Include 'down'.

1221 READ_J_E  SCF                    Signal: success.

RET

The bits are moved to their positions according to port 2.

1223 PORT_2    RRCA                   Swap left and right.

RRCA                   (bits 0 and 1)

SLA  A

RL   A

JR   NC,0221,READ_J_E

OR   +02

SCF                    Signal: success.

RET

THE 'FORMAT THE JOYSTICK PORT' SUBROUTINE

On entry H holds the status for formatting.  Bit  7  of  address

#3000 is returned SET when the joystick-port is formatted.

122F FORMAT_J  LD   A,H               A := status.

LD   HL,+3000

SET  7,(HL)            Signal: not formatted.

AND  A                 Status = 0?

RET  Z                 Then exit.

RES  7,(HL)            Signal: formatted.

RET

The disk subtable of table #08.

123A TAB_8:M   DEFW +1142,WRITE_M     Write.

DEFW +1248,READ_M      Read.

DEFW +1414,INQUIRE_M   Inquire.

DEFW +132C,FORMAT_M    Format.

THE 'WRITE TO DISK' SUBROUTINE

On entry HL holds the block number,  DE  the  startaddress  from

where the data should be written, C the number of bytes,  A  the

drive number and IX points to the start of the disk-info table.

1242 WRITE_M   LD   B,+AF             Signal: write.

JR   124A,RD_WR_M_1    Step to the routine.

1246 REPORT_k  RST  0008,SH_ERROR     Report 'Ram corrupt'.

DEFB +57

THE 'READ FROM DISK' SUBROUTINE

On entry HL holds the block number,  DE  the  startaddress  from

where the data should be loaded, C the number of  bytes,  A  the

drive number and IX points to the start of the disk-info table.

1248 READ_M    LD   B,+8C             Signal: read.

124A RD_WR_M_1 PUSH DE                DE = startaddress.

PUSH BC                C = number of bytes.

EXX                    Save alternate registers.

PUSH BC

PUSH DE

PUSH HL

EXX

PUSH HL                HL = block number.

The current disk number is fetched and when the the current  and

new drive have different drive numbers but  refer  to  the  same

physical drive, then the disks have to be swapped.

1252 RD_WR_M_2 LD   E,A               Store the drive number.

LD   A,(3000)          A := current drive number.

XOR  (IX+02)           Same side (left/right)?

AND  (IX+08)           Same drive?

JR   Z,127F,RD_WR_M_5  Then skip.

125E RD_WR_M_3 LD   HL,(CURCHL)       HL := start of channel.

PUSH HL                Store it.

PUSH DE                E = drive number.

LD   A,+FD             Open channel K.

RST  0010,CAL_SPEC

DEFW +1601,CHAN_OPEN

LD   A,+E2             Print 'Insert disk '

CALL 05EB,P_ERR_MSG

POP  BC                C := drive number.

LD   B,+00             BC := drive number.

RST  0010,CAL_SPEC     Print the drive number.

DEFW +1A1B,OUT_NUM_1

LD   A,+E3             Print ',  then  press  any

CALL 0DB1,PR_&_WAIT    key' and wait for a key.

CALL 1540,TEST_BREAK   Test the 'BREAK' key.

127B RD_WR_M_4 POP  HL                HL := start of channel.

RST  0010,CAL_SPEC     Set the channel flags.

DEFW +1615,CHAN_FLAG

127F RD_WR_M_5 CALL 14F1,TAB_START    HL := start of disk tables

LD   D,A               D := current drive number.

XOR  (IX+02)           Same side (left/right)?

AND  (HL)

PUSH AF                Store result.

LD   A,(HL)            Get primary info

OR   (IX+08)           OR virtual info

LD   E,A               E := 00000111 or 00001011

CPL                    A := 11111000 or 11110100

AND  D

LD   B,A

LD   A,E

AND  (IX+02)

OR   B

AND  +EF

LD   (3000),A

POP  AF

JR   Z,12AA,RD_WR_M_7

XOR  D

129E RD_WR_M_6 INC  HL

RRA

JR   NC,129E,RD_WR_M_6

LD   A,(HL)            Read current trackregister

LD   (2801),A          Pass it to the hardware.

INC  A                 If at end, then restore.

CALL Z,14D3

12AA RD_WR_M_7 POP  HL                HL := block number.

LD   C,(IX+04)         BC := no. extra blocks.

LD   B,(IX+05)

ADD  HL,BC             HL := physical block num.

LD   B,H               Copy it to BC.

LD   C,L

RST  0010,CAL_SPEC     Stack it.

DEFW +2D2B,STACK_BC

LD   A,(IX+01)         A := number of sectors.

RST  0010,CAL_SPEC     Stack it too.

DEFW +2D28,STACK_A

RST  0010,CAL_SPEC     blocknum MOD sectors

DEFW +36A0,n-mod-m

LD   A,(IX+00)         A := number of tracks.

RST  0010,CAL_SPEC     Stack it too.

DEFW +2D28,STACK_A

RST  0010,CAL_SPEC     (bl MOD sect) MOD tracks.

DEFW +36A0,n-mod-m     sect, track, side.

RST  0010,CAL_SPEC     A := side of disk.

DEFW +1E94,FIND_INT1

AND  A                 Handling side 1?

JR   Z,12DD,RD_WR_M_9  Then skip.

12D2 RD_WR_M_8 DEC  A                 Handling side 2?

JR   NZ,1304,REPORT_h  Report 'Disk I/O error' if

BIT  4,(IX+02)         not. Has disk 2 sides?

JR   Z,1304,REPORT_h   Report the error if not.

LD   HL,+3000          Signal: handling side 2.

SET  4,(HL)

12DD RD_WR_M_9 RST  0010,CAL_SPEC     B := track number.

DEFW +2307,STK_TO_BC   C := sector number.

LD   A,C               A := track number.

ADD  A,(IX+03)         Add extra sectors.

LD   (2802),A          Signal: go to sector 'A'.

LD   H,B               H := track number.

EXX                    Restore   the    alternate

POP  HL                registers.

POP  DE

POP  BC

EXX

LD   B,(IX+06)         B := number of retries.

LD   C,+1F

12F2 RD_WR_M10 LD   A,H               A := track number.

LD   (2803),A          Signal: go to track 'A'.

CALL 14D5,CL_CNTRL     Call disk-controller.

AND  +18               Found & no CRC errors?

JR   Z,1306,RD_WR_M11  Then skip.

PUSH BC                B = number of retries.

CALL 14D3,CL_RESTOR    Restore disk-controller.

POP  BC                Loop back if more  retries

DJNZ 12F2,RD_WR_M10    are allowed.

1304 REPORT_h  RST  0008,SH_ERROR     Report 'Disk I/O error'.

DEFB +2F

1306 RD_WR_M11 POP  BC                C := number of bytes.

LD   L,B               L := +AF  for  write,  +8C

LD   B,+00             for read.

LD   A,C

AND  A                 Read/write a whole block?

JR   NZ,1311,RD_WR_M12 Step if not.

CALL 1467,BC:=SIZE     BC := block-size.

1311 RD_WR_M12 POP  DE                DE := startaddress

LD   H,(IX+06)         H := number of retries.

1315 RD_WR_M13 PUSH BC

PUSH DE

PUSH HL

CALL 14FE,FIND_ADDR    Load the block.

POP  HL

POP  DE

POP  BC

AND  +7C               Return if no error occured

RET  Z

AND  +40               Error = 'write protected'?

JR   Z,1327,RD_WR_M14  Step if not.

1325 REPORT_f  RST  0008,SH_ERROR     Report 'Write protected'.

DEFB +2D

1327 RD_WR_M14 DEC  H                 Another retry allowed?

JR   NZ,1315,RD_WR_M13 Then try again.

RST  0008,SH_ERROR     Report 'Disk I/O error'.

DEFB +2F

THE 'FORMAT THE 'M' CHANNEL' SUBROUTINE

On entry IX points to the start of the subtable of table #0A for

the specified drive-number, in A. The disk is formatted, and the

extra block (boot-block) is formed and stored on the first block

on the first track.

132C FORMAT_M  PUSH AF                A = drive-number.

RST  0010,CAL_SPEC     Clear the work-space.

DEFW +16BF,SET_WORK

LD   B,+00

LD   C,(IX+01)         BC := number of sectors.

INC  BC

INC  BC

INC  BC

INC  BC                Make room for  (number  of

RST  0010,CAL_SPEC     sectors) + 4 bytes.

DEFW +0030,BC_SPACES

INC  DE                Step to the 3rd location.

INC  DE

LD   A,(IX+02)         A := disk-info flag.

AND  +C0               Keep only  the  block-size

RLCA                   (bits 6 and 7).

RLCA                   Copy it to bits 0 and 1.

LD   (DE),A            Store it in the work-space

INC  A                 0-3 := 1-4

LD   B,A               B := counter.

LD   A,+10

A will be made to hold 32 (for B=1), 64 (for B=2), 128 (for B=3)

or 0 (for B=4).

134A FORMAT_M2 ADD  A,A               A := (2 ^ B) * 16.

DJNZ 134A,FORMAT_M2

INC  DE                Store the  result  in  the

LD   (DE),A            4th location.

LD   B,(IX+01)         B := number of sectors.

1352 FORMAT_M3 LD   (HL),+F7          HL  pointed  to  the  last

DEC  HL                byte of the work-space.

DJNZ 1352,FORMAT_M3    Fill the space with F7's.

INC  HL                HL := 1st byte of sec-info

LD   D,(IX+0B)         D := interleave.

LD   B,(IX+01)         B := number of sectors.

LD   C,(IX+03)         C := no. extra sectors.

1361 FORMAT_M4 LD   (HL),C            Store it.

INC  C

PUSH DE                D = interleave.

1364 FORMAT_M5 CALL 14B9,SHIFTLEFT    Shift the buffer left.

DEC  D

JR   NZ,1364,FORMAT_M5 Repeat 'D' times.

LD   A,(HL)            A := first byte in buffer.

CP   +F7               Is it a F7?

CALL NZ,14B9,SHIFTLEFT Shift left if not.

POP  DE                D = interleave.

DJNZ 1361,FORMAT_M4    Next sector.

LD   DE,+0000          Start-sector = 0.

CALL 1478,SETUP_FMD    Setup the format-data.

LD   B,D

LD   C,E

RST  0010,CAL_SPEC

DEFW +0030,BC_SPACES

PUSH DE                DE = start of new space.

CALL 14D3,CL_RESTOR    Restore disk-controller.

POP  DE

1383 NEXTTRACK LD   HL,+3000

BIT  4,(IX+02)         Drive single sided?

JR   Z,1392,FORMAT_M6  Then skip.

SET  4,(HL)            Signal: side 2.

LD   A,+01

JR   1395,FORMAT_M7    Step forward.

1392 FORMAT_M6 RES  4,(HL)            Signal: side 1

XOR  A

1395 FORMAT_M7 LD   HL,(WORKSP)       Store the side in the  1st

LD   (HL),A            location of the work-space

INC  HL

LD   A,(2801)          A := current track number.

LD   (HL),A            Store it in the 2nd byte.

PUSH DE                DE = format-data start.

CALL 1478,SETUP_FMD    Setup the format-data.

POP  DE

PUSH DE

LD   A,(DE)

LD   B,A

INC  DE

LD   A,(DE)

EX   AF,AF'

LD   L,+FE             Signal: format.

CALL 14FE,FIND_ADDR    Format the described track

AND  +44               Disk write protected?

JP   NZ,1325,REPORT_f  Then report the error.

POP  DE                DE = format-data location.

LD   HL,+3000

BIT  4,(HL)            Double sided drive?

JR   NZ,1392,FORMAT_M6 Then format side 1 too.

LD   A,(2801)          A := current track.

INC  A

CP   (IX+00)           Is it the last one?

JR   Z,13D6,EX_BLOCK   Then step out.

13C4 FORMAT_M8 LD   C,+5B

PUSH DE

CALL 14D5,CL_CNTRL     Step-in the drive-head.

LD   L,(IX+0C)         L := number of retries.

13CD FORMAT_M9 CALL 14B9,SHIFTLEFT

DEC  L

JR   NZ,13CD,FORMAT_M9

POP  DE

JR   1383,NEXTTRACK    Handle the next track.

Now the boot-block is stored on the disk on real-block-number 0.

13D6 EX_BLOCK  LD   B,+06             Signal: entry 4.

CALL 14B1,LOOKUP_18    HL := start of routine.

LD   BC,+0080          Reserve 128 bytes.

LD   DE,(WORKSP)       Start of bytes.

PUSH DE

LDIR                   Copy boot-block to buffer.

POP  HL                HL := start of block.

PUSH HL

INC  HL                HL := start of parameters.

INC  HL

LD   A,(IX+00)         A := number of tracks.

LD   (HL),A            Store it.

INC  HL

LD   A,(IX+01)         A := number of sectors.

LD   (HL),A            Store it too.

INC  HL

LD   A,(IX+02)         A := disk-info flag.

AND  +D0               Keep only bits 4, 6 and 7.

LD   (HL),A            Store block-size and sides

INC  HL

LD   DE,(FRAMES)       Make the disk unique.

LD   (HL),E

INC  HL

LD   (HL),D

1401 SAVE_BOOT POP  DE                DE := start of block.

LD   HL,+0000          HL := real-block-number.

LD   C,(IX+04)         BC := - boot-location.

LD   B,(IX+05)

AND  A

SBC  HL,BC             HL := boot-location.

POP  AF

CALL 1242,WRITE_M      Store new configuration.

JR   1446,HL:=BLCKS    Jump to set registers.

THE 'INQUIRE THE DISK' SUBROUTINE

On entry C holds 0 when the info should be read from  the  disk,

and +80 when the info should come from the disk tables. A  holds

the drive-number and IX points to the start of the corresponding

subtable of table #0A.

1414 INQUIRE_M BIT  7,C               Get new configuration?

JR   NZ,1446,HL:=BLCKS Skip if not.

1418 GET_CNFG  LD   HL,+3001

BIT  1,(HL)            IC 6116 present?

JR   Z,1446,HL:=BLCKS  Skip if not.

141F GET_CNFG2 LD   HL,+0000          HL := real-block-number.

LD   C,(IX+04)         BC := - boot-location.

LD   B,(IX+05)

AND  A

SBC  HL,BC             HL := boot-location.

LD   C,+80             Load 128 bytes.

LD   DE,+2780          From address +2780.

PUSH AF                From drive 'A'.

CALL 1248,READ_M

POP  AF

AND  A

LD   HL,(2780)         Test  if  the  routine  is

LD   DE,+0518          still there. 0518 is JR +5

SBC  HL,DE

LD   HL,+2780          (HL := start of routine)

INC  HL

INC  HL                (HL := start of disk-data)

CALL Z,2780            If present, call it.

HL is made to hold the number of blocks on the disk.

1446 HL:=BLCKS LD   H,+00

LD   L,(IX+00)         HL := number of tracks.

LD   D,H

LD   E,(IX+01)         DE := number of sectors.

BIT  4,(IX+02)         Double sided disk?

JR   Z,1456,NEXT_1     Skip if not.

ADD  HL,HL             Count tracks of both sides

1456 NEXT_1    RST  0010,CAL_SPEC     HL := number of blocks.

DEFW +30A9,HL:=HL*DE

LD   C,(IX+04)         BC := no. extra blocks.

LD   B,(IX+05)

AND  A

SBC  HL,BC             HL := usable blocks number

DE is made to hold the number of blocks in the catalog-file.

1462 DE:=CATLN LD   D,+00

LD   E,(IX+0D)         DE := catalog length.

BC is made to hold the block-size.

1467 BC:=SIZE  LD   A,(IX+02)         A := disk-info byte.

AND  +C0               Keep only bits 6 and 7.

LD   BC,+0040          BC := 64.

146F BC:=SIZE2 SLA  C                 BC := 64*(2^((A/64)+1))

RL   B

SUB  +40

JR   NZ,146F,BC:=SIZE2

RET

THE 'SETUP THE DATA FOR FORMATTING' SUBROUTINE

First the data from the first subtable  of  table  #18  is  used

once, then (number of sectors) times the data  from  the  second

subtable and finally one time the data from the third subtable.

1478 SETUP_FMD LD   B,+00             Signal: entry 1.

CALL 148E,SETUP_2      Store the data.

LD   B,(IX+01)         B := number of sectors.

1480 NXTSECTOR PUSH BC                Store it.

LD   B,+02             Signal: entry 2.

CALL 148E,SETUP_2

CALL 14B9,SHIFTLEFT    Shift the buffer left.

POP  BC                Restore sector counter.

DJNZ 1480,NXTSECTOR

LD   B,+04             Signal: entry 3.

The data from subtable 'B' of table #18 is used.

148E SETUP_2   PUSH DE                DE = format-data start.

CALL 14B1,LOOKUP_18    Lookup table #18.

POP  DE

LD   B,(HL)            B := length-byte.

1494 NXT_FM_DT INC  HL

LD   A,(HL)            A := format-data.

CP   +F0               Test if +F0 <= A <= +F4

JR   C,94AC,ST_FN_DT   Simply store byte if not.

CP   +F5

JR   NZ,94AC,ST_FM_DT

PUSH HL

LD   HL,(WORKSP)       HL := start of buffer.

SUB  +F0               F0-F4 := 0-4.

PUSH BC                Store length-byte.

LD   B,+00

LD   C,A               Copy A to BC.

ADD  HL,BC             HL := location of byte 'A'

POP  BC                (restore length-byte)

LD   A,(HL)            Read byte 'A' from buffer.

POP  HL

14AC ST_FM_DT  LD   (DE),A            Store byte in format-data

INC  DE                Point to next location.

DJNZ 1494,NXT_FM_DT    Repeat 'length-byte' times

RET

14B1 LOOKUP_18 LD   A,(IX+0A)         A := drive density.

RST  0030,LOOKUP       Lookup   the   format-data

DEFB +18               startaddress.

LD   D,H               It is wanted in DE.

LD   E,L

RET

THE 'SHIFT FORMAT-DATA LEFT' SUBROUTINE

The format data for each sector  is  moved  left  (downwards  in

memory) one location.

14B9 SHIFTLEFT PUSH BC                Save HL and BC during this

PUSH HL                loop.

LD   HL,(WORKSP)       HL := start of sector-data

INC  HL                in the work-space.

INC  HL

INC  HL

INC  HL

LD   A,(HL)            A := first sector-data.

LD   B,+00

LD   C,(IX+01)         BC := number of sectors.

ADD  HL,BC             HL := end of sector-data.

LD   B,C               B := number of sectors.

14CA SHIFTLEF2 DEC  HL                Go down one location.

LD   C,(HL)            Read the entry in C.

LD   (HL),A            A = data previous sector.

LD   A,C               A := data this sector.

DJNZ 14CA,SHIFTLEF2    Repeat for all sectors.

POP  HL

POP  BC

RET

THE 'RESTORE THE DISK-CONTROLLER' SUBROUTINE

All controller error-registers are cleared, and  the  drive-head

is repositioned at track 0.

14D3 CL_RESTOR LD   C,+03

THE 'CALL THE DISK-CONTROLLER' SUBROUTINE

A is made to hold %00000000 (restore (enable spinup, no  verify) for C=+03), %00011100 (seek (disable spinup, verify for  C=+1F),

or %01011000 (step-in (disable spinup, update) for C=+5B). Bit 7 is SET if an NMI should occur. (see appendix D for more informa-

tion on the controller hardware).

14D5 CL_CNTRL  LD   A,(IX+07)         A := control register.

RRCA                   Swap nibbles.

RRCA

RRCA

RRCA

OR   +F0               Keep bits 0-3, SET others.

AND  C

CALL 1519,DO_M_I/O     Pass the command  directly

PUSH AF                to the controller.

CALL 14F1,TAB_START    Find track register buffer

AND  (HL)              of current drive.

14E7 NEXT_2    INC  HL

RRA

JR   NC,14E7,NEXT_2

LD   A,(2801)          A := current track.

LD   (HL),A            Store it.

POP  AF                A :=  result  of  hardware

RET                    call.

THE 'HL := START OF DISK TABLES' SUBROUTINE

This subroutine is used to made HL point to  the  start  of  the

disk-info tables.

14F1 TAB_START XOR  A

LD   B,+FF             Signal: no subtables.

RST  0030,LOOKUP       Find the  startaddress  of

DEFB +0A               the disk-info tables.

INC  HL

INC  HL

INC  HL

INC  HL

LD   A,(3000)          A := current drive number.

RET

THE 'FIND THE ADDRESS FOR THE NMI' SUBROUTINE

This subroutine is used to form the address of  the  routine  to

jump to with the NMI, since address +66 holds the instruction JP

(HL). A is set up as follows: bit 5 is RESET  for  read,  bit  6

RESET for write. When both are SET, this indicates format. Bit 7

is SET to trigger  the  NMI,  bit  1  is  RESET  when  a  write-

protection should be tested. L holds +8C for read, +AF for write

and +FE for format.

14FE FIND_ADDR LD   A,(IX+07)         Use only bits 0-3.

OR   +F0               Set bits 4-7.

AND  L                 Include the command bits.

LD   HL,+1886          HL := +1886,NMI_READ.

BIT  5,A               Read if bit 5 is RESET.

JR   Z,1513,GO_M_I/O

LD   L,+95             HL := +1895,NMI_WRITE.

BIT  6,A               Write if bit 6 is RESET.

JR   Z,1513,GO_M_I/O

LD   L,+9F             HL := +189F,NMI_FORMT.

1513 GO_M_I/O  DI                     No interrupts allowed.

CALL 1519,DO_M_I/O     Call the hardware.

EI                     Restore interrupt status.

RET

1519 DO_M_I/O  LD   (2800),A          Pass the command  directly

XOR  A                 to the hardware.

151E WAIT_NMI  DEC  A                 Wait a while for  the  NMI

JR   NZ,151E,WAIT_NMI  to finish. (4095 T states)

1520 WAIT_I/O  LD   A,(2800)          Is  the  diskdrive   still

BIT  0,A               doing I/O?

RET  Z                 Finished if not.

CALL 1548,TST_BREAK    Test the 'BREAK'-key.

JR   C,1520,WAIT_I/O   Loop back if not pressed.

152B BREAK_I/O LD   A,+D0             Stop the drive.

LD   (2800),A

LD   B,+70             Wait +70 interrupts (about

EI                     1.5 seconds).

1533 BREAK_WT  HALT

DJNZ 1533,BREAK_WT

CALL 14F1,TAB_START

OR   (HL)

XOR  (HL)

LD   (3000),A

JR   1544,BREAK_REP    Step forward.

THE 'TEST FOR 'BREAK' AND REPORT IF PRESSED' SUBROUTINE

The 'BREAK' key is tested and when it is pressed, then the error

message 'BREAK - CONT repeats' is printed.

1540 T/R_BREAK CALL 1548,TST_BREAK    Test the 'BREAK' key.

RET  C                 Ready if not pressed.

1544 BREAK_REP EI                     Ensure interrupt allowence

1545 REP_BREAK RST  0010,CAL_SPEC     Report the error from  the

DEFW +0552,REPORT_D    Spectrum ROM.

THE 'TEST FOR 'BREAK' PRESSED' SUBROUTINE

A test is made for the <CAPS SHIFT> and <SPACE> keys. If pressed

both, then the carry flag is returned RESET.

1548 TST_BREAK LD   A,+FE             First test <CAPS SHIFT>.

IN   A,(+FE)

RRCA

RET  C                 Exit if not pressed.

LD   A,+7F             Else test <SPACE> too.

IN   A,(FE)

RRCA                   This RESETS the carry flag

RET                    if <SPACE> is pressed too.

THE 'INITIALIZE THE IC 6116' SUBROUTINE

First a check is made whether the 8K ROM is not  currupted.  The

main table and all subtables are moved to  the  IC  6116  and  a

check is made whether the tables are still there.  If  so,  then

the presence of the IC is signalled to the hardware.

1554 INIT_RAM2 LD   HL,+7132          HL := check-sum.

LD   DE,+0000          DE := address to test.

LD   B,D               B := 0.

155B TEST_ROM  LD   A,(DE)            Read a byte.

LD   C,A               Pass it to BC.

ADD  HL,BC             Add it to the check-sum.

INC  DE                Point to the next byte.

LD   A,D               Last byte checked?

CP   +20

JP   NZ,155B,TEST_ROM  Loop back if not.

1565 TEST_ROM2 LD   A,H               Everything is still OK  if

OR   L                 the check-sum = 0.

JR   NZ,1533,BREAK_WT  Report an error if found.

1569 INIT_RAM3 LD   HL,+18AD          HL := start of tables - 4.

LD   BC,+0089          BC := length of tables + 4

LDIR                   Move them to the IC 6116.

LD   B,+89             +89 bytes to test.

1573 TEST_RAM  DEC  HL

DEC  DE

LD   A,(DE)            A := data in RAM.

CP   (HL)              Data in ROM = data in RAM?

LD   A,+05             (prepare for 'absent')

JR   NZ,1585,REP_RAM   Step out if not.

DJNZ 1573,TEST_RAM     Test next byte.

SET  1,A               A := +07.

1585 REP_RAM   LD   HL,+2023          Make alterations in RAM.

LD   (2013),HL         (Point to new subtable 0A)

LD   HL,+3001

LD   (HL),C            (3001) := %00000000

DEC  HL

LD   (HL),+FF          (3000) := %11111111

INC  HL

LD   (HL),A            (3001) := %000001b1

DEC  HL                b = IC 6116 present?

LD   (HL),+C0          (3000) := %11000000

RET

The addresses 1592 to 1707 are unused in version 2.2 and contain

+FF's.

THE 'PAGE IN' ROUTINE

This address can be reached in two ways.  Firstly  it  could  be

called directly, with the intension to  page  in  the  Discovery

ROM, secondly by the CLOSE_2 subroutine in the Spectrum ROM.

When coming from the CLOSE_2 subroutine, the  last value on  the

stack points to a position in the channel information  area  and

is between +5C16 and +5C36. Note that when the routine is called

to page in the Discovery ROM, this cannot be the  case,  as  the

returnaddress is the last entry then.

A test is made whether the Discovery ROM should be paged in,  or

a channel should be closed. When closing a channel, HL points to

the start if  this  channel  and  the  CLOSE-routines  from  the

Spectrum ROM are used. This is why some subtables of  table  #04

do not  contain  an  entry  to  close  the  channel.  The  first

instruction is fetched from the Spectrum ROM and is an  INC  HL.

That is why the  next  instruction  decreases  the  registerpair

again.

1708 PAGE_IN   NOP                    = INC HL.

DEC  HL                Compensate.

EX   (SP),HL           PUSH HL, POP returnaddress

PUSH AF                Store other registers too.

PUSH DE

PUSH HL                And the returnaddress.

LD   DE,+A3EA          DE := - 5C15.

ADD  HL,DE             HL >= 5C15?

JR   NC,171A,JUST_PAGE Skip if not.

LD   DE,+FFE0          DE := -32.

ADD  HL,DE             HL < 5C36?

JR   NC,171F,CLOSE_2   Then close a channel.

The routine was called to page in the Discovery ROM.

171A JUST_PAGE POP  HL                Restore the  stack  as  it

POP  DE                was before the call.

POP  AF

EX   (SP),HL

RET                    Finished paging.

The routine was called to close a channel.

171F CLOSE_2   POP  HL                Restore the  stack  as  it

POP  DE                was before the call.

POP  AF

EX   (SP),HL

LD   A,B               Channel already closed?

OR   C

JR   Z,1734,P_IN_EXIT  Then exit.

DEC  HL                HL := start of the channel

DEC  HL                to be closed.

DEC  HL

PUSH HL                Pass it to IX.

POP  IX

CALL 0368,CLOSE_CHN    Close the channel.

POP  HL                Clear stack (Spectrum ROM)

RST  0010,CAL_SPEC     Resume closing the channel

DEFW +16EB,CLOSE+6

1734 P_IN_EXIT JP   015A,OPEN_'S'     Open the 'S' channel.

DEFW +FFFF, +FFFF      Unused locations.

DEFW +FFFF, +FFFF

DEFW +FFFF, +FFFF

DEFW +FFFF, +FFFF

DEFB +FF

THE 'PAGE OUT' ADDRESS

When the Discovery ROM should be paged out, this address  is  to

be called. Although it is merely just  a  RET,  the  address  is

detected by the hardware, and the Spectrum ROM is paged in again

1748 PAGE_OUT  RET                    Page out the Discovery ROM

The '48K ramdisk' subtable of table #08

1749 TAB_8:5   DEFW +1763,WRITE_5     Write.

DEFW +1769,READ_5      Read.

DEFW +1751,FM/INQ_R    Inquire.

DEFW +1751,FM/INQ_R    Format.

THE 'FORMAT & INQUIRE THE RAMDISK' SUBROUTINE

The registers are set up correctly; no catalog should be written

1751 FM/INQ_R  LD   C,(IX+02)         BC := block-size.

LD   B,(IX+03)

LD   L,(IX+04)         HL := total no. blocks.

LD   H,(IX+05)

LD   E,(IX+06)         DE := no. blocks in 'CAT'

LD   D,+00

RET

THE 'WRITE TO THE 48K RAMDISK' SUBROUTINE

On entry DE holds the startaddress of the data to be written  to

the ramdisk, BC the length and HL the block number. The  address

within the ramdisk is calculated and the data moved.

1763 WRITE_5   CALL 1770,FD_ADD_48    Find the location.

LDIR                   Move the data.

RET

THE 'READ FROM THE 48K RAMDISK' SUBROUTINE

On entry DE holds the startaddress of the data to be  read  from

the ramdisk, BC the length and HL the block number. The  address

within the ramdisk is calculated and the data moved.

1769 READ_5    CALL 1770,FD_ADD_48    Find the location.

EX   DE,HL             Switch source/destination.

LDIR                   Move the data.

RET

THE 'FIND THE ADDRESS IN THE 48K RAMDISK' SUBROUTINE

Find the address of  block  HL  in  the  ramdisk.  First  HL  is

multiplied with the block-size, then the startaddress is added.

1770 FD_ADD_48 PUSH DE                Save startaddress.

LD   C,(IX+02)         BC := block-size.

LD   B,(IX+03)

LD   D,B               Copy it to DE.

LD   E,C

RST  0010,CAL_SPEC     Multiply HL by DE.

DEFW +30A9,HL:=HL*DE

LD   E,(IX+00)         DE := start of ramdisk.

LD   D,(IX+01)

ADD  HL,DE             HL := required address.

EX   DE,HL             Pass it to DE.

LD   HL,(RAMTOP)       Ramtop below start of  the

AND  A                 ramdisk?

SBC  HL,DE

POP  HL                (HL = memory, DE = disk)

RET  C                 Exit if so.

RST  0008,SH_ERROR     Report 'Disk I/O error'.

DEFB +2F

The '128K ramdisk' subtable of table #08.

178E TAB_8:6   DEFW +1796,WRITE_6     Write.

DEFW +179F,READ_6      Read.

DEFW +1751,FM/INQ_R    Inquire.

DEFW +1751,FM_INQ_R    Format.

THE 'WRITE TO THE 128K RAMDISK' SUBROUTINE

On entry DE holds the startaddress of the data to be written  to

the ramdisk, BC holds the length and HL the  block  number.  The

address within the ramdisk is calculated and the data moved.

1796 WRITE_6   CALL 17D3,FD_AD_128    Find the location.

PUSH HL                Store location.

PUSH BC                Copy start to HL.

POP  HL

LD   D,A

JR   17A4,RD_WR_R_1    Step forward.

THE 'READ FROM THE 128K RAMDISK' SUBROUTINE

On entry DE holds the startaddress of thedata to  be  read  from

the ramdisk, BC holds the length and HL the  block  number.  The

address within the ramdisk is calculated and the data moved.

179F READ_6    CALL 17D3,FD_AD_128    Find the location.

LD   E,A

PUSH BC                BC = startaddress.

17A4 RD_WR_R_1 LD   C,(IX+08)         BC := page port (+7FFD)

LD   B,(IX+09)

EXX

EX   (SP),HL           HL' := start/location.

PUSH DE                Stack DE' and BC'.

PUSH BC                The tack holds HL' too.

LD   C,(IX+02)         BC := block-size.

LD   B,(IX+03)

LD   D,A

DI

17B6 RD_WR_R_2 EXX

LD   A,D               Go to current ramdisk.

OUT  (C),A

LD   A,(HL)            Read byte into A'.

INC  HL                Point to next location.

EX   AF,AF'

LD   A,E               GO to ramdisk to read from

OUT  (C),A             or write to.

EXX

EX   AF,AF'            Get A' in A.

LD   (HL),A            Store the byte.

INC  HL                Point to next location.

DEC  BC                1 Less to go.

LD   A,B               Test if finished.

OR   C

JR   NZ,17B6,RD_WR_R_2 Loop back if not.

17C9 RD_WR_R_3 LD   A,D

EXX

OUT  (C),A             Go to current ramdisk.

EI

POP  BC                Clear the stack and resto-

POP  DE                re the registers.

POP  HL

EXX

RET

THE 'FIND THE ADDRESS IN THE 128K RAMDISK' SUBROUTINE

On entry HL holds the blocknumber in the RAM-disk.  On  exit  HL

holds the 'real' address in 128K-block 'D' and A will  hold  the

current 128K-block number.

17D3 FD_AD_128 XOR  A

LD   C,(IX+02)         BC := block-size.

LD   B,(IX+03)

17DA F_A_128_2 SRL  B                 AHL := blocknumber * size.

RR   C

JR   C,17E4,F_A_128_3

ADD  HL,HL

RLA

JR   17DA,F_A_128_2

17E4 F_A_128_3 LD   B,(IX+07)         AHL := AHL * 2^(IX+07)

LD   C,B

17E8 F_A_128_4 ADD  HL,HL

RLA

DJNZ 17E8,F_A_128_4

LD   B,C               HL := HL / 2^(IX+07)

17ED F_A_128_5 SRL  H

RR   L

DJNZ 17ED,F_A_128_5

LD   C,(IX+00)         BC := start of ramdisk.

LD   B,(IX+01)

ADD  HL,BC

PUSH IX                Store location, IX to HL.

EX   (SP),HL           HL := HL + 11 + A.

LD   BC,+000B

ADD  HL,BC

LD   C,A

ADD  HL,BC

LD   B,D               Start in BC.

LD   C,E

LD   D,(HL)            D := E := (HL) :=  ramdisk

LD   E,D               to write to.

POP  HL                HL := location.

LD   A,(IX+0A)         A := current ramdisk.

BIT  4,(FLAGS)         If in 128K mode, then A :=

RET  Z                 (5B5C).

LD   A,(5B5C)

RET

The printerport subtable of table #08:

1814 TAB_8:T   DEFW +181C,WRITE_T     Write.

DEFW +1853,READ_T      Read.

DEFW +001C             Inquire (return).

DEFW +001C             Format (return).

THE 'WRITE TO THE PRINTERPORT' SUBROUTINE

On entry H holds the characer to be written.

181C WRITE_T   LD   A,H               A := character.

LD   HL,+3003

LD   (HL),+38          Clock on for bits program

DEC  HL

LD   (HL),+FF          Signal: all databits = out

DEC  HL

RES  2,(HL)            Clock on for BUSY program

DEC  HL

RES  6,(HL)            Signal: BUSY = input.

INC  HL

SET  2,(HL)            Clock off for BUSY program

INC  HL

INC  HL

LD   (HL),+3C          Clock off for bits program

DEC  HL

LD   (HL),A            The actual write.

1834 WR_T_WAIT CALL 1540,T/R_BREAK    Test the 'BREAK' key.

LD   A,(3000)

BIT  6,A               Printer BUSY?

JR   NZ,1834,WR_T_WAIT Then wait until ready.

INC  HL

DI                     No interrupts while timing

LD   (HL),34           STROBE on.

LD   B,+05

1844 WR_T_STB  DJNZ 1844,WR_T_STB     Wait 66T states.

LD   (HL),+3C          STROBE off.

DEC  HL

DEC  HL

184A WR_T_ACK  CALL 1540,T_R_BREAK    Test the 'BREAK' key.

BIT  6,(HL)            Wait for ACK to go high.

JR   Z,184A,WR_T_ACK   (Don't wait for it  to  go

EI                     low again). Restore inter-

RET                    rupt status before return.

THE 'READ FROM THE PRINTERPORT' SUBROUTINE

On exit A holds the read character. If the read was successfull,

then the carry flag is returned SET.

1853 READ_T    CALL 1540,T/R_BREAK    Test the 'BREAK' key.

LD   HL,+3003

XOR  A

LD   (HL),A            Clock on for bits program.

DEC  HL

LD   (HL),A            Signal: all databits = in.

DEC  HL

RES  2,(HL)            Clock on for BUSY program.

DEC  HL

SET  6,(HL)            Signal: BUSY = output.

INC  HL

SET  2,(HL)            Clock off for BUSY program

DEC  HL

RES  6,(HL)            Signal: ready to read.

INC  HL

INC  HL

INC  HL

LD   (HL),+04          Clock off for bits program

BIT  6,(HL)            STROBE signal detected?

RET  Z                 Step out if not.

DEC  HL

LD   A,(HL)            The actual read.

DEC  HL

DEC  HL

SET  6,(HL)            Signal: BUSY.

INC  HL

SET  4,(HL)            Signal: ACK = output.

SET  5,(HL)            ACK on.

LD   B,+05

187E RD_T_ACK  DJNZ 187E,RD_T_ACK     Wait 66T states.

RES  5,(HL)            ACK off.

RES  4,(HL)            Signal: ACK = input.

SCF                    Signal: succesfully read.

RET

THE 'NMI READ A BYTE' SUBROUTINE

BC holds the number of bytes to be read, and DE the  address  to

be read into.

1886 NMI_READ  EX   AF,AF'

LD   A,B               No more bytes to be read?

OR   C

LD   A,(2803)          (get byte from hardware)

JP   Z,1892,NMI_READ2  Then skip.

LD   (DE),A            Store the read byte.

INC  DE                Point to next location.

DEC  BC                1 Byte less to go.

1892 NMI_READ2 EX   AF,AF'

RETN                   Return from NMI.

THE 'NMI WRITE A BYTE' SUBROUTINE

BC holds the number of bytes to be written, and DE  the  address

from which the data should be read.

1895 NMI_WRITE EX   AF,AF'

LD   A,(DE)            Get byte from memory.

LD   (2803),A          Give byte to hardware.

INC  DE                Point to next location.

DEC  BC                1 Byte less to go.

EX   AF,AF'

RETN                   Return from NMI.

THE 'NMI FORMAT A BYTE' SUBROUTINE

On entry DE holds the address in  the  format-data,  and  B  the

number of repetitions of the byte in the A' register.

189F NMI_FORMT EX   AF,AF'

LD   (2803),A          Give byte to hardware.

DJNZ 18AA,NMI_FRMT2    Skip if this  byte  should

be written more times.

INC  DE                Point to next location.

LD   A,(DE)            Get byte from memory  into

LD   B,A               B; the no. repetitions  of

INC  DE                the next byte.

LD   A,(DE)            Read that byte in A.

18AA NMI_FRMT2 EX   AF,AF'            Byte is now in A'.

RETN                   Return from NMI.

These two addresses are the first to be copied into the IC 6116.

They contain the start of the main table and the  start  of  the

free space after all tables.

18AB COPY_RAM  DEFW +2009,MAIN_TABL   Start of main table.

18AD COPY_RAM2 DEFW +2089,FREE_MEM    Start of free space.

DEFB +03               Which drives are primary?

DEFB +FF               Current track of drives

DEFB +FF

DEFB +FF

DEFB +FF

THE MAIN TABLE

This table holds the startaddresses of the other tables.

18B6 MAIN_TABL DEFW +1936,TABLE_00    Hook-code table.

DEFW +1957,TABLE_02    Command table.

DEFW +1B25,TABLE_04    Channel table 1.

DEFW +1B58,TABLE_06    Channel table 2.

DEFW +1B6E,TABLE_08    Device table.

DEFW +1B91,TABLE_0A    Disk-info table.

DEFW +197E,TABLE_0C    Error message table.

DEFW +1ADD,TABLE_0E    Exit table.

DEFW +1AE4,TABLE_10    General utility table.

DEFW +1AF7,TABLE_12    System utility table.

DEFW +1B12,TABLE_14    File manipulation table.

DEFW +1BA6,TABLE_16    Print character table.

DEFW +1BCA,TABLE_18    Format-data table.

THE DISK-INFO TABLE #0A IN RAM

This table and the subtables following it are copied to  the  IC

6116 if it is present. The pointer in to main table  is  altered

when it is copied. It is not used when the IC is not present.

18D0 TB_0A_RAM DEFB +01               Drive 1.

DEFW +2038,DRIVE_1

DEFB +02               Drive 2.

DEFW +2046,DRIVE_2

DEFB +03               Drive 3.

DEFW +2054,DRIVE_3

DEFB +04               Drive 4.

DEFW +2063,DRIVE_4

DEFB +05               Ramdisk 48K.

DEFW +2070,RAMD_48

DEFB +06               Ramdisk 128K.

DEFW +2077,RAMD_128

DEFB +00               Error.

DEFW +2000

THE SUBTABLES OF TABLE #0A

Byte 2 is used as follows: bit 0: SET for righthand drive;

bit 1: SET for lefthand drive;

bit 2: SET if drive has a virtual;

bit 3: RESET if drive is primary.

bit 4: SET if disk is double sided.

bit 5: SET if drive is double sided.

bits 6 and 7: block-size : 00 = 128

01 = 256

10 = 512

11 = 1024

Bit 4 and 5 of byte 7 hold the  steprate  for  the  arm  of  the

drive: 00 = 6 ms., 01 = 12 ms., 10 = 20 ms., 11 = 30 ms.

The information for drive 1:

18E5 DRIVE_1   DEFB +28               (00) 40 tracks.

DEFB +12               (01) 18 sectors/track.

DEFB +41               (02) %01000001

DEFB +00               (03) no extra sectors.

DEFW +0001             (04) boot-block = -1

DEFB +05               (06) number of retries

DEFB +CE               (07) control register

DEFB +04               (08) Drive 3 as virtual

DEFB +28               (09)

DEFB +00               (0A) format type 0

DEFB +0D               (0B) interleave = 13

DEFB +05               (0C) number of retries

DEFB +06               (0D) 6 + 1 blocks in 'CAT'

The information for drive 2:

18F3 DRIVE_2   DEFB +28               (00) 40 tracks.

DEFB +12               (01) 18 sectors/track.

DEFB +42               (02) %01000010

DEFB +00               (03) no extra sectors.

DEFW +0001             (04) boot-block = -1

DEFB +05               (06) number of retries

DEFB +CE               (07) control register

DEFB +08               (08) Drive 4 as virtual

DEFB +28               (09)

DEFB +00               (0A) format type 0

DEFB +0D               (0B) interleave = 13

DEFB +05               (0C) number of retries

DEFB +06               (0D) 6 + 1 blocks in 'CAT'

The information for drive 3:

1901 DRIVE_3   DEFB +28               (00) 40 tracks.

DEFB +12               (01) 18 sectors/track.

DEFB +45               (02) %01000101

DEFB +00               (03) no extra sectors.

DEFW +0001             (04) boot-block = -1

DEFB +05               (06) number of retries

DEFB +CE               (07) control register

DEFB +04               (08) Virtual of drive 1

DEFB +28               (09)

DEFB +00               (0A) format type 0

DEFB +0D               (0B) interleave = 13

DEFB +05               (0C) number of retries

DEFB +06               (0D) 6 + 1 blocks in 'CAT'

The information for drive 4:

190F DRIVE_4   DEFB +28               (00) 40 tracks.

DEFB +12               (01) 18 sectors/track.

DEFB +4A               (02) %01001010

DEFB +00               (03) no extra sectors.

DEFW +0001             (04) boot-block = -1

DEFB +05               (06) number of retries

DEFB +CE               (07) control register

DEFB +08               (08) Virtual of drive 2

DEFB +28               (09)

DEFB +00               (0A) format type 0

DEFB +0D               (0B) interleave = 13

DEFB +05               (0C) number of retries

DEFB +06               (0D) 6 + 1 blocks in 'CAT'

The information for the 48K ramdisk

191D RAMD_48   DEFW +8000             (00) startaddress = 32768

DEFW +0020             (02) block-size = 32

DEFW +0400             (04) no. blocks = 1024

DEFB +07               (06) 7 + 1 blocks in 'CAT'

The information for the 128K ramdisk

1924 RAMD_128  DEFW +C000             (00) startaddress = 49152

DEFW +0100             (02) block-size = 256

DEFW +0100             (04) no. blocks = 256

DEFB +03               (06) 3 + 1 blocks in 'CAT'

DEFB +02               (07) 16K boundary multipl.

DEFW +7FFD             (08) page port 128K

DEFB +16               (0A) Default 'current' bl.

DEFB +10, +13, +14,    (0B) 128K  block  replace-

DEFB +17, +11, +12,         ment numbers.

DEFB +10

THE HOOK-CODE TABLE

In this table the startaddress for the different hook-codes  are

stored.

1936 TABLE_00  DEFB +E1               Character I/O

DEFW +00A5,CHAR_I/O

DEFB +0B               Spectrum error

DEFW +0100,BASIC_ERR   ('Nonsense in BASIC')

DEFB +0E               Spectrum error

DEFW +0100,BASIC_ERR   ('Invalid file name')

DEFB +17               Spectrum error

DEFW +0100,BASIC_ERR   ('Invalid I/O device')

DEFB +47               Length current channel

DEFW +0093,LENGTH_1

DEFB +FF               Spectrum error

DEFW +01C2,AUTO_RUN    ('OK')

DEFB +FD               Version number ROM

DEFW +0088,VERSION

DEFB +E3               Initialize IC 6116

DEFW +009E,INIT_RAM

DEFB +D4               Create Mdrive syvars.

DEFW +00E2,M_DRIVE

DEFB +2D               Copyright message

DEFW +001E,(C)_MSG

DEFB +00               Other

DEFW +00F7,HOOKERROR

THE COMMAND TABLE

The ASCII-values of the different commands are  stored  in  this

table. The 'CLOSE #'  character  is  not  included,  since  this

command is handled by the +1708 entry point.

1957 TABLE_02  DEFB +F8               SAVE

DEFW +029D,LD_SA_ETC

DEFB +EF               LOAD

DEFW +029D,LD_SA_ETC

DEFB +D6               VERIFY

DEFW +029D,LD_SA_ETC

DEFB +D5               MERGE

DEFW +029D,LD_SA_ETC

DEFB +D1               MOVE

DEFW +0329,MOVE

DEFB +D3               OPEN #

DEFW +0200,OPEN_#

DEFB +A9               POINT

DEFW +03AA,POINT

DEFB +FD               CLEAR

DEFW +03E5,CLEAR

DEFB +CF               CAT

DEFW +04BF,CAT

DEFB +D0               FORMAT

DEFW +04DC,FORMAT

DEFB +D2               ERASE

DEFW +04D8,ERASE

DEFB +FB               CLS

DEFW +04F2,CLS_#

DEFB +00               Other

DEFW +016D,CONTSIG_2

THE ERROR MESSAGE TABLE

This table holds the startaddresses for the  characters  of  the

various error reports.

197E TABLE_0C  DEFB +29               a

DEFW +19D0,REPORT_A

DEFB +2A               b

DEFW +19E3,REPORT_B

DEFB +2B               c

DEFW +19F6,REPORT_C

DEFB +2D               e

DEFW +1A0A,REPORT_E

DEFB +2E               f

DEFW +1A19,REPORT_F

DEFB +2F               g

DEFW +1A28,REPORT_G

DEFB +30               h

DEFW +1A36,REPORT_H

DEFB +31               i

DEFW +1A44,REPORT_I

DEFB +32               j

DEFW +1A53,REPORT_J

DEFB +34               l

DEFW +1A62,REPORT_L

DEFB +35               m

DEFW +1A75,REPORT_M

DEFB +36               n

DEFW +1A84,REPORT_N

DEFB +47               (c)

DEFW +19BD,REP_(C)

DEFB +48               Wally

DEFW +1A8E,REP_WALLY

DEFB +57               RAM

DEFW +1A9E,REP_RAM

DEFB +E0               Message 1

DEFW +1AB3,MESSAGE_1

DEFB +E1               Message 2

DEFW +1ABC,MESSAGE_2

DEFB +E2               Message 3

DEFW +1ABE,MESSAGE_3

DEFB +E3               Message 4

DEFW +1ACA,MESSAGE_4

DEFB +E4               Message 5

DEFW +1ADC,MESSAGE_5

DEFB +00               Other

DEFW +1AA9,UNKNOWN

The error messages and other reports. Each last byte has b7 SET.

19BC REP_(C)   DEFM 1984-86 Dave Corney

19D0 REPORT_A  DEFM Invalid device name

19E3 REPORT_B  DEFM Stream already open

19F6 REPORT_C  DEFM Invalid drive number

1A0A REPORT_E  DEFM Write protected

1A19 REPORT_F  DEFN No room on disk

1A28 REPORT_G  DEFM Disk I/O error

1A36 REPORT_H  DEFM File not found

1A44 REPORT_I  DEFM Hook code error

1A53 REPORT_J  DEFM File size error

1A62 REPORT_L  DEFM Verification failed

1A75 REPORT_M  DEFM Wrong file type

1A84 REPORT_N  DEFM Wrong disk

1A8E REP_WALLY DEFM Don't be a wally

1A9E REP_RAM   DEFM Ram corrupt

1AA9 UNKNOWN   DEFM No message

1AB3 MESSAGE_1 DEFM Destroy "

1ABC MESSAGE_2 DEFM "?

1ABE MESSAGE_3 DEFM Insert disk

1ACA MESSAGE_4 DEFM , then press any key

1ADC MESSAGE_5 DEFM y+CHR$(0)+LPRINT         (Probably an error)

THE EXIT TABLE

This table holds the addresses of the return and EXIT routines.

1ADD TABLE_0E  DEFB +00

DEFW +1AE0,SUB_0E

1AE0 SUB_0E    DEFW +001C             Simply return.

DEFW +0586,EXIT        Use the EXIT routine.

THE GENERAL UTILITY TABLE

This table holds the startaddresses of  the  routines  that  are

accessible from the BASIC-level.

1AE4 TABLE_10  DEFB +00

DEFW +1AE7,SUB_10

1AE7 SUB_10    DEFW +03D8,CALL_CHAN

DEFW +04E9,CALL_UTIL

DEFW +06A9,OPEN_CHAN

DEFW +0368,CLOSE_CHN

DEFW +036D,CLEAR_CHN

DEFW +0486,WIPE_CHAN

DEFW +06BF,MOVE_CHAN

DEFW +06F4,SAVE_CHAN   Both SAVE and LOAD.

THE SYSTEM UTILITY TABLE

This table holds the startaddresses of  the  routines  that  are

used by the interpreter routines.

1AF7 TABLE_12  DEFB +00

DEFW +1AFA,SUB_10

1AFA SUB_12    DEFW +0FE5,CAL_PHY     Call the hardware.

DEFW +1540,T/R_BREAK   Test/report 'BREAK' key.

DEFW +016D,CONTSIG_2   Keep signalling an error.

DEFW +050E,CHK_END     Check for end of statement

DEFW +053A,CHK_CHAN    Check channel parameters.

DEFW +0525,TEST_SEP    Check for a seperator.

DEFW +0532,USE_M1      Pass -1 to the calc. stack

DEFW +0818,MAKE_CHAN   Make channel in memory.

DEFW +0863,TEST_C      Test channel parameters.

DEFW +05FB,CL_T_CHNS   Clear all temporary chans.

DEFW +062F,CD_TO_STK   Copy code to the stack.

DEFW +0656,EXEC_STK    Execute code in the stack.

THE FILE MANIPULATION TABLE

This table holds the startaddresses of  the  routines  that  are

used by the OPEN_CHAN, CLOSE_CHN and SAVE_CHAN routines.

1B12 TABLE_14  DEFB +00

DEFW +1B15,SUB_14

1B15 SUB_14    DEFW +0DD4,MAKE_CAT    Make a 'CAT' channel.

DEFW +0C63,PUT_BLOCK   Put a block on disk.

DEFW +0E3D,FIND_FILE   Search catalog for file.

DEFW +0EA4,MAKE_FILE   Make a file in catalog.

DEFW +0E64,GET_FREE    Get free space on the disk

DEFW +0F48,POSN_BC     Position 'CAT'-file at BC.

DEFW +0F3F,READ_CAT    Read from the 'CAT' chan.

DEFW +0B6E,WRITE_CT    Write to the 'M' channel.

THE CHANNEL TABLE 1

This table holds the startaddresses of the subtables for all the

channels. The pointers allow writing, reading, opening, closing,

length calculation and positioning of channels to be done.

1B25 TABLE_04  DEFB +20               ' '

DEFW +0A75,TAB_4:M

DEFB +6D               'M'

DEFW +0A75,TAB_4:M

DEFB +23               '#'

DEFW +08EA,TAB_4:#

DEFB +62               'B'

DEFW +0954,TAB_4:B

DEFB +74               'T'

DEFW +0948,TAB_4:T

DEFB +6B               'K'

DEFW +089C,TAB_4:KSP

DEFB +73               'S'

DEFW +089C,TAB_4:KSP

DEFB +70               'P'

DEFW +089C,TAB_4:KSP

DEFB +6A               'J'

DEFW +11BC,TAB_4:J

DEFB +64               'D'

DEFW +1032,TAB_4:D

DEFB +EF               'CAT'

DEFW +0DC0,TAB_4:CAT

DEFB +AF               'CODE'

DEFW +0FFF,TAB_4:COD

DEFB +00               Other

DEFW +1B4C,ERROR_04

1B4C ERROR_04  DEFW +08E4,WRITE_KSP   Write.

DEFW +08C6,READ_KSP    Read.

DEFW +0594,REPORT_2A   Open (error-message).

DEFW +1246,REPORT_k    Close (error-message).

DEFW +1246,REPORT_k    Length (error-message).

DEFW +1246,REPORT_k    Position (error-message).

THE CHANNEL TABLE 2

This table holds the startaddresses of the subtables of the ' ',

'M' and 'J' channels. The subtables  allow  erase,  catalog  and

format to be done to the channel.

1B58 TABLE_06  DEFB +20               ' '

DEFW +0C8B,TAB_6:' '

DEFB +6D               'M'

DEFW +0C83,TAB_6:'M'

DEFB +6A               'J'

DEFW +1B66,TAB_6:'J'

DEFB +00               Other

DEFW +1B64,ERROR_06

1B64 ERROR_06  DEFW +0594,REPORT_2A   Error-message.

1B66 TAB_6:'J' DEFW +0594,REPORT_2A   Erase (error-message).

DEFW +0594,REPORT_2A   Inquire (error-message).

DEFW +0594,REPORT_2A   Catalog (error-message).

DEFW +11DE,FORMAT_J    Format.

THE DEVICE TABLE

This table holds the startaddresses  of  the  subtables  of  the

devices. The subtables allow read, write, inquire and format  to

be done to the device.

1B6E TABLE_08  DEFB +01               Drive 1.

DEFW +123A,TAB_8:M

DEFB +02               Drive 2.

DEFW +123A,TAB_8:M

DEFB +03               Drive 3.

DEFW +123A,TAB_8:M

DEFB +04               Drive 4.

DEFW +123A,TAB_8:M

DEFB +82               Joystick port.

DEFW +11EE,TAB_8:J

DEFB +05               Ramdisk 48K.

DEFW +1749,TAB_8:5

DEFB +06               Ramdisk 128K.

DEFW +178E,TAB_8:6

DEFB +81               Printer port.

DEFW +1814,TAB_8:T

DEFB +00               Other.

DEFW +1B89,ERROR_08

1B89 ERROR_08  DEFW +1246,REPORT_k    Write (error-message).

DEFW +1246,REPORT_k    Read (error-message).

DEFW +0DE0,REPORT_c    Inquire (error-message).

DEFW +0DE0,REPORT_c    Format (error-message).

THE DISK-INFO TABLE #0A IN ROM

This table holds the startaddresses of the  subtables  with  the

information for the drives and the ramdisks. It is not used when

the RAM tables are used (they include their own disk-info table)

1B91 TABLE_0A  DEFB +01               Drive 1.

DEFW +18E5,DRIVE_1

DEFB +02               Drive 2.

DEFW +18F3,DRIVE_2

DEFB +03               Drive 3.

DEFW +1901,DRIVE_3

DEFB +04               Drive 4.

DEFW +190F,DRIVE_4

DEFB +05               Ramdisk 48K.

DEFW +191D,RAMD_48

DEFB +06               Ramdisk 128K.

DEFW +1924,RAMD_128

DEFB +00               Other.

DEFW +18AD,COPY_RAM2

THE PRINT CHARACTER TABLE

This table holds the ASCII-values of the characters that can  be

printed when emulating the ZX PRINTER.

1BA6 TABLE_16  DEFB +06               Print ,

DEFW +09B2,PRINT_,

DEFB +08               Cursor left

DEFW +09FD,PR_CRSLEF

DEFB +09               Cursor right

DEFW +09A4,PR_CRSRIG

DEFB +10               Ink

DEFW +09AF,PRINT_2ND

DEFB +11               Paper

DEFW +09AF,PRINT_2ND

DEFB +12               Flash

DEFW +09AF,PRINT_2ND

DEFB +13               Bright

DEFW +09AF,PRINT_2ND

DEFB +14               Inverse

DEFW +09AF,PRINT_2ND

DEFB +15               Over

DEFW +09AF,PRINT_2ND

DEFB +16               At

DEFW +09A9,PRINT_AT

DEFB +17               Tab

DEFW +09AD,PRINT_TAB

DEFB +00               Any other charater

DEFW +09C1,PR_OTHER

THE FORMAT-DATA TABLE

This table holds the  addresse  of  the  data  that  the  format

routine uses, and also the code for the boot-block.

1BCA TABLE_18  DEFB +01               Format type special

DEFW +1C09,TAB_18_1

DEFB +00               Format type normal

DEFW +1BD0,TAB_18_0

1BD0 TAB_18_0  DEFW +1BD8,TAB_18_00   Entry 1

DEFW +1BDB,TAB_18_01   Entry 2

DEFW +1C00,TAB_18_02   Entry 3

DEFW +1C3E,LOST_BLCK   Location boot-block.

1BD8 TAB_18_00 DEFB +02               Length-byte.

DEFB +0B, +4E

1BDB TAB_18_01 DEFB +24               Length-byte.

DEFB +18, +4E

DEFB +0C, +00

DEFB +03, +F5

DEFB +01, +FE

DEFB +01, +F1          (track)

DEFB +01, +F0          (side)

DEFB +01, +F4          (sector)

DEFB +01, +F2          (block-size)

DEFB +01, +F7

DEFB +16, +4E

DEFB +0C, +00

DEFB +03, +F5

DEFB +01, +FB

DEFB +F3, +E5

DEFB +F3, +E5

DEFB +F3, +E5

DEFB +F3, +E5

DEFB +01, +F7

1C00 TAB_18_02 DEFB +08               Length-byte.

DEFB +00, +4E

DEFB +00, +4E

DEFB +00, +4E

DEFB +00, +4E

1C09 TAB_18_1  DEFW +1C11,TAB_18_10   Entry 1.

DEFW +1C14,TAB_18_11   Entry 2.

DEFW +1C35,TAB_18_12   Entry 3.

DEFW +1C3E,LOST_BLCK   Location boot-block.

1C11 TAB_18_10 DEFB +02               Length-byte.

DEFB +06, +FF

1C14 TAB_18_11 DEFB +20               Length-byte.

DEFB +0A, +FF

DEFB +06, +00

DEFB +01, +FE

DEFB +01, +F1          (track)

DEFB +01, +F0          (side)

DEFB +01, +F4          (sector)

DEFB +01, +F2          (block-size)

DEFB +01, +F7

DEFB +0B, +FF

DEFB +06, +00

DEFB +01, +FB

DEFB +F3, +E5

DEFB +F3, +E5

DEFB +F3, +E5

DEFB +F3, +E5

DEFB +01, +F7

1C35 TAB_18_12 DEFB +08               Length-byte.

DEFB +00, +FF

DEFB +00, +FF

DEFB +00, +FF

DEFB +00, +FF

The next block is stored on a  specially  reserved  block.  It's

number can be found by subtracting from 65536 the 4th (lo-)  and

5th (hi-) byte in the disk-info table (table #0A).

1C3E LOST_BLCK JR   1C45,GO_BOOT      Step over the databytes.

DEFB +28               40 tracks.

DEFB +12               18 sectors/track.

DEFB +40               256 bytes a block, SS.

DEFW +0000             Room for unique number.

1C45 GO_BOOT   LD   A,(HL)            Get number of tracks.

LD   (IX+00),A         Store it in the table.

INC  HL

LD   A,(HL)            Get number of sectors.

LD   (IX+01),A

LD   A,(IX+02)         Get disk-info byte.

AND  +2F               This is %00101111

LD   D,A               Store drive-info.

INC  HL

LD   A,(HL)            Get disk-info from disk.

AND  +D0               This is %11010000

OR   D                 Add disk + drive info.

LD   (IX+02),A         Store it in the table.

RET                    Finished.

The addresses +1C5D to +1FFD are not used  in  version  2.2  and

contain +FF's.

1FFE TAB_POINT DEFW +18B6,MAIN_TABL   Pointer to ROM tables.

After this ROM comes the 2K RAM, to which the tables are copied.

APPENDIX A - AN INDEX TO ROUTINES

Address Routine                                             Page

0000  Reset RESTART                                          1

0008  Shadow_ROM error RESTART                               1

000B  Entry_1                                                1

0010  Call a spectrum ROM routine RESTART                    1

0018  Running or checking syntax? RESTART                    1

001E  Print the copyright message                            2

0020  Next character RESTART                                 2

0028  Floating point calculator RESTART                      2

0030  Lookup RESTART                                         2

0038  Maskable interrupt RESTART                             2

0048  Key_int                                                3

0065  Non-maskable interrupt                                 3

0067  Call a spectrum ROM routine (2)                        3

007A  Entry_2                                                4

0088  Version HOOKCODE                                       4

0093  Length HOOKCODE                                        4

009E  Initialise RAM HOOKCODE                                5

00A5  Character I/O HOOKCODE                                 5

00C0  Input extra characters                                 5

00E2  Mdrive HOOKCODE                                        6

00F7  Hookcode error                                         6

0100  BASIC error HOOKCODE                                   6

0168  Continue signalling of an error                        7

016D  Contsig_2                                              8

01C2  Auto_run HOOKCODE                                      9

0200  OPEN # COMMAND                                         9

029D  LOAD, SAVE, VERIFY and MERGE COMMANDS                 12

0329  MOVE COMMAND                                          14

0368  Close a channel                                       14

036D  Clear a channel                                       15

03AA  POINT COMMAND                                         16

03DB  Call a channel                                        16

03E5  CLEAR COMMAND                                         17

0462  Wipe channels from memory                             19

04A7  Get line startaddress                                 19

04B6  Check line address                                    20

04BF  CAT COMMAND                                           20

04D8  ERASE COMMAND                                         20

04DC  FORMAT COMMAND                                        21

04DE  Prepare utility                                       21

04E9  Call utility                                          21

04F2  CLS # COMMAND                                         21

050E  Check end of statement                                22

0525  Test for a seperator                                  22

0532  Use minus one                                         22

Address Routine                                             Page

053A  Check the channel                                     22

0586  Exit                                                  24

05B1  Lookup (2)                                            25

05EB  Print an error-message                                26

05EF  Print a message at HL                                 26

05FB  Clear all temporary channels                          26

0623  Neat return                                           27

062F  Copy code to the stack                                27

0648  Floating point calculator (2)                         28

0656  Execute code in the stack                             28

0679  Make room in the stack                                29

0698  Free the room in the stack                            29

06A9  Open a channel                                        30

06BF  Move a channel                                        30

06F4  SAVE or LOAD a channel                                31

07EE  Find the I/O address                                  36

07FF  Test if end of file pressed                           36

080C  Get the offset of a channel                           36

0818  Make a channel in memory                              37

0863  Test the channel parameters                           38

08A8  Open the 'K', 'S' or 'P' channel                      39

08C6  Read from the 'K', 'S' or 'P' channel                 40

08CC  Position the 'K', 'S' or 'P' channel                  40

08E4  Write to the 'K', 'S' or 'P' channel                  40

08F6  Open the '#' channel                                  41

0905  Write to the '#' channel                              41

0911  Read from the '#' channel                             41

091C  Length of the '#' channel                             42

0926  Position the '#' channel                              42

0930  Check for an infinite loop                            42

0960  Open the 'B' channel                                  43

096B  Open the 'T' channel                                  43

0986  Close the 'T' channel                                 44

0988  Write to the 'T' channel                              44

09A4  PRINT_OUT                                             45

09C1  Print other characters                                45

0A63  Write to the 'B' channel                              47

0A6B  Read from the 'T' channel                             47

0A71  Read from the 'B' channel                             48

0A81  Open the 'M' channel                                  48

0B6E  Write to the 'M' channel                              52

0B7C  Read from the 'M' channel                             53

0B81  Calculate the length of the 'M' channel               53

0BB8  Position the 'M' channel                              54

0BF8  Close the 'M' channel                                 55

0C63  Put a block on disk                                   56

0C93  Catalog the 'M' channel                               56

0CF7  Erase from the 'M' channel                            58

Address Routine                                             Page

0CFE  Erase from the ' ' channel                            58

0D12  Format the 'M' channel                                58

0D6B  Check for a formatted disk                            59

0DCC  Open the 'CAT' channel                                61

0E2E  Store 12 FF's after HL                                62

0E38  Search the catalog from the start                     63

0E3D  Find the file-record                                  63

0E64  Get free space on the disk                            63

0EA4  Make a file in the catalog                            64

0EE3  Read a file-record from the catalog                   65

0EF6  Write the file-record to the catalog                  66

0F07  Make or free space in the catalog                     66

0F3F  Read from the 'CAT' channel                           67

0F45  Position the 'CAT'-file at the start                  67

0F48  Position the 'CAT'-file at BC                         68

0F51  Stack the block-size                                  68

0F5B  End of file test                                      68

0FA1  HL := IX + DE + 33                                    69

0FAA  Read next block from disk                             69

0FE5  Call a physical device                                70

100B  Open the 'CODE' channel                               71

1014  Read from the 'CODE' channel                          71

101E  Write to the 'CODE' channel                           71

102D  Position the 'CODE' channel                           72

103E  Open the 'D' channel                                  72

104E  Write to the 'D' channel                              72

1060  Read from the 'D' channel                             73

106E  Length of the 'D' channel                             73

1076  Close the 'D' channel                                 73

1176  At-end test and load the next block                   77

11AB  Open the 'CAT'-file on the current 'M' channel        78

11C8  Open the 'J' channel                                  79

11D4  Read from the 'J' channel                             79

11DE  Format the 'J' channel                                79

11F6  Read from the 'J' channel                             79

122F  Format the joystick port                              80

1242  Write to disk                                         81

1248  Read from disk                                        81

132C  Format the 'M' channel                                84

1414  Inquire the disk                                      87

1478  Setup the data for formatting                         88

14B9  Shift format-data left                                89

14D3  Restore the disk-controller                           90

14D5  Call the disk-controller                              90

14F1  HL := start of disk tables                            90

14FE  Find the address for the NMI                          90

1540  Test for 'BREAK' and report if pressed                91

1548  Test for 'BREAK' pressed                              92

Address Routine                                             Page

1554  Initialise the IC 6116                                92

1708  PAGE IN                                               93

1748  PAGE OUT                                              94

1751  Format and inquire the ramdisk                        94

1763  Write to the 48K ramdisk                              95

1769  Read from the 48K ramdisk                             95

1770  Find the address in the 48K ramdisk                   95

1796  Write to the 128K ramdisk                             96

179F  Read from the 128K ramdisk                            96

17D3  Find the address in the 128K ramdisk                  97

181C  Write to the printerport                              98

1853  Read from the printerport                             98

1886  NMI read a byte                                       99

1895  NMI write a byte                                      99

189F  NMI format a byte                                    100

18B6  The main table                                       100

18D0  The disk-info table #0A in RAM                       101

18E5  The subtables of table #0A                           101

1936  The hook-code table                                  103

1957  The command table                                    103

197E  The error-message table                              104

1ADD  The exit table                                       105

1AE4  The general utility table                            106

1AF7  The system utility table                             106

1B12  The file manipulation table                          106

1B25  The channel table 1                                  107

1B58  The channel table 2                                  107

1B6E  The device table                                     108

1B91  The disk-info table #0A in ROM                       108

1BA6  The print character table                            109

1BCA  The format-data table                                110

APPENDIX B - LOCATION OF CHANNELINFO

Channel  Type                                        Length Page

#        linked channel                                   8   41

T        printer text channel                            10   43

B        printer binary channel                           7   43

M, CAT   microdrive/disk & catalog channel     33+blocksize   48

CODE     memory channel                                   9   71

D        disk move channel                                9   72

J        joystick channel                                 8   78

APPENDIX C - LOCATION OF CHANNEL SUBTABLES

Channel    Table Address Page

K, S, P      4    089C     39

#            4    08EA     41

T            4    0948     42

B            4    0954     43

M            4    0A75     48

M            6    0C83     56

' '          6    0C8B     56

CAT          4    0DC0     60

CODE         4    0FFF     71

D            4    1032     72

J            4    11BC     78

J            8    11EE     79

M            8    123A     81

ramdisk 48K  8    1749     94

ramdisk 128K 8    178E     95

T            8    1814     98