.po 3

Object File Structure

#1.0 Introductions#

Object files are produced by assembler,  AS, and may be manipulated

by the librarian, LIB. Object files are formed of five distinct blocks:

o    Directory

o    Code

o    Static Data

o    Import Table

o    Export Table

Each  block starts at a 512 byte boundary within the object file and may

occupy zero or more pages. The first block is always the directory block

and  the  rest of the blocks are arbitrarily allocated  by  either  that

assembler or librarian.

#1.1 Directory#

Each directory block contains information for up to ten modules. If

more  than  ten  modules are contained in the ojbect file  then  further

contiguous blocks are allocated.  The directory block is formed of three

distinct sections:  a header section, an array of module information and

a trailer (padding to 512 bytes).

#NAME#   #BITS#           #CONTENTS#

#Header#

NEXTDIR     16    Pointer to next directory block, -1 if last block

MODCOUNT    16    Number of modules in this block

#Array of module information (maximum of ten entries per block)#

MODNM       64    Module name, left justified, space padded

MODTYPE      3    Language (0=pascal, 1=assembler, 2=C, 3=fortran)

STATUS       1    Module status (0=main, 1=not main)

UNUSED      12    Reserved

STARTADDR   32    Execution start address (offset from code start)

CODELEN     32    Code size in bytes

SBSIZE      32    Static Base data size in bytes

SBBLK       16    Static base block number

SYMLEN      16    Symbol table length (currently reserved)

SBADDR      32    Static Base address (-1 if no SB data)

CODEADDR    32    Code address (-1 if no code)

EXPOBLK     16    Start block of export symbols

IMPOBLK     16    Start block of import symbols

CODEBLK     16    Start block of code

SYMBLK      16    Start block of symbol table (currently reserved)

EXPOLEN     16    Number of export symbols

IMPOLEN     16    Number of import symbols

#NAME#   #BITS#           #CONTENTS#

FILLER     304    Reserved

VERSION     16    Version number

CHECK       64    Character string that contains 'CODEFIL '

#1.2 Code#

The code block contains the actual program code to be executed. The

code blocks are contiguous.

#1.3 Static Base data#

The static base data contains the data that is referenced by the SB

register  of  the  32000  series family.  The  static  base  blocks  are

contiquous.

#1.4 Import Table#

#NAME#   #BITS#           #CONTENTS#

INAME       72    The import symbol name

ISIZE       32    Size of data (used when symbol is common)

ITYPE        1    Symbol type (0=data, 1=entry point)

ICOMM        1    Common flag (0=not common, 1=common)

ISTAT        1    Static flag (0=global symbol, 1=local symbol)

UNUSED      13    Reserved

#1.5 Export Table#

#NAME#   #BITS#           #CONTENTS#

ENAME       72    The export symbol name

ESIZE       32    Byte offset within segment

ESTAT        1    Static flag (0=global symbol, 1=local symbol)

UNUSED       2    Reserved

ETYPE        1    Type (0=SB data, 1=ABS data, 2=PC, 3=PC data 4=PC

entry

UNUSED      10    Reserved

.pa

Executable File Structure

#2.0 Introduction#

Executable  files  are produced by linker,  An executable  file  is

typically made up of serveral object files. They have and extension .E32

and  are  loaded  into the DSI-32 board  by  the  Definicon  Loader.  An

executable file consists of five distinct blocks.

o    General Information

o    Module directory

o    Code

o    Static data

o    Link tables

Each  block starts at a 512 byte boundary within the executable file and

may  occupy  zero or more pages.  The first block is always the  general

information  block and the rest of the blocks are arbitrarily  allocated

by the linker.

#2.1 General Information#

The  general information block contains information that is  common

to  all modules.  Although also contained in this  section,  the  Global

Module Table, is not physically part of the general information section.

It  contains a copy of the module data that enables the separate modules

in the executable file to communicate and pass data between themselves.

#NAME#   #BITS#           #CONTENTS#

EXECID      16    7699 plus Version number

DIRBLK      16    Pointer to first directory block

HEAP_LOW    32    Heap low address

HEAP_HIGH   32    Heap high address

STACK_LOW   32    Stack low address

STACH_HIGH  32    Stack high address

MAIN        32    Main module number

MODCOUNT    32    Number of modules in the module table

MODADDR     32    Module table load address

#Global Module Table (MODCOUNT of these items)#

SBAD        32    Static Base data

LINKAD      32    Link table address

CODEAD      32    Code address

UNUSED      32    Reserved

.pa

#2.2 Module Directory#

The Module Directories are contiquous arrays of data,  one for each

General  Module  Table  entry  (ie  MODCOUNT  of  them).   Eight  Module

Directories are held in each 512 byte block.

#NAME#   #BITS#           #CONTENTS#

MODNM       64    Module name (left justified, space padded)

MODTYPE      3    Reserved

UNUSED      13    Reserved

STRTADDR    32    Execution start address (offset from code start)

SLEN        32    Length of static base data in bytes

LLEN        32    Length of link table in bytes

CLEN        32    Length of code in bytes

SYMLEN      32    Reserved

SADDR       32    Static base data start address

LADDR       32    Link table start address

CADDR       32    Code start address

LBLK        16    Link table start block number

CBLK        16    Code start block number

SYMBLK      16    Reserved

SBLK        16    Static base start block number

UNUSED     112    Reserved

#2.3 Code#

The code blocks contain the actual code that belongs to the module.

#2.4 Static Base#

The  static base blocks contain the data that is referenced by  the

series 32000 microprocessor's SB register.

#2.5 Link Table#

The  link  table  blocks  store the linkage  information  for  each

module.  The link table information allows the external addressing  mode

of  the  series 32000 microprocessors to determine either  the  absolute

address  of  a  data  item or the module and offset of  a  procedure  in

another module.

##################