TERMINAL ACCESS PACKAGE (TAP)

USER'S GUIDE

PDP-11 VERSION

REVISION B

INTRODUCTION

The Terminal Access Package (TAP allows the Fortran programmer full

exploitation of the AED 512/767's capabilities.  For every instruction in

the 512/767's command set there is a corresponding Fortran callable sub-

routine to issue the instruction.  Operands, or arguments, are encoded by

TAP into the appropriate format.

LEGEND

1.  All arguments not ending in S are 16-bit scalars.

2.  Arguments ending is S are 8-bit one dimensional arrays (type BYTE or

LOGICAL*1).  It is apparently all right in PDP-11/LSI-11 Fortran to

use a text string in this context.  It is also all right to use a scalar

instead of an array name where only the first element will be required.

For example:

CALL SCT (5,1,RED,GREEN,BLUE),

where RED, GREEN, and BLUE are scalars, can be used to set up a single

color table location.

3.  Arguments not preceded by a slash are used by, but not modified by the

subprogram.

4.  Arguments preceded by a slash (/) are returned arguments.

5.  Routines are invoked with the CALL statement, except those listed in

the form

n = FUNC (arg1,lll,argN),

which are integer functions.

6.  CAP is abbreviation for Current Access Position.

SERIAL INTERFACE

OBYTE and IBYTE are routiens which should respectively send and

receive a character from or to the terminal over the desired interface

channel (See TAP Applications Note).  As supplied, the encoding package,

IOBYTE assumes that these characters are a full eight bits.  However, if

due to operating system deficiencies, it is not possible to communicate

with eight bit characters, the routines in I0BYTE should be modified to use

one of the other enoding schemes available with the AED 512/767 7-bit binary,

hexadecimal, modified hexadecimal, or decimal.  Modified hexadecimal is a

good choice, since is tis simple and fast.  In terms of operating speed,

it is equivalent to run modified hex at 9600 baud to standard 8-bit at

4800 baud.

PARALLEL INTERFACE FUNCTIONS

The following subprograms are to be used with a 16-bit bidirectional

data bus prot, except for IBYTE and OBYTE which can be serial.  The user

is cautioned to assure that no information is sent over the parallel

interface while there is still serial data to be processed by the terminal.

DMA FUNCTIONS:  RRD, RDA, WRD, WDA and SCD should specify the transfer

lenght in words.  For more detail, refer to PARALLEL COMMUNICATION and

TERMINAL COMMAND PROTOCOL Sections of the manual.

IBYTE (/bytes,n)  -  Read (n) characters into array (bytes).  Normally

used internally by TAP, but can be invoked by the user for special

purposes.

OBYTE (bytes,n)  -  Output (n) characters from array (bytes).  Normally

used internally by TAP, but can be invoked by the user for special

purposes.

SELECT (byte)  -  Select terminal corresponding to bits #0-7 of (byte).

SKS (/bytes,n)  -  Send (n) keystroke characters into array (bytes).

RESET -  Full reset of selected terminals, provided option dip switch

#5 is set.  This routine should be followed by a short delay (del150).

RRD (/data,n)  -  Read raster direct.  Perform terminal to host DVMA

transfer of (n) words into array (data).

RDA (data,n)  -  Read direct from area of interest specified by (DAI)

function.  Perform terminal to host DVMA transfer of (n) words into

array (data).

WRD (data,n)  -  Write raster direct.  Perform host to terminal DVMA

transfer of (n) words from array (data).

WDA (data,n)  -  Write direct into area of interst specified by (DAI)

function.  Perform host to terminal DVMA transfer of (n) words from

array (data).

SCD (data,n)  -  Start command DMA.  Terminal receives commands from the

DMA channel.  command array (data) should be (n) words long.  Termi-

nating byte should = 60.

TERMINAL COMMAND FUNCTIONS

The following subprograms are equivalent to the AED 512/767 terminal

instructions which carry the same name.  For more detail regarding their

action, the user is referred to the especially "Terminal Control Protocol"

section of the manual.

AAV (code)  -  Anti-alias vectors.  Code = disable, Code=1 enables anti-

aliasing.  Code = 2 enables and sets anti-aliasing to ramp with each

of the 8 default colors.

AED -  Prints the general status of the AED terminal on the screen. Status

information includes: version #, CAP, current color , etc...

BFL (color)  -  Boundary fill.  Begins at CAP, fills until reaches pixels

with value (color).

BSO (x,y)  -  Set both screen origins, horizontal to (x), vertical to (y).

CAI (x,y)  -  Copy pixels within the area of interest to an equal size area

of which the lower left corner is at x,y.

CHR (n) -  Set character size. n=1,2,3,4 for 5x6,7x9,5x6(double),7x9(double)

DAI(x1,y1,x2,y2)  -  Define Area of Interest with corners at (x1,y1)

and (x2,y2).

DCA (x,y)  -  Draw Cursor Absolute, centered on pixel (x,y).

DCL (radius)  -  Draw "Fat" Circle, centered at CAP.

DFP (Flag color, n, coords)  -  Draw filled polygon of n points defined

by coords.  Flag color is a color which the user will not use in

the background prior to drawing the polygon.  The color of the

polygon is determined by the previous SEC command.

DFR (x,y)  -  Draw Filled Rectangle, with one corner including CAP, opposite

corner including (x,y), which becomes new CAP.

DJC  -  Disable Joystick/Cursor.

DMV (dxs,dys)  -  Draw multiple Vectors, beginning at CAP.  Will draw

vectors until dxs(i)=dys(i)=0.  Each dxs(i) and sys(i) is a signed

differential in the range (-128,127).

DPA  -  Disable joystick pan.

DPK (keyno,icodes)  -  Define Programmable Key.  (keyno) specifies one of

key 0 through key 7.  (icodes) gives the sequence to send each time

the key is hit.  For example:

CALL DPK (3, 'EXIT')

will send the sequence "EXIT" each time key number 3 is hit.

DRL (display)  -  Loads the LED array with the binary encoding of (display).

DSF (code,xsize,ysize,nplane,plane masks,bits)  -  Define user symbol

("Special Font").  (code) is unique for this symbol, must be between

0 and 127, inclusive.  (xsize) and (ysize) give the character matrix

dimensions in pixels.  (nplane) gives the number of planes in which

the character is defined.  (planemasks) is a list of these planes,

each specified by a one in the corresponding bit position.  (bits)

contains the binary matrices to be scanned left to right, top to

bottom, plane to plane.  consult AED 512/767 literature for the

exact packing technique.

DSK (keyno,sizes,icodes)  -  Define software key.  (keyno) specifies one

of key 0 through key 7.  (icodes) gives the sequence to send each time

the key is hit, with (sizes) being the length (max = 15).

DSP (pat#,pattern)  -  Define stipple fill.

DTM (origin,yorigin,xscale,yscale)  -  Define Tablet Mapping.  (xorigin,

yorigin) is the tablet coordinate of the lower left-hand corner

rectangular area to be mapped onto the display screen.  (xscale)

and (yscale) are inverse scale factors which determine the rectangle's

size according to the following relationship:

xscale=32768/xsize

yscale=32769/ysize

where (xsize) and (ysize) give the size of the rectangle in tablet

units.

DVA (x,y)  -  Draw Vector Absolute, from CAP to (x,y),which becomes new

CAP.

DVR (dx,dy)  -  Draw Vector Relative, from CAP to (CAPx+dx,CAPy+dy), which

becomes new CAP.

ECU  -  Erase Cursor Unconditionally.

EJC  -  Enable Joystick/Cursor.

ELP (xsize,ysize)  -  Draw Elipse at CAP with radii defined by xsize, ysize.

EPA  -  Enable Joystick Pan

ERS  -  Erase the entire memory, write-enabled plans only, to the background

color, and remain in interpreter.

ESC  -  Escape.  Enter interpreter.  Does not normally have to be issued

when using TAP.  however if the programmer uses his own routine to

write text, he must re-enter the interpreter with a call ESC.

ESF (xsize,ysize,dx,dy)  -  Erase user symbol(s).  Actually erases an area

of size xsize * ysize with lower left-hand corner at CAP.  Updates

CAP to (CAPx+dx,CAPy+dy).  (dx) and (dy) must be in the range (-128,

127).  This function will erase all planes, so the user must mask out

the planes to be saved with the SWM function, below.

ETC (/key,/flag,/code,/x,/y)  -  Enable Tablet Cursor.  The cursor light

will come on and the operator is free tomove the cursor about.  When

you depress the pen, or hits a cursor button, TAP will return coordi-

nates if (flag) is 1.  Tablet coordinates will be returned if the pen

or cursor is outside of rectangle defined with the DTM subroutine when

the hit is made.

(code) Gives the code for the combination of cursor buttons depressed.

(key) Gives the code of the last of any keystrokes that occurred before

the tablet hit.  If no key was struck 0 will be returned in (code).

Note:  This subroutine differs considerably from the TCP definition of

ETC.

ETP (Idchar)  -  Enable tablet polling if Idchar does not equal zero.  Data

will be sent to host only when issue an RTP.

FFD  -  Erase the entire memory, write-enabled planes only, to the background

color, and exit interpreter.

FRR  -  Fill Rectangle Relative from CAP to CAP+DX,DY which becomes new CAP.

GFL (mask,color)  -  Generalized overlay fill.  Fill in current color, through

current write mask until reaching pixels of value (color) after ending

with (mask).

HOM  -  Return the alpha cursor to the first character of the top line,

according to the current font size.

HSR (dx)  -  Horizontal Scroll Relative.  Modify the current horizontal

origin by addition of (dx), which should be in the range (-128,127).

IFL  -  Interior Fill, beginning at CAP until a pixel with color other

than pixel at CAP, with the current color.

JUS (address)  -  Jump to User Subroutine at location (address).

LAT (Base,values)  -  Load Anti-alias color Table.  Sets anti-alias

table with ramping at color base which is defined by values.

LMR (address,n,bytes)  -  Load Microprocessor RAM, with (n) (bytes)

starting with (address).

MAR (left,right) - set the text margins to left and right.

MOV (x,y)  -  Move CAP to (x,y).

MVR (dx,dy)  -  Move CAP to (CAPx+dx,CAPy+dy).  (dx) and (dy) must be in

the range (-128-127).

OFL  -  Overlay fill, beginning at CAP until reaching a pixel which

logically intersect the current color, filling with the current

color while the Write Mask is temporarily set to the current color.

OPT (optnum,value)  -  Set option number (optnum) to (value).

RCP (/x,/y)  -  Return CAP in (x,y).  If cursor is enabled this will be

the pixel which the curosr is centered upon.

RCT (first,n,reds,greens,blues)  -  Read Color Table.  (n) triples beginn-

ing at (first), both of which should be in the range (0,255).

n=RHO (/xorigin)  -  Return Horizontal Origin as function value and in

(xorigin).

n=RHR (n,/bytes)  -  Read Horizontal Runs.  Return number of runs as

function value.  The runs are sequenced in (bytes) identically to the

output format (ref. WHR).

n=RHS (n,/pixels)  -  Read Horizontal Scan.  N must be the number of

pixels in the area of interest.  (pixels) should be at least

(n) bytes long; it will be filled with the pixels in the area of

interest ordered according to the AOI parameters.

RJP (/x,/y)  -  Return Joystick Position.  (x) and (y) will be in the

range (0,511).

ROT (n) -  Rotate memory planes n times (towards bottom) within the

area of interest.

n=RPX (/value)  -  Return Pixel as function value and in (value) of the

pixel at the CAP.

RST  -  Reset.  Perform a full reset of the AED terminal equivalent to

the Power-Reset.

RTP (Idchar,stat,x,y)  -  Read Tablet Polling.  Idchar, pen status and

x, y coordinates are sent to the host if ETP is enabled.

n=RVO (/yorigin)  -  Return Vertical Origin as function value and in

(yorigin), which will be in the range (0,511).

RZR (x,y) - Read zoom registers (x,y). See SZR command.

SAC (color)  -  Set alpha cursor color.

SAP (size,font,xspace,yspace,link)  -  Set Alphanumeric Parameters as

follows:

size = '1'   Normal Sized            font = '5'    5 x 8 font

size = '2'   Double Sized            font = '7'    7 x 12 font

size = '.'   Same Size as before     font = '.'    Same font as before

hspace - horizontal distace between character centers

yspace - vertical distance between character centers

link = 'L'  Alpha cursor position identical with CAP (default)

link = 'U'  Alpha cursor position not identical with CAP

SAR (xysigns,xrates,yrates)  -  Set auto roam.  Horizontal rate - xrates

*60/32 pixels per second; analogous for vertical.  Bits 7 and 6 of

xysigns determine respectively the sign of horizontal and vertical

roam.

SBC (color)  -  Set Background Color.

SBL (value,red,green,blue,ontime,offtime)  -  Set Blink.  Blink pixels

of logical color (value) between teir current color definition and

(red,green,blue).  Current definition will be in effect for (ontime)

fields; alterante definition for (offtime)  number of fields.

(offtime) = 0 will disable blink.  Up to eight colors may be blink-

enabled simultaneously.

SBR (main,aus)  -  Set Baud Rates.  Override baud rates selected by back

panel DIP switch for main and aux. serial interfaces.  Set rates as

follows:

'0'    '1'    '2'    '3'    '4'    '5'    '6'     '7'        '8'

300    600    1200   2400   4800   9600   19200   External   Leave

Baud   Baud   Baud   Baud   Baud   Baud   Baud    /16        Current

Rate

Unchanged

Note:  These argumetns are 1-character ASCII in all cases.  Also, a

a programming error can change the main baud rate, garbling

further serial communication until a manual reset.

SCC (color1,color2,bltime)  -  Set Cursor Color(s).  The cursor will blink

between (color1) and (color2). (bltime) specifies number of consecutive

fields between color changes (1 field=approx. 1/60 sec).  If (color1) =

(color2) cursor will not appear to blink.

SCP (ctype,hsize,vsize)  -  Set Cursor Parameters.  (ctype) select cursor

type as follows:

ctype='X'         Crosshair, x-shaped, 13 units large

ctype='+'         Crosshair, +-shaped, 13 units large

ctype='L'         Full screen-width crosshairs

if crosshair cursor style is selected (hsize) does not give cursor

size, but is complemented to give a display memory address bit mask.

If full screen crosshair is selected (vsize) specifies the plane in

which to write the curosr; (visze) is in binary decoded format.

SCR  -  Send Carriage Return.  Included in TAP for sake of completeness

only.

SCS (value)  -  Set Console Status, to (value).  See TCP for details.

SCT (first,n,reds,greens,blues)  -  Set up Color Table.  (n) triples

beginning at (first), both of which should be in the range (0,255).

SEC (color)  -  Set Current Color.

SHO (xorigin)  -  Set Horizontal Origin to (xorigin), which should be in

the range (0,767).

SIF ('P' or 'S')  -  Select Interface, parallel 'P' or serial 'S' for data

sent by the terminal.

SLS (pattern,scale)  -  Set Line Style.  The binary encoding of (pattern)

specifies a basic broken line pattern, or template, with 1's correspond-

ing to visible portions and 0's to invisible portions.  The scan of

pattern is from most significant bit to least.  A value of 255 will

accordingly revert the terminal to the default solid line vectors.

(scale) allows the basic pattern to be scaled up as follows:

255         times 1

85         times 2

17         times 4

1         times 8

SPF (pat#)  -  Set Patterned Fill.  Enable stipple fill of pat# defined

by DSP.

SRM (mask0,mask1,mask2,mask3)  -  Set Read Masks.  See TCP for details.

SSE (addr)  -  Set Stack End address to (addr).  See Application Note

'User Firmware' for details

STD (delay)  -  Set turnaround delay for serial communication.

STP (code)  -  Set tablet parameters.  Data is sent to host when pen is

depressed is code = 0.  If code = 1, date is sent when pen is depressed

or relased.  ETC msut be enabled.

STW (code)  -  Set Tektronix Window to 9-bit if code - 0 and set window

to 10-bit if code = 1.

SUC (byte)  -  Set Up counters for DVMA.  Bit assignments in the encoding

of (byte): (1 = true, 0 = false).

0:  X autoincrement (default=1)

1:  Y autonic.  (default = 0)

2:  X up counter (left to right) (default = 1)

3:  Y up counter (bottom to top) (default = 0)

4:  Unlinked counters.  Overflow from one counter will not

increment the other.  (default = 0)

5:  16-bit transfers (as opposed to 8-bit) (default=1)

6:  0 (reserved)

7:  0 (reserved)

Only bit 5 is used in Area of interest transfers.

SUP (mode)  -  Set Superroam.  (mode) = 1 enables Superroam.  (mode = 0)

disables superroam.  See TCP for details.

SVO (yorigin)  -  Set Vertical Origin to (yorigin), which should be in the

range (0,579).

SWM (mask)  -  Set Write Mask.

SZR (xzoom,yzoom)  -  Set-up Zoom Registers.  Zoom factors (xzoom) and

(yzoom) should be in the range (1,16).

VSR (dy)  -  Vertical Scroll Relative.  Modify the current horizontal

origin by additon of (dy), which should be in the range (-128,127).

WHC (npixels)  -  Write horiozntal scan, non-AOI.  (pixels) should be at

least (n) bytes long.

WHR (bytes)  -  Write horizontal runs, beginning at CAP.  Write runs until

bytes (I) = 0.  See TCP for encoding of the byte stream (bytes).

WHS (n,pixels)  -  Write Horizontal Scan of the entire AOI.  (n) should be

equal to the number of pizels in the AOI.

WHU (n,bytes)  -  Write horizotnal runs alternate.  (n) should be equal to

the number of runs (bytes).  Last run should be 0.

WIP (n,bytes)  -  Write Incremental Plotter mode.  (n) gives the total

number of direction codes.  (bytes) holds the codes, packed two per

byte.  See TCP for more information.

WMP (dxs,dys)  -  Srite multiple pixels, beginning at CAP.  Write pixels

until dxs(I) = dys (I) = 0.

WPX (color)  -  Write pixel.

WSF (nchar,codes,dxs,dys)  -  Write user symbols ('Special Font Characters').

Will write (nchar) symbols with (codes) defined by DSF, above.  First

character is written based at CAP, which is updated by addition of

(dxs(i)) for each charcter written.  The numbers in (dxs) and (dys)

should be in the range (-128,127).

SPECIAL PURPOSE FUNCTIONS

TAPINT (option1, option2)  -  Initialize TAP.  Must be called before any

other call to TAP.  (option1) and (option2) have no current usage;

they should have the value 0.

TEXT (screen text')

TEXT (message)  -  Draw a text string beginning at CAP.  (message) is an

array of type byte.  CAP control, first character in message:

Space  -  Advance one line.

(0)  Zero  -  Advance two lines.

(1)  One   -  Top of Screen (home).

(+)  Plus  -  Type on same line.

Special Character:   ($)  Dollar Sign - anywhere in message will cause

advancement of one line.

A null byte serces to terminate the byte string.  Although the terminal

enters alphanumeric mode while generating the string, both entry into

and exit from TXT 512 are with the interpreter active.

ALF  -  Enter Alphanumeric mode.  Should the user need to leave the inter-

preter he should use this subroutine.  Then before issuing any other

call to TAP, he should issue a CALL ESC, to reactivate the interpreter.

NUMBER (number,fieldwidth)  -  Interpret (number) as a 16-bit signed

integer and display beginning at CAP.  If (fieldwidth) is 0, number

will be left-adjusted.  Otherwise (fieldwidth) gives the width of a

field in which the number is right-adjusted.  If insufficient to

express the number, asterisks (*) will be displayed instead.

5.10  TAP APPLICATIONS NOTE - OBYTE AND IBYTE

Tap has been designed so that it is not particularly sensitive to the

specific operating system and hardware configuration employed.  This

was achieved by isolating all input and output to two routines, IBYTE

and OBYTE.  These two routines are not properly part of TAP but are the

drivers that TAP requires.  They may take several different forms depend-

ing upon the interface used and the operating system which might come

between the interface and TAP.  Several examples are given below.

TAP, RT-11, AND AED DMA Interfaces

A driver, currently named DMA5, provides the required OBYTE and IBYTE,

as well as DMA input and output.  This is the simplest of all configura-

tions, and is recommended for its spleed and simplicity over systems

using a serial interface for all communications.

TAP, RSX-11, and AED DMA Interfaces

A driver is available from the AED User's Group which provides OBYTE and

IBYTE as well as DMA input and output.  Otherwise, an identical situation

to the RT-11 case presented above.

TAP, RT-11, with the 512 as the Systems Console

A simple approach is to implement IBYTE and OBYTE with the "System" sub-

routines ITTINR and ITTOUR

C  IBYTE.FOR

C

INTEGER FUNCTION IBYTE(I)

C

100         ICHAR=ITTINR()

IF (ICHAR .LT. 0) GO TO 100

C

I=1CHAR

IBYTE=1CHAR

C

RETURN

END

C  OBYTE.FOR

C

SUBROUTINE OBYTE (ICHAR)

C

100         IF (ITTOUR(ICHAR).NE. 0) GO TO 100

C

RETURN

END

However, there are several problems with this approach.  The chief one is

that RT-11 will truncate characters to seven bits on both input and output.

The solution is to use alternate coding schemes to the standard 8-bit binary.

In this case the routines in I)512 must be modified.  Fortunately this is

straight-forward.  An example is given at the end of this document.

Another problem is that of the overhead introduced by RT-11.  On a PDP11-03,

the maximum rate using this technique is about 400 characters per second,

regardless of baud rate.  However, it is rumored that RT-11 version 4 allows

both 8-bit character transmission and higher throughput.

An approach which also takes care of both these problems (7-bit and low speed)

is to use the following assembly language OBYTE:

.TITLE OBYTE

.GLOBL OBYTE

;

XCSR=177564

XBUF=177566

;

OBYTE:      MOV (R5)+,R0

;

WAIT:       MOV @#XCSR,R0      ;GET STATUS

BIT #200,R0        ;WAIT FOR XMIT READY

BEQ WAIT

;

MOVB @ (R5),@#XBUF ;SEND CHARACTER

;

RTS PC

;

.END

Provided that the actual interface is strapped for eight data bits, this

will work fine and allow output at up to 1920 characters per second (at

19,200 baud).  There will be difficulties, though, when the terminal

becomes busy, since no XON/XOFF handshaking is provided.  If this becomes

a problem, we suggest that the Clear-to-Send bit be examined, where avail-

able, or that the user intercept characters coming back from the terminal

by patching the keyboard (input) interrupt vector and looking for XOFF and

XON.

TAP, RT-11, and a Non-console Serial Interface

This is similar to using the console serial interface, but instead of

ITTINR and ITTOUR use the analogous multi-terminal calls MTIN and MTOUT.

To use the assembly code routine above simply change the definitions of

SCSR and SBUF to the appropriate device addresses.

TAP, RSX11, and a Serial Interface

The ibyte/obyte package is written in RATFOR (FORTRAN).  It uses logical

number 1 for I/O and uses event flag 1 for the same purpose.  These may

be altered, if necessary.  It is assumed that the logical device "VG:"

has been assigned to a specific terminal (or to TTY).  The first call

causes a call to SETUP which assigns a LUN to the graphics terminal and

attaches it.  OBYTE sets up a qio to do a write-pass-all if one byte and

then does a wait-qio on it.  Higher performance would possibly be obtained

by doing non-wait qio's and callilng WAITFR (IFLAG) on the next entry - but

this requires that a flag can be reserved for that purpose.  IBYTE does a

send-pass-all-no-echo if one byte with wait mode qio.

There are four error conditions, all fatal:

1.  QIO directive failed for obyte - probably LUN did not assign

correctly.

2.  Ditto for IBYTE

3.  OBYTE's write qio failed - could be LUN assignment problems or

hardware problems.

4.  IBYTE's send failed - ditto.

C  FORTRAN VERSION - Low level tap routines - RSX

C------------------------------------------------------------------------

SUBROUTINE OBYTE (VAL)

BYTE VAL

INTEGER IOSB(2), IPARN (6)

COMMON/TAPIO/IFIRST,ILUN,IFLAG

IF 9IFIRST.EQ.0) CALL SETUP

CALL GETADR (IPARN (1), VAL)

IPARN (2) = 1

IPARN (3) = 0

CALL WTQIO (410,ILUN,IFLAG,,ISOB,IPARN,IDS)

IF (IDS.NE.1) STOP 'OBYTE'

IF (IOSB(1).NE.1)STOP 'OBYTE-2'

RETURN

END

C------------------------------------------------------------------------

SUBROUTINE IBYTE (VAL)

BYTE VAL

INTEGER IOSB(2),IPARN(6)

COMMON/TAPIO/IFIRST,ILUN,IFLAG

IF (IFIRST.EQ.0) CALL SETUP

CALL GETADR (IPARN(1), VAL)

IPARN (2) = 1

IPARN (3) = 0

CALL WTQIO 9'1030,ILUN,IFLAG,,IOSB,IPARN,IDS)

IF (IDS.NE.1) STOP 'IBYTE-1'

IF 9IOSB(1).NE.1) STOP 'IBYTE-2'

RETURN

END

C--------------------------------------------------------------------------

SUBROUTINE SETUP

COMMON/TAPIO/IFIRST,ILUN,IFLAG

DATA IFIRST/0/

DATA ILUN,IFLAG/1, 1/

CALL ASNLUN 9ILUN, 'VG', 0, IDS)

IF (IDS.NE.1) STOP 'SETUP-ASSIGN FAILED'

CALL WRQIO ("1400,ILUN)

IFIRST = 1

RETURN

END

#  RATFOR VERSION OF I/O BYTE ROUTINES

define          YES          0          #flags for first-time through

define          NO           1

define          LUN          1          #adjust to suit

define          FLAG         1

define          TIMEOUT0                #how many IO seconds to wait

before input error (0 - infinite)

define          IOATT        "1400"     #attach LUN function

define          IOWAL        "0410"     #write - pass - all - bits

define          IORALN       "1030"     #send - pass - all - no echo

define          SUCCESS1                #RSX success code

subroutine obyte (val)

byte val                                #an 8-bit quantity

integer iosb (2),ids,iparn(6),ifirst,ilun,iflag

common / tapio / ifirst,ilun,iflag

if (ifirst == YES) call setup

call getadr (iparn(1),val)

iparn(2) = 1

iparn(3) - 0

call wtqio (IOWAL, ilun,iflag,,iosb,iparn,ids)

if (ids n = success)  stop "OBYTE - QSO directive failed'

if (iosb(1) n = success)  stop 'OBYTE - write failed'

return

end

subroutine ibyte (val)

byte val

integer iosb(2),ids,iparn(6),ifirst,ilun,iflag

common / tapio / ifirst,ilun,iflag

if (ifirst == YES) CALL SETUP

call getadr (iparn(1), val)

iparn (2) = 1

iparn (3) = TIMEOUT

call wtqio (IORALN,ilun,iflag,,iosb,iparn,ids)

if (ids n-SUCCESS)       stop 'IBYTE -QIO directive failed

if (iosb(1) n-SUCCESS)   stop 'BYTE - read failed'

return

end

subroutine setup

common / tapio / ifirst,ilun,iflag

data ifirst/YES/

ilun - LUN

iflag = FLAG

call asnlun (ilun, 'VG',0/)

wall wtqio (IOATT, ilun)

ifirst = NO

return

end

TAP, Other Computer/Operating Systems and a Serial Interface

It is hoped that by rading the above sections you can obtain a sufficient

appreciation of the issues to devise your own IBYTE and OBYTE.  It is

important to assure that the operating system will not garble the input

or output stream such as attempting to convert tabs to spaces, or follow

carriage returns with  line feeds.

EXAMPLE - USING MODIFIED HEX CODING

Step 1.  Modify TAPINT to set up the terminal to accept modified hex

coding.  Simply add the following:

CALL OBYTE('G')

CALL OBYTE('1')

CALL OBYTE('M')

CALL OBYTE('M')

CALL OBYTE('M')

CALL OBYTE('7')

Step 2.  Modify IO512 to perform I/O in modified hex.  For example:

SUBROUTINE OPOUT(I)

CALL OBYTE(MOD(I,16) + 48)

RETURN

END