HP2100 Simulator Usage

30-Mar-2012

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2012, written by Robert M Supnik

Copyright (c) 1993-2012, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	HP2100 Features	4

2.1	2114C/2115A/2116C, 2100A, 1000-M/E/F CPUs	5

2.2	12581A/12892B Memory Protect	9

2.3	12607B/12578A/12895A DMA Controllers, 12897B DCPC Controller	10

2.4	Select Code Assignments	10

2.4.1	Device State	11

2.5	Programmed I/O Devices	11

2.5.1	12597A-002 Duplex Register Interface (PTR) with 2748 Paper Tape Reader	11

2.5.2	12597A-005 Duplex Register Interface (PTP) with 2895 Paper Tape Punch	12

2.5.3	12531C Buffered Teleprinter Interface (TTY) with 2752 Teleprinter	13

2.5.4	12966A Buffered Asynchronous Communications Interface (BACI)	14

2.5.5	12653A Printer Controller (LPS) with 2767 Line Printer,
12566B Microcircuit Interface with Loopback Connector	16

2.5.6	12845B Printer Controller (LPT) with 2607 Line Printer	17

2.5.7	12539C Time Base Generator (TBG)	18

2.5.8	12792C 8-Channel Asynchronous Multiplexer (MPX)	18

2.5.9	12920A 16-Channel Terminal Multiplexer (MUX, MUXL, MUXM)	22

2.5.10	12875A Interprocessor Link (IPLI, IPLO)	24

2.5.11	12620A/12936A Privileged Interrupt Fence (PIF)	25

2.6	Disk Controllers	26

2.6.1	12557A Disk Controller (DPC, DPD) with Four 2781 Drives,
13210A Disk Controller (DPC, DPD) with Four 7900 Drives	26

2.6.2	12565A Disk Controller (DQC, DQD) with Two 2883 Drives	28

2.6.3	12606B Fixed Head Disk Controller (DRC, DRD) with 2770/2771 Disk,
12610B Drum Controller (DRC, DRD) with 2773/2774/2775 Drum	29

2.6.4	13037D Disk Controller (DS) with Eight 7905/7906/7920/7925 Drives	30

2.6.5	12821A Disk Interface (DA) with Four 7906H/7920H/7925H Drives	33

2.7	Magnetic Tape	35

2.7.1	12559C Magnetic Tape Controller (MTC, MTD) with One 3030 Drive	35

2.7.2	13181A Magnetic Tape Controller (MSC, MSD) with Four 7970B Drives,
13183A Magnetic Tape Controller (MSC, MSD) with Four 7970E Drives	36

3	Symbolic Display and Input	38

This memorandum documents the HP 2100 simulator.

# Simulator Files

sim/		scp.h

sim_console.h

sim_defs.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_fio.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/hp2100/    hp2100_cpu.h

hp2100_cpu1.h

hp2100_defs.h

hp2100_fp.h

hp2100_fp1.h

hp2100_baci.c

hp2100_cpu.c

hp2100_cpu0.c

hp2100_cpu1.c

hp2100_cpu2.c

hp2100_cpu3.c

hp2100_cpu4.c

hp2100_cpu5.c

hp2100_cpu6.c

hp2100_cpu7.c

hp2100_di.c

hp2100_di.h

hp2100_di_da.c

hp2100_fp.c

hp2100_fp1.c

hp2100_dp.c

hp2100_dq.c

hp2100_dr.c

hp2100_ds.c

hp2100_ipl.c

hp2100_lps.c

hp2100_lpt.c

hp2100_mpx.c

hp2100_mt.c

hp2100_ms.c

hp2100_mux.c

hp2100_pif.c

hp2100_stddev.c

hp2100_sys.c

hp2100_diag.txt

hp_disclib.c

hp_disclib.h

# HP2100 Features

The HP2100 simulator is configured as follows:

device names	simulates

CPU			2114C CPU with up to 16KW of memory

2115A CPU with up to 8KW of memory

2116C CPU with up to 32KW of memory

2100A CPU with up to 32KW of memory

1000 (21MX) M/E/F-Series CPU with up to 1024KW of memory

EAU, FP, IOP, DMS, FFP, DBI, VIS, SIGNAL, RTE-IV EMA, and/or RTE-6/VM OS and VMA microcode extensions

MP			12581A/12892B memory protect

DMA0, DMA1		12607B/12578A/12895A direct memory access controller

DCPC0, DCPC1	12897B dual-channel port controller

PTR			12597A duplex register interface with 2748 paper tape

reader

PTP			12597A duplex register interface with 2895 paper tape

punch

TTY			12531C buffered teleprinter interface with 2752

teleprinter

BACI			12966A buffered asynchronous communications interface

LPS			12653A printer controller with 2767 line printer

12566B microcircuit interface with loopback connector

LPT			12845B printer controller with 2607 line printer

TBG			12539C time base generator

MPX			12792C 8-channel asynchronous multiplexer

MUX, MUXL, MUXM	12920A 16-channel terminal multiplexer

DPD, DPC		12557A disk controller with four 2871 drives

13210A disk controller with four 7900 drives

DQD, DQC		12565A disk controller with two 2883 drives

DRD, DRC		12606B fixed head disk controller with 2770/2771 disk

12610B drum controller with 2773/2774/2775 drum

DS			13037D disk controller with eight 7905/7906/7920/7925

drives

DA			12821A disk interface with four 7906H/7920H/7925H drives

MTD, MTC		12559C magnetic tape controller with one 3030 drive

MSD, MSC		13181A magnetic tape controller with four 7970B drives

13183A magnetic tape controller with four 7970E drives

IPLI, IPLO		12875A interprocessor link

PIF			12620A/12936A privileged interrupt fence

The simulator has been tested with and supports the following operating systems:

- 2000 E, F, and Access Time-Shared BASIC

- DOS-III

- RTE-II, III, IV, IVB, and 6/VM

In addition, the simulator generally passes the HP 24396 Diagnostics Suite; see hp2100_diag.txt for details.

The simulator implements several unique stop conditions:

- Attempted execution of an undefined instruction, and STOP_INST is set

- Attempted I/O to an unassigned select code, and STOP_DEV is set

- More than INDMAX indirect references are detected during memory reference address decoding

The LOAD command supports standard absolute binary format.  The DUMP command is not implemented.

Refer to the note in Section 2.5.1 regarding the use of the LOAD command with HP software.

## 2114C/2115A/2116C, 2100A, 1000-M/E/F CPUs

CPU options include choice of model, memory size, and instruction sets.  Several microcode options are simulated:

EAU			Extended Arithmetic Unit

FP			Single-Precision Floating Point

IOP			2000/Access I/O Processor

DMS			Dynamic Mapping System

FFP			Fast FORTRAN Processor

DBI			Double Integer Instructions

VIS			Vector Instruction Set

SIGNAL		SIGNAL/1000 Instructions

EMA/VMA		Extended Memory/Virtual Memory/OS Instructions

The following standard microcode is automatically enabled when the applicable 1000-Series CPU is selected:

EIG			Extended Instruction Group

FPP			Floating Point Processor

SIS			Scientific Instruction Set

The 2100 and 1000 CPUs supported user microprogramming.  Under simulation, execution on these machines of all instructions in the octal ranges 101400-101777 and 105000-105777 that are not allocated to installed firmware options will be dispatched to a user-alterable module to aid in the implementation of user-written microcode simulations.  In the absence of such simulations, execution will cause undefined instruction stops.  See the comments in the hp2100_cpu0.c source file for details.

The general command form is:

SET {-F} CPU <option>

Options that may be specified are:

SET CPU 2114			2114C CPU

SET CPU 2115			2115A CPU

SET CPU 2116			2116C CPU

SET CPU 2100			2100A CPU

SET CPU 1000-M			1000 M-Series CPU

SET CPU 1000-E			1000 E-Series CPU

SET CPU 1000-F			1000 F-Series CPU

SET CPU 21MX-M			21MX M-series CPU (same as 1000-M)

SET CPU 21MX-E			21MX E-series CPU (same as 1000-E)

SET CPU LOADERENABLE		enable device loader

SET CPU LOADERDISABLE		disable device loader

SET CPU EAU				EAU instructions

SET CPU NOEAU			no EAU instructions

SET CPU FP				FP instructions

SET CPU NOFP			no FP instructions

SET CPU IOP				IOP instructions

SET CPU NOIOP			no IOP instructions

SET CPU FFP				FFP instructions

SET CPU NOFFP			no FFP instructions

SET CPU DMS				DMS instructions

SET CPU NODMS			no DMS instructions

SET CPU DBI				Double Integer instructions

SET CPU NODBI			no Double Integer instructions

SET CPU VIS				Vector Instruction Set

SET CPU NOVIS			no Vector Instruction Set

SET CPU SIGNAL			SIGNAL instructions

SET CPU NOSIGNAL			no SIGNAL instructions

SET CPU EMA				Extended Memory Area instructions

SET CPU NOEMA			no Extended Memory Area instructions

SET CPU VMA				Virtual Memory Area and OS instructions

SET CPU NOVMA			no Virtual Memory Area and OS instructions

SET CPU 4K				set memory size = 4K

SET CPU 8K				set memory size = 8K

SET CPU 12K				set memory size = 12K

SET CPU 16K				set memory size = 16K

SET CPU 24K				set memory size = 24K

SET CPU 32K				set memory size = 32K

SET CPU 64K				set memory size = 64K

SET CPU 128K			set memory size = 128K

SET CPU 256K			set memory size = 256K

SET CPU 512K			set memory size = 512K

SET CPU 1024K			set memory size = 1024K

The CPU models and feature applicability are shown in the following table:

Feature	2114C	2115A	2116C	2100A	1000M	1000E	1000F

Max mem	16K	8K	32K	32K	1024K	1024K	1024K

MP	no	no	opt	std	opt	opt	opt

DMA	opt	opt	opt	opt	opt	opt	opt

EAU	no	opt	opt	std	std	std	std

FP	no	no	no	opt	std	std	std

IOP	no	no	no	opt	opt	opt	opt

FFP	no	no	no	opt	opt	opt	std

EIG	no	no	no	no	std	std	std

DMS	no	no	no	no	opt	opt	opt

DBI	no	no	no	no	no	opt	std

EMA	no	no	no	no	no	opt	opt

VMA/OS	no	no	no	no	no	opt	opt

FPP	no	no	no	no	no	no	std

SIS	no	no	no	no	no	no	std

VIS	no	no	no	no	no	no	opt

SIGNAL	no	no	no	no	no	no	opt

If a feature is standard for a given CPU, it cannot be disabled; if a feature is not included, it cannot be enabled.

On the 2100, the FP or FFP option and the IOP option are mutually exclusive.  On the 1000-E/F, the RTE-IV EMA and RTE-6/VM VMA and OS options are mutually exclusive.  On the 1000-F, the VIS and IOP options are mutually exclusive.  To support CPU detection in the HP diagnostic configurator, the 1000-E/F TIMER instruction decodes as MPY on the 1000-M, and the RRR 16 instruction decodes as NOP on the 2114.  The 1000-F FPP supports several undocumented instructions used by the FPP-SIS-FFP diagnostic.  All other disabled or undefined instructions cause a simulator halt if STOP_INST is set and execute as NOP if not.

Setting the CPU model establishes a consistent set of standard features and common options.  Additional SET CPU commands may follow to fine-tune the desired feature set.

The 2114, 2115, 2116, and 2100 models support a protected area of memory containing an initial binary loader.  The loader always resides in the highest 64 memory locations, regardless of capacity.  Enabling the loader makes this area available.  Disabling the loader renders this area non-existent, so that reads from the area return zero, and writes to the area are ignored.  The paper tape loader, configured for select code 10, initially resides in this area when the simulator is started.  The LOAD command may be used with an appropriate absolute binary file to install a different loader.

The initial memory size is set to a maximum of 32K.  If the memory size is being reduced, either by setting a smaller size or by changing to a CPU with a smaller memory capacity, and the memory being truncated contains non-zero data, the simulator asks for confirmation before proceeding.  The confirmation request may be suppressed by using the -f switch.  Data in the truncated portion of memory is lost.

These switches are recognized when examining or depositing to CPU memory:

-n			interpret address without mapping (physical address)

-v			if DMS enabled, use current map, else no mapping

-s			if DMS enabled, use system map, else illegal

-u			if DMS enabled, use user map, else illegal

-p			if DMS enabled, use port A map, else illegal

-q			if DMS enabled, use port B map, else illegal

If no switch is specified, the default is to use -v, i.e., the address is virtual using the current map if DMS is enabled; otherwise the address is physical.

The simulator attempts to detect when the CPU is idle.  When idle, the simulator does not use any host system processor time.  Idle detection is controlled by these commands:

SET CPU IDLE			enable idle detection

SET CPU NOIDLE			disable idle detection

Idle detection is operating-system specific and is currently supported under all versions of the DOS and RTE operating systems.  It is disabled by default.

The CPU implements four different kinds of instruction breakpoints:

-e			break unconditionally

-n			break if DMS is disabled

-s			break if DMS and system map are enabled

-u			break if DMS and user map are enabled

If no switch is specified, the default is to use -n if DMS is currently disabled, otherwise to use –s if the system map or –u if the user map is currently enabled.  In a DMS environment, breakpoints on interrupt trap cells should specify the –s switch, as the system map will be enabled when instruction execution is attempted.

When the simulator stops after reaching a breakpoint or completing a STEP command, the current program counter (P register) value and the next instruction to execute are displayed.  For example:

Simulation stopped, P: 03306 (JMP 3311)

sim>

If an interrupt is pending, however, the instruction contained in the trap cell corresponding to the highest-priority interrupting device will be displayed instead:

Simulation stopped, P: 03306 (IAK 11: JSB 1644,I)

sim>

This indicates that when simulation resumes, the device with select code 11 will receive the interrupt acknowledgement, and JSB 1644,I will be executed.  Note that P does not change when an interrupt occurs and a trap cell instruction is executed.

The CPU may be configured to send debugging information to the previously enabled debug output device using these commands:

SET CPU DEBUG			provide debug printouts

SET CPU NODEBUG			inhibit debug printouts

Six debug flags that control the reporting level are supported:

OS			Report RTE OS firmware instruction executions, except

for time base generator interrupt instructions

OSTBG			Report RTE OS TBG firmware instruction executions

VMA			Report RTE VMA firmware instruction executions

EMA			Report RTE EMA firmware instruction executions

VIS			Report Vector Instruction Set firmware executions

SIG			Report SIGNAL firmware instruction executions

Because the time base generator interrupts 100 times per second and executes three firmware instructions per interrupt, a large number of debug statements can be produced very quickly.  Therefore, tracing of these sequences must be enabled separately from the other operating system accelerator instructions.

CPU registers include the visible state of the processor and control registers for the interrupt system.

name			models	size	comments

P			all		15	program counter

A			all		16	A register

B			all		16	B register

M			all		15	M (memory address) register

T			all		16	T (memory data) register

X			1000		16	X index register

Y			1000		16	Y index register

S			all		16	switch/display register

E			all		1	extend flag

O			all		1	overflow flag

ION			all		1	interrupt enable flag

ION_DEFER		all		1	interrupt defer flag

CIR			all		6	central interrupt register

DMSENB		1000		1	DMS enabled flag

DMSCUR		1000		1	DMS current map (0/1 = system/user)

DMSSR			1000		16	DMS status register

DMSVR			1000		16	DMS violation register

DMSMAP[0:127]	1000		16	DMS map registers

[0:31] system map

[32:63] user map

[64:95] port A map

[96:127] port B map

IOPSP			2100		16	I/O processor stack pointer

STOP_INST		all		1	stop on undefined instruction

STOP_DEV		all		1	stop on undefined device

INDMAX		all		16	indirect address limit

PCQ[0:63]		all		15	origin of last programmed jump,

most recent change first

BOOT CPU implements the 1000 Initial Binary Loader facility.  The IBL is controlled by the switch register S.  S<15:14> selects the device to boot:

00			2748B paper-tape reader (12992K ROM)

01			7900A/2883 disk (12992A ROM)

10			7970B/E tape (12992D ROM)

11			13037D disk (12992B ROM)

For the 7900A/2883 only, S<13:12> specifies the type of disk:

00			7900A

10			2883

S<11:6> contains the select code.  If the device has two select codes, S<11:6> specifies the lower one.  S<5:3> are passed to the bootstrap program.  S<2:0> specify options for the boot loader.  The IBL will not report an error if the select code in S<11:6> is incorrect.

## 12581A/12892B Memory Protect

Memory protect is standard equipment on the 2100 (although it may be disabled by removing a jumper) and optional on the 2116 and 1000.  The following registers are implemented:

name			size		comments

CTL			1		memory protection enable

FLG			1		protection violation flag

FBF			1		protection violation flag buffer

FR			15		fence register

VR			16		violation register

EVR			1		enable violation register flag

MEV			1		memory expansion (DMS) violation flag

The 1000 memory protect card (12892B) has three feature options that are implemented by jumper settings.  These are controlled by the following commands:

SET MP JSBIN			jumper W5 installed

SET MP JSBOUT			jumper W5 removed

SET MP INTIN			jumper W6 installed

SET MP INTOUT			jumper W6 removed

SET MP SEL1IN			jumper W7 installed

SET MP SEL1OUT			jumper W7 removed

W5 determines whether JSB instructions referencing memory locations 0 and 1 are legal (installed) or illegal (removed).  W6 controls whether the first three levels of indirect addressing hold off (installed) or permit (removed) pending interrupts.  W7 determines whether I/O instructions referencing select codes other than 1 are legal (installed) or illegal (removed); note that I/O instructions referencing select code 1 are legal, and HLT instructions are illegal, regardless of the setting of W7.

The default configuration is JSB (W5) installed, INT (W6) installed, and SEL1 (W7) removed, providing compatibility with the 2116 and 2100 memory protect cards.

## 12607B/12578A/12895A DMA Controllers, 12897B DCPC Controller

The direct memory access/dual channel port controller is an option for all CPUs.  The 12607B DMA controller provides one channel, DMA0, on the 2114.  The other DMA/DCPC controllers provide two channels (DMA0 and DMA1 on the 2115, 2116, and 2100, or DCPC0 and DCPC1 on the 1000).  Each channel has the following visible state:

name			models	size	comments

XFR			 all		 1	channel transfer enabled

CTL			 all		 1	interrupt enabled

FLG			 all		 1	channel ready

FBF			 all		 1	channel ready buffer

CTL2			 all		 1	command word 2/3 selector

CW1			 all		16	command word 1

CW2			 all		16	command word 2

CW3			 all		16	command word 3

BYTE			12578A	 1	even/odd byte-packing flag

PACKER		12578A	 8	byte-packing register

## Select Code Assignments

HP 21xx/1000 I/O interfaces receive their select code assignments from the backplane slot into which they are plugged.  Thus, select code assignments vary considerably from system to system and software package to software package.  The HP2100 simulator supports dynamic select code assignment.  To show the current select code assigned to a device, use the SHOW <dev> SC command:

sim> SHOW PTR SC

select code=10

To change the select code, use the SET <dev> SC=<num> command:

sim> SET PTR SC=30

sim> SHOW PTR SC

select code=30

The new select code must be in the range 10-77 octal.  For devices with two select codes, <num> specifies the lower number; the higher is automatically set to <num>+1.  If a select code conflict occurs, the simulator will report the error when program execution is attempted.

The hardware requires that no empty slots exist between interface cards.  This is not required under simulation; empty slots behave as though they contain HP 12777A Priority Jumper Cards.

### Device State

All devices other than the CPU and TTY may be disabled or enabled.  Disabling a device simulates removing the associated interface from the CPU card cage.  To disable or enable a device, use:

SET <dev> DISABLED		disable device

SET <dev> ENABLED			enable device

For devices with more than one select code, disabling or enabling any device in the set disables or enables all of the devices.

Devices consisting of multiple addressable units connected to a controller typically allow the units to be individually enabled or disabled.  Disabling simulates disconnecting the associated unit from the controller.  The commands to set units enabled and disabled are:

SET <unit> DISABLED		disable unit

SET <unit> ENABLED 		enable unit

Some devices and units allow simulation of power-down conditions.  Power settings are controlled by these commands:

SET <dev> POWEROFF		turn power off

SET <dev> POWERON			turn power on

Peripherals that provide operator-selectable disconnection, typically via an "offline" switch, provide these simulation equivalents:

SET <dev> OFFLINE			set peripheral offline

SET <dev> ONLINE			set peripheral online

## Programmed I/O Devices

### 12597A-002 Duplex Register Interface (PTR) with 2748 Paper Tape Reader

The paper tape reader (PTR) reads data from a disk file.  The POS register specifies the number of the next data byte to read.  Thus, by changing POS, the user can backspace or advance the reader.  For example, setting POS to 0 rewinds the attached tape.

For diagnostic purposes, a tape loop may be simulated with the commands:

SET PTR DIAG			rewind tape at EOF

SET PTR READER			supply tape trailer at EOF

The paper tape reader supports the BOOT command.  BOOT PTR copies the HP 12992K boot loader ROM into memory and starts it running.  The switch register (S) is modified automatically to the value expected by the loader:

<15:14>	=	00

<13:12>	=	00

<11:6>	=	reader select code

<5:3> 	= 	unchanged

<2:0>		=	000

Usage note: the LOAD command does not alter the A and B registers, whereas BOOT PTR leaves A and B set to zero after a successful load.  BOOT PTR is preferred when loading HP software that may depend on the initial values of the A and B registers.

The paper tape reader implements these registers:

name			size		comments

BUF			8		last data item processed

CTL			1		device/interrupt enable

FLG			1		device ready

FBF			1		device ready buffer

TRLLIM		8		number of trailing nulls to append

after end-of-file is detected

POS			32		position in the input file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

The TRLLIM register specifies the number of nulls to supply as paper tape trailer when EOF is detected.  If input is attempted when TRLLIM is set to zero or the count is exhausted, the reader will hang.

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

end of file		1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

### 12597A-005 Duplex Register Interface (PTP) with 2895 Paper Tape Punch

The paper tape punch (PTP) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the punch.

The paper tape punch implements these registers:

name			size		comments

BUF			8		last data item processed

CTL			1		device/interrupt enable

FLG			1		device ready

FBF			1		device ready buffer

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

### 12531C Buffered Teleprinter Interface (TTY) with 2752 Teleprinter

The console teleprinter has three units: keyboard (unit 0), printer (unit 1), and punch (unit 2).  The keyboard reads from the console keyboard; the printer writes to the simulator console window.  The punch writes to a disk file.  The keyboard and printer units (TTY0, TTY1) can be set to one of four modes: UC, 7P, 7B, or 8B:

mode	input characters		output characters

UC	lower case converted	lower case converted to upper case,

to upper case,		high-order bit cleared,

high-order bit cleared	non-printing characters suppressed

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is UC.

Some HP software systems expect the console to transmit line-feed automatically following carriage-return.  This feature is enabled with:

SET TTY AUTOLF

and disabled with:

SET TTY NOAUTOLF

The console teleprinter implements these registers:

name			size		comments

BUF			8		last data item processed

MODE			16		mode

CTL			1		device/interrupt enable

FLG			1		device ready

FBF			1		device ready buffer

KPOS			32		number of characters input

KTIME			24		keyboard polling interval

TPOS			32		number of characters printed

TTIME			24		time from I/O initiation to interrupt

PPOS			32		position in the punch output file

STOP_IOE		1		punch stop on I/O error

Error handling for the punch is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

### 12966A Buffered Asynchronous Communications Interface (BACI)

The BACI was designed specifically to interface HP 264x and HP 262x CRT terminals to the HP 1000 via hardwired and modem connections.  It contains a 128-byte FIFO to support block-mode reads from the terminals.  Under simulation, it connects with an HP terminal emulator via Telnet to a user-specified port.

The ATTACH command specifies the port to be used, and DETACH disconnects the port:

ATTACH BACI <port>		set up listening port

DETACH BACI				disconnect port

where <port> is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  Once the BACI is attached and the simulator is running, the specified port will be monitored for Telnet connections.  The port may be forcibly disconnected from its associated Telnet session with:

SET BACI DISCONNECT		terminate Telnet session

Connections remain open until disconnected by the Telnet client, by a DETACH BACI command, or by a SET BACI DISCONNECT command.

A diagnostic mode is provided for use with the 12966-16001 Asynchronous Data Set diagnostic:

SET BACI DIAG			configure for diagnostic tests

SET BACI TERMINAL			configure for terminal connections

Enabling the diagnostic mode simulates the installation of the HP 12966-60003 diagnostic test (loopback) connector onto the 12966A card.  Enabling terminal mode simulates the installation of the HP 12966-60008 (264x terminal) connector and RS-232 cable.

The BACI supported fifteen programmable baud rates from 50 to 9600 baud, as well as an external rate determined by the connected terminal, which was usually set for 9600 baud as well.  The simulator can be set to model the actual transmission times (in machine instructions elapsed per character), or it can be set to operate substantially faster with these commands:

SET BACI REALTIME			use realistic timing

SET BACI FASTTIME			use optimized timing

Setting the BACI to fast timing enables three optimizations.  First, buffered characters are transferred via Telnet in blocks, rather than a character at a time.  Second, ENQ/ACK handshaking is done locally, without involving the Telnet client.  Third, reception of characters entered during output operations is delayed until after the first ENQ/ACK handshake at the end of the line; this significantly improves break-mode response under RTE.

Many HP operating systems require command input in upper case, although mixed-case output is supported.  As an aid to avoid toggling the host keyboard in and out of CAPS LOCK mode, the BACI provides this function locally:

SET BACI CAPSLOCK			upshift lower case input to upper case

SET BACI NOCAPSLOCK		characters are processed as received

Logging of BACI output to a file may be turned on and off with the following commands:

SET BACI LOG=<filename>		log terminal output to a file

SET BACI NOLOG			terminate output logging

Information about BACI operation is provided by these commands:

SHOW BACI CONNECTION		displays active connection address and time

SHOW BACI STATISTICS		displays active connection I/O statistics

The BACI may be configured to send debugging information to the previously enabled debug output device by using these commands:

SET BACI DEBUG{=f1;f2;...}	provide debug printouts

SET BACI NODEBUG{=f1;f2;...}	inhibit debug printouts

SHOW BACI DEBUG			display current debug settings

Four debug flags that control various levels of reporting are supported:

CMDS					Command initiations and completions

CPU					Words received from and sent to the CPU

BUF					Data read from and written to the FIFO

XFER					Data received and transmitted via Telnet

If individual flags are not specified in the SET BACI DEBUG and SET BACI NODEBUG commands, then all reporting levels are implied.

The BACI implements these registers:

name			size		comments

IBUF			16		input buffer (from card to CPU)

OBUF			16		output buffer (from CPU to card)

STATUS		16		card status word

EDSIW			16		enable device status interrupt word

DSRW			16		device status reference word

CFCW			16		character frame control word

ICW			16		interface control word

ISRW			16		interrupt status reset word

FIFOPUT		 8		FIFO load index

FIFOGET		 8		FIFO unload index

FIFOCNTR		 8		FIFO character counter

BRKCNTR		16		BREAK clock counter

FIFO[0:127]		 8		FIFO buffer

SPCHAR[0:255]	 1		special character RAM

UARTTHR		16		UART transmitter holding register

UARTTR		16		UART transmitter register

UARTRHR		16		UART receiver holding register

UARTRR		16		UART receiver register

UARTCLK		16		UART transmitter/receiver clock counter

CTIME			19		inter-character poll time

LKO			 1		interface interrupt lockout

CTL			 1		interface control

FLG			 1		interface flag

FBF			 1		interface flag buffer

SRQ			 1		interface DMA service request

While the BACI can connect to generic Telnet clients, RTE drivers for the BACI required the use of HP terminals.  QCTerm, a free HP 700/92 terminal emulator for Windows, is available from AICS Research at http://www.aics-research.com/qcterm/.

### 12653A Printer Controller (LPS) with 2767 Line Printer,
12566B Microcircuit Interface with Loopback Connector

The 2767 line printer uses the 12653A line printer interface as its controller.  As a line printer, LPS writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer responds to SET LPS POWEROFF as if the power were removed or the printer cable were disconnected, and to DETACH LPS as if the paper were out.  It also provides these additional state commands:

SET LPS OFFLINE			simulate ONLINE button up

SET LPS ONLINE			simulate ONLINE button down

SET LPS REALTIME			use realistic timing for print operations

SET LPS FASTTIME			use optimized timing for print operations

As a 12566B microcircuit interface, LPS provides the test device for running several of the HP diagnostics.  Printer mode versus diagnostic mode is controlled by the commands:

SET LPS PRINTER			configure as line printer

SET LPS DIAG			configure for diagnostic tests

In diagnostic mode, LPS simulates the installation of the HP 1251-0332 diagnostic test (loopback) connector onto the 12566B card and the setting of the appropriate card jumpers for the diagnostic.

LPS may be configured to send debugging information to the previously enabled debug output device using these commands:

SET LPS DEBUG			provide debug printouts

SET LPS NODEBUG			inhibit debug printouts

Diagnostic information includes characters supplied to and status received from the interface, as well as data transfer initiations and completions.

The 12653A is disabled by default.

The 12653A implements these registers:

name			size		comments

BUF			16		output buffer

STA			16		input buffer or status

POWER			2		printer power state (read-only)

CTL			1		device/interrupt enable

FLG			1		device ready

FBF			1		device ready buffer

CCNT			7		current character count

LCNT			7		current line count

POS			32		position in the output file

CTIME			24		character transfer time

PTIME			24		per-zone print operation time

STIME			24		per-line paper slew time

RTIME			24		power-on ready delay time

STOP_IOE		1		stop on I/O error

In printer mode, error handling is as follows:

error	       STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

SET POWEROFF	1		report error and stop

0		powered off

SET OFFLINE		1		report error and stop

0		offline

OS I/O error	x		report error and stop

With STOP_IOE set to 0, output performed when the device is powered off or offline will initiate but then hang, waiting for the device to be returned online.  When it is, the output operation will complete.

In diagnostic mode, there are no errors; data sent to the output buffer is looped back to the status register immediately.

### 12845B Printer Controller (LPT) with 2607 Line Printer

The line printer (LPT) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer responds to SET LPT POWEROFF as if the power were removed or the printer cable were disconnected and DETACH LPT as if the paper were out.  It also provides these additional state commands:

SET LPT OFFLINE			simulate PAPER button up

SET LPT ONLINE			simulate PAPER button down

The line printer implements these registers:

name			size		comments

BUF			7		last data item processed

CTL			1		device/interrupt enable

FLG			1		device ready

FBF			1		device ready buffer

LCNT			7		line count within page

POS			32		position in the output file

CTIME			24		time between characters

PTIME			24		time for a print operation

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	       STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

SET POWEROFF	1		report error and stop

0		powered off

SET OFFLINE		1		report error and stop

0		offline

OS I/O error	x		report error and stop

With STOP_IOE set to 0, output performed when the device is powered off or offline will initiate but then hang, waiting for the device to be returned online.  When it is, the output operation will complete.

### 12539C Time Base Generator (TBG)

The time base generator (TBG) may be set for diagnostic mode:

SET TBG DIAG			configure for diagnostic mode

SET TBG CALIBRATED		configure for calibrated timing mode

Diagnostic mode corresponds to setting jumper W2 to position "B.”  This turns off autocalibration and divides the longest time intervals down by 1000.

The time base generator implements these registers:

name			size		comments

SEL			3		time base select

CTR			14		repeat counter for < 1Hz operation

CTL			1		device/interrupt enable

FLG			1		device ready

FBF			1		device ready buffer

ERR			1		error flag

TIME[0:7]		31		clock intervals, select = 0-7

IPTICK		24		calculated CPU instructions per clock tick

The time base generator autocalibrates; the clock interval is adjusted up or down so that the clock tracks actual elapsed time.  Operation at the fastest rates (100 microseconds, 1 millisecond) is not recommended.

### 12792C 8-Channel Asynchronous Multiplexer (MPX)

The 12792C was an eight-line asynchronous serial multiplexer that connected terminals, modems, serial printers, and other devices that used the RS-232 standard to the CPU.  It used an on-board Z80 microprocessor and provided input and output buffering to support block-mode reads from HP 264x and 262x terminals.  The card handled character editing, echoing, and read-terminator detection, reducing the load on the CPU.  Under simulation, it connects with HP terminal emulators via Telnet to a user-specified port.

#### MPX Device Commands

The ATTACH command specifies the Telnet port to be used to establish connections, and DETACH disconnects both the multiplexer control and all line sessions:

ATTACH MPX <port>			set up listening port

DETACH MPX				disconnect multiplexer and all lines

...where <port> is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  Once the multiplexer is attached and the simulator is running, the specified port will be monitored for Telnet connections.

The order in which connections are assigned to multiplexer lines may be specified and displayed with these commands:

SET MPX LINEORDER=l1{;l2;...}	set line connection order

SHOW MPX LINEORDER		show line connection order

...where <ln> may be a single line number or a range of line numbers of the form m-n.  Telnet connections to the listening port will be assigned to multiplexer lines in the sequence specified.  In the absence of a SET MPX LINEORDER command, connections will be assigned by default in ascending line order.  The default order may be reestablished by specifying this command:

SET MPX LINEORDER=ALL		restore default ascending connection order

A multiplexer line may be forcibly disconnected from its associated Telnet session with:

SET MPX DISCONNECT=<n>		terminate Telnet session on line <n>

Connections remain open until disconnected by the Telnet client, by a DETACH MPX command, or by a SET MPX DISCONNECT command.

Information about multiplexer operation is provided by these commands:

SHOW MPX CONNECTIONS		displays connection addresses and times

SHOW MPX STATISTICS		displays connection I/O statistics

The multiplexer may be configured to send debugging information to the previously enabled debug output device by using these commands:

SET MPX DEBUG{=f1;f2;...}	provide debug printouts

SET MPX NODEBUG{=f1;f2;...}	inhibit debug printouts

SHOW MPX DEBUG			display current debug settings

Four debug flags that control various levels of reporting are supported:

CMDS					Command initiations and completions

CPU					Words received from and sent to the CPU

BUF					Data read from and written to line buffers

XFER					Data received and transmitted via Telnet

If individual flags are not specified in the SET MPX DEBUG and SET MPX NODEBUG commands, then all reporting levels are implied.

#### MPX Line Commands

The 12792C supported twelve programmable baud rates from 50 to 19200 baud.  The simulator can be set to model the actual transmission times (in machine instructions elapsed per character), or it can be set to operate substantially faster with these commands:

SET MPX<n> REALTIME		use realistic timing

SET MPX<n> FASTTIME		use optimized timing

...where <n> is the desired line number.  Setting the multiplexer to fast timing enables three optimizations.  First, buffered characters are transferred via Telnet in blocks, rather than a character at a time.  Second, ENQ/ACK handshaking is done locally, without involving the Telnet client.  Third, when editing and echo is enabled, entering BS echoes a backspace, a space, and a backspace, and entering DEL echoes a backslash, a carriage return, and a line feed, providing better compatibility with prior RTE terminal drivers.

Many HP operating systems require command input in upper case, although mixed-case output is supported.  As an aid to avoid toggling the host keyboard in and out of CAPS LOCK mode, the multiplexer provides this function locally:

SET MPX<n> CAPSLOCK		upshift lower case input to upper case

SET MPX<n> NOCAPSLOCK		characters are processed as received

Logging of multiplexer output to a file may be turned on and off with the following commands:

SET MPX<n> LOG=<filename>	log line <n> output to a file

SET MPX<n> NOLOG			terminate output logging

#### MPX Implementation

The multiplexer is simulated with ten units:

unit					function

0					I/O line 0

1					I/O line 1

2					I/O line 2

3					I/O line 3

4					I/O line 4

5					I/O line 5

6					I/O line 6

7					I/O line 7

8					Card controller

9					Telnet line poll

Units 8 and 9 are disabled so that they do not appear in the normal unit list, but they will appear in the simulator event queue.

There were four major revisions to the multiplexer firmware, designated revisions A-D.  Revisions A-C had an upward-compatible command set and were supported under RTE-M, RTE-IVB, and RTE-6/VM.  Revision D implemented completely different commands, required different RTE drivers, and was supported only under RTE-6/VM.  This simulation implements revision C.

Revisions B and C added support for the 37214A Systems Modem subsystem and the RTE-A Virtual Control Panel (VCP), as the same firmware was used in the A-Series version of the multiplexer (HP 12040B/C).  Under simulation, the modem commands return status codes indicating that no modems are present, and the VCP commands are not implemented.

After pressing the CPU front-panel PRESET button or executing a CLC 0, the multiplexer would enter “echoplex” mode.  In this mode, each line would default to 9600 baud, 8 bits per character, 1 stop bit, no parity, and would echo characters as received.  Echoplex is implemented under simulation; until a line is configured by the appropriate RTE driver control call, characters entered in a Telnet session will be echoed.

The 12792C had two baud-rate generators that were assigned to lines by the wiring configuration in the I/O cable connector hood.  Two counter/timer circuits were used to implement the BRGs for all eight lines.  For lines connected to the same BRG, only subsets of the configurable rates were allowed, and assigning mutually incompatible rates caused corruption of the rates on lines assigned earlier.  Under simulation, any baud rate may be assigned to any line without interaction.

While the multiplexer can connect to generic Telnet clients, RTE programs generally assumed the use of HP terminals.  QCTerm, a free HP 700/92 terminal emulator for Windows, is available from AICS Research at http://www.aics-research.com/qcterm/.

The multiplexer does not support save and restore.  All open connections are lost when the simulator shuts down.

#### MPX Registers

The multiplexer implements these registers:

name			size		comments

STATE			 3		controller state (idle, cmd, param, exec)

IBUF			16		input buffer (from multiplexer to CPU)

OBUF			16		output buffer (from CPU to multiplexer)

CMD			 8		command code

PARAM			16		command parameter

PORT			 8		port number for last command

PORTKEY		 8		port key for last command

IOLEN			16		I/O transfer length for last command

UIEN			 1		unsolicited interrupts enabled

UIPORT		 3		unsolicited interrupt port

UICODE		 3		unsolicited interrupt reason code

KEYS[0:7]		 8		port keys, ports 0-7

PCONFIG[0:7]	16		port configuration, ports 0-7

RCVTYPE[0:7]	16		receive type, ports 0-7

CHARCNT[0:7]	16		character count, ports 0-7

FLOWCNTL[0:7]	16		flow control, ports 0-7

ENQCNTR[0:7]	 7		ENQ character counter, ports 0-7

ACKWAIT[0:7]	10		ACK wait counter, ports 0-7

PFLAGS[0:7]		12		port flags, ports 0-7

RBUF[0:7][0:513]	 8		read buffers, ports 0-7

WBUF[0:7][0:513]	 8		write buffers, ports 0-7

GET[0:7][0:1]	16		read/write get indexes, ports 0-7

SEP[0:7][0:1]	16		read/write separator indexes, ports 0-7

PUT[0:7][0:1]	16		read/write put indexes, ports 0-7

CTL			 1		interface control

FLG			 1		interface flag

FBF			 1		interface flag buffer

### 12920A 16-Channel Terminal Multiplexer (MUX, MUXL, MUXM)

The 12920A is a 16-line terminal multiplexer, with five additional receive-only diagnostic lines.  It consists of three devices:

MUX		scanning logic (corresponds to the upper data card)

MUXL		individual lines (corresponds to the lower data card)

MUXM		modem control and status logic (corresponds to the control

card)

The MUX performs input and output through Telnet sessions connected to a user-specified port.  The ATTACH command to the scanning logic specifies the port to be used:

ATTACH MUX <port>			set up listening port

where <port> is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.

Each line (each unit of MUXL) can be set to one of four modes: UC, 7P, 7B, or 8B:

mode	input characters		output characters

UC	lower case converted	lower case converted to upper case,

to upper case,		high-order bit cleared,

high-order bit cleared	non-printing characters suppressed

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is UC.  If the MUX is used with RTE driver DVS00 and logical driver LDV05, or DOS driver DVR73 and logical drivers PMT01 or PMT02, the mode must be changed to 7B or 8B to allow ESC, DC1, and ENQ characters to pass through to the terminal.  Alternatively, the SET CONSOLE PCHAR command may be used to redefine the set of printable characters.  Modes UC and 7P are compatible with DVS00 and DVR73 using the default logical driver.

In addition, each line supports the DATASET option:

SET MUXLn DATASET			enable modem control on line n

SET MUXLn NODATASET		disable modem control on line n

When modem control is enabled, a connected line that has DTR (command bit C1) high will set CD (status bit S2) in the line status, and a programmed drop of the DTR line will disconnect the Telnet session.

Finally, each line supports output logging:

SET MUXLn LOG=filename		log output of line n to filename

SET MUXLn NOLOG			terminate output logging of line n

Terminating logging also closes the log file.

The modem controls model a simplified Bell 103A dataset with just four lines: data terminal ready and request to send from the computer to the data set, and carrier detect and data set ready from the data set to the computer.  There is no ring detection.  A new Telnet connection sets data set ready and is established whether data terminal ready is set or not.

Once MUX is attached and the simulator is running, the multiplexer listens for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connections remain open until disconnected either by the Telnet client, a SET MUXL DISCONNECT command, or a DETACH MUX command.

The order in which connections are assigned to multiplexer lines may be specified and displayed with these commands:

SET MUX LINEORDER=l1{;l2;...}	set line connection order

SHOW MUX LINEORDER		show line connection order

...where <ln> may be a single line number or a range of line numbers of the form m-n.  Telnet connections to the listening port will be assigned to multiplexer lines in the sequence specified.  In the absence of a SET MUX LINEORDER command, connections will be assigned by default in ascending line order.  The default order may be reestablished by specifying the command:

SET MUX LINEORDER=ALL		restore default ascending connection order

Other special multiplexer commands:

SET MUX DIAG			configure for diagnostic tests

SET MUX TERM			configure for terminal connections

SHOW MUX CONNECTIONS 		show current connections

SHOW MUX STATISTICS 		show statistics for active connections

SET MUXLn DISCONNECT		disconnects the specified line

Enabling the diagnostic mode simulates the installation of eight HP 30062-60003 diagnostic test (loopback) cables between lines 0-1, 2-3, etc., as required by the multiplexer diagnostic.  In addition, all Telnet sessions are disconnected, and the multiplexer is detached from the listening port.  While in diagnostic mode, the ATTACH MUX command is not allowed.  Enabling terminal mode allows the multiplexer to be attached to accept incoming connections again.

The multiplexer may be configured to send debugging information to the previously enabled debug output device using these commands:

SET MUX DEBUG			provide debug printouts

SET MUX NODEBUG			inhibit debug printouts

Three debug flags that control the reporting level are supported:

CMDS					Command initiations and completions

CPU					Words received from and sent to the CPU

XFER					Data transferred via Telnet or loopback

The scanner (MUX) implements these registers:

name			size		comments

IBUF			16		input buffer, holds line status

OBUF			16		output buffer, holds channel select

The lines (MUXL) implements these registers:

name			size		comments

CTL			1		device/interrupt enable

FLG			1		device ready

FBF			1		device ready buffer

STA[0:20]		16		line status, lines 0-20

RPAR[0:20]		16		receive parameters, lines 0-20

XPAR[0:15]		16		transmit parameters, lines 0-15

RBUF[0:20]		16		receive buffer, lines 0-20

XBUF[0:15]		16		transmit buffer, lines 0-15

RCHP[0:20]		1		receive character present, lines 0-20

XDON[0:15]		1		transmit done, lines 0-15

BDFR[0:15]		1		break deferred, lines 0-15

TIME[0:15]		24		transmit time, lines 0-15

The modem control (MUXM) implements these registers:

name			size		comments

CTL			1		device/interrupt enable

FLG			1		device ready

FBF			1		device ready buffer

SCAN			1		scan enabled

CHAN			4		current line

DSO[0:15]		6		C2,C1,ES2,ES1,SS2,SS1, lines 0-15

DSI[0:15]		2		S2,S1, lines 0-15

The terminal multiplexer does not support save and restore.  All open connections are lost when the simulator shuts down or MUX is detached.

### 12875A Interprocessor Link (IPLI, IPLO)

The interprocessor link is a pair of 12566A parallel interfaces that are cross-coupled to provide interprocessor communications to another pair of interfaces in a second copy of the HP2100 simulator.  The IPL is intended to support simulation of a two-system HP 2000 Time-Shared BASIC configuration and occupies two adjacent I/O slots in both the system and I/O processors.  The input-line interface is installed in the higher-priority (lower-numbered) slot, and the output-line interface is installed in the lower-priority (higher-numbered) slot.  Each interface is actually a bi-directional, half-duplex line that is used in the indicated direction for commands and in the reverse direction for status.  The IPL is disabled by default.

To operate, the IPL devices must be enabled and then connected to the IPL devices in another copy of the simulator.  The IPLI device in the first simulator is connected to the IPLO device in the second and vice versa.  Connections are established with the ATTACH command.  One copy of the simulator listens for connections on the specified ports (ATTACH -L), and the other establishes connections to those ports (ATTACH -C).  Either copy may perform either operation, but the operations must be done in matched pairs:

simulator #1	simulator #2

sim> SET IPLI ENABLED	sim> SET IPLI ENABLED

(also enables IPLO)	(also enables IPLO)

sim> ATTACH -LW IPLI 4000

Listening on port 4000

Waiting for connection

sim> ATTACH -C IPLO 4000

Connection established	Connecting to IP address 127.0.0.1, port 4000

sim> ATTACH -LW IPLO 4001

Listening on port 4001

Waiting for connection

sim> ATTACH -C IPLI 4001

Connection established	Connecting to IP address 127.0.0.1, port 4001

Both forms of ATTACH take a modifier -W (wait); if specified, the command will wait up to 30 seconds for the connection process to complete.  ATTACH -C can specify both an IP address and a port, in the form aa.bb.cc.dd:port; if the IP address is omitted, it defaults to 127.0.0.1 (local system).  The connections may be forcibly broken with the SET IPLI DISCONNECT and SET IPLO DISCONNECT commands.

Both IPLI and IPLO implement the BOOT command.  BOOT IPLI or BOOT IBLO loads the HP 2000/Access Basic Block Loader for the IOP into memory and starts it running.

The interprocessor link supports the HP processor interconnect cable diagnostic with these commands:

SET IPLI DIAG			configure for diagnostic tests

SET IPLI LINK			configure for interprocessor connections

Enabling the diagnostic mode simulates the connection of the interprocessor cable from one card to the other within the same machine.  Configuring either IPLI or IPLO for diagnostic mode automatically configures the other card as well.

Each link device may be configured separately to send debugging information to the previously enabled debug output device using these commands:

SET IPLI DEBUG			provide input-side debug printouts

SET IPLI NODEBUG			inhibit input-side debug printouts

SET IPLO DEBUG			provide output-side debug printouts

SET IPLO NODEBUG			inhibit output-side debug printouts

Three debug flags that control the reporting level are supported:

CMDS					Command initiations and completions

CPU					Words received from and sent to the CPU

XFER					Data transferred via the link

Both IPLI and IPLO implement these registers:

name			size		comments

IBUF			16		input buffer

OBUF			16		output buffer

CTL			1		device/interrupt enable

FLG			1		device ready

FBF			1		device ready buffer

HOLD			8		holding buffer

TIME			24		polling interval for input

STOP_IOE		1		stop on I/O error

### 12620A/12936A Privileged Interrupt Fence (PIF)

The Privileged Interrupt Fence (PIF) was used in DOS and RTE systems to provide privileged interrupt capability.  The PIF was installed in the I/O backplane to separate privileged from unprivileged devices by controlling the interrupt priority chain signal (PRL) to lower-priority devices.  The privileged cards, located below the fence, were allowed to interrupt the operating system and service routines of the unprivileged cards that were located above the fence.  Privileged devices employed specially written device drivers that bypassed the operating system during interrupts.  This provided very fast interrupt service time.

HP produced two PIF cards: the 12620A Breadboard Interface for the RTE operating systems, and the 12936A Privileged Interrupt Fence Accessory for DOS.  They behaved quite differently and were not interchangeable.  Under simulation, the type of the PIF card may be selected and displayed with these commands:

SET PIF 12620A			simulate 12620A RTE fence

SET PIF 12936A			simulate 12936A DOS fence

SHOW PIF TYPE			display current fence type

The PIF may be configured to send debugging information to the previously enabled debug output device by using these commands:

SET PIF DEBUG			provide debug printouts

SET PIF NODEBUG			inhibit debug printouts

SHOW PIF DEBUG			display current debug state

Because the PIF is accessed twice for each unprivileged interrupt, systems employing the time base generator will generate a large number of debug statements very quickly.  In such cases, debug tracing should be enabled for the minimum time necessary.

The PIF implements these registers:

name			size		comments

CTL			 1		interface control

FLG			 1		interface flag

FBF			 1		interface flag buffer

## Disk Controllers

### 12557A Disk Controller (DPC, DPD) with Four 2781 Drives,
13210A Disk Controller (DPC, DPD) with Four 7900 Drives

The 12557A and 13210A disk controllers have two separate devices: a data channel (DPD) and a command channel (DPC).  The data channel includes a 128-word (one sector) buffer for reads and writes.  The command channel includes the four disk drives.  The controller can be configured as either a 12557A, supporting 2.5MB drives, or a 13210A, supporting 5MB drives, with the commands:

SET DPC 12557A			provide 2781 (2.5MB) drives

SET DPC 13210A			provide 7900 (5.0MB) drives

SHOW DPC TYPE			display current controller type

Drive types cannot be intermixed; the controller is configured for one type or the other.  The 13210A is selected by default.

Individual drives may be protected against writing.  These commands simulate the Upper/Lower Disc Protect switches on the drives:

SET DPCn LOCKED			set unit n write locked

SET DPCn WRITEENABLED		set unit n write enabled

Separate protection for the upper and lower platters of the 7900 drive is not supported.  Also, the drive Protect/Override switch is not supported; drive protection is permanently overridden.

Drives may also have their heads unloaded and loaded:

SET DPCn UNLOADED			unload heads on unit n

SET DPCn LOADED			load heads on unit n

This provides a convenient method of setting a drive "down" without detaching the associated disk image file.  Drives can also be set DISABLED or ENABLED.

The controller supports the BOOT command.  BOOT DPC copies the HP 12992F boot loader ROM into memory and starts it running.  BOOT -R DPC boots from the removable platter (head 0).  The switch register (S) is modified automatically to the value expected by the loader:

<15:14>	=	01

<13:12>	=	00

<11:6>	=	data channel select code

<5:3> 	= 	unchanged

<2:1> 	=	00

<0> 		=	1 if booting from the removable platter

The data channel (DPD) implements these registers:

name			size		comments

IBUF			16		input buffer

OBUF			16		output buffer

DBUF[0:127]		16		sector buffer

BPTR			7		sector buffer pointer

CMD			1		channel enable

CTL			1		interrupt enable

FLG			1		channel ready

FBF			1		channel ready buffer

XFER			1		transfer in progress flag

WVAL			1		write data valid flag

The command channel (DPC) implements these registers:

name			size		comments

OBUF			16		output buffer

BUSY			4		busy (unit #, + 1, of active unit)

CNT			5		check record count

CMD			1		controller enable

CTL			1		interrupt enable

FLG			1		controller ready

FBF			1		controller ready buffer

EOC			1		end of cylinder pending

POLL			1		attention polling enabled

RARC			8		record address register (cylinder)

RARH			2		record address register (head)

RARS			4		record address register (sector)

CYL[0:3]		8		current cylinder, drives 0-3

STA[0:3]		16		drive status, drives 0-3

CTIME			24		data transfer command delay time

DTIME			24		data channel command delay time

STIME			24		seek delay time, per cylinder

XTIME			24		inter-word transfer time

Error handling is as follows:

error	     		processed as

not attached	disk not ready (heads unloaded)

end of file		assume rest of disk is zero

OS I/O error	report error and stop

### 12565A Disk Controller (DQC, DQD) with Two 2883 Drives

The 12565A disk controller has two separate devices: a data channel (DQD) and a command channel (DQC).  The data channel includes a 128-word (one sector) buffer for reads and writes.  The command channel includes the two disk drives.

Individual drives may be protected against writing:

SET DQCn LOCKED			set unit n write locked

SET DQCn WRITEENABLED		set unit n write enabled

Drives may have their heads unloaded and loaded:

SET DQCn UNLOADED			unload heads on unit n

SET DQCn LOADED			load heads on unit n

This provides a convenient method of setting a drive "down" without detaching the associated disk image file.  Disk drives can also be set DISABLED or ENABLED.

The 12565A supports the BOOT command.  BOOT DQC copies the HP 12992A boot loader ROM into memory and starts it running.  The switch register (S) is modified automatically to the value expected by the loader:

<15:14>	=	01

<13:12>	=	10

<11:6>	=	data channel select code

<5:3> 	= 	unchanged

<2:0> 	=	000

The data channel (DQD) implements these registers:

name			size		comments

IBUF			16		input buffer

OBUF			16		output buffer

DBUF[0:127]		16		sector buffer

BPTR			7		sector buffer pointer

CMD			1		channel enable

CTL			1		interrupt enable

FLG			1		channel ready

FBF			1		channel ready buffer

XFER			1		transfer in progress flag

WVAL			1		write data valid flag

The command channel (DQC) implements these registers:

name			size		comments

OBUF			16		output buffer

BUSY			2		busy (unit # + 1 of active unit)

CNT			9		check record count

CMD			1		controller enable

CTL			1		interrupt enable

FLG			1		controller ready

FBF			1		controller ready buffer

RARC			8		record address register (cylinder)

RARH			5		record address register (head)

RARS			5		record address register (sector)

CYL[0:1]		8		current cylinder, drives 0-1

HED[0:1]		5		current head, drives 0-1

STA[0:1]		16		drive status, drives 0-1

CTIME			24		data transfer command delay time

DTIME			24		data channel command delay time

STIME			24		seek delay time, per cylinder

XTIME			24		inter-word transfer time

Error handling is as follows:

error	     		processed as

not attached	disk not ready (heads unloaded)

end of file		assume rest of disk is zero

OS I/O error	report error and stop

### 12606B Fixed Head Disk Controller (DRC, DRD) with 2770/2771 Disk,
12610B Drum Controller (DRC, DRD) with 2773/2774/2775 Drum

The 12606B and 12610B fixed head disk/drum controllers have two separate devices: a data channel (DRD) and a command channel (DRC).  The command channel includes the actual drive.  Ten models are supported:

command		interface and size	model

SET DRC 180K	12606B, 180K words	2770A

SET DRC 360K	12606B, 360K words	2771A

SET DRC 720K	12606B, 720K words	2771A-001

SET DRC 384K	12610B, 384K words	2773A

SET DRC 512K	12610B, 512K words	2773A-001

SET DRC 640K	12610B, 640K words	2773A-002

SET DRC 768K	12610B, 768K words	2774A

SET DRC 896K	12610B, 896K words	2774A-001

SET DRC 1024K	12610B, 1024K words	2774A-002

SET DRC 1536K	12610B, 1536K words	2775A

The command channel supports write-protected tracks.  Track protection is enabled with this command:

SET DRC PROTECTED

In addition, the number of protected tracks is specified by the command:

SET DRC TRACKPROT=count

The track protect count must be a power of two from 1 to 128 on the 12606 interface and from 1 to 512, or 768, on the 12610 interface.  If the drive has fewer tracks than the track protect count, then all tracks on the drive are eligible for protection.

Track protection is disabled with this command:

SET DRC UNPROTECTED

The controller supports the BOOT command.  BOOT DRD loads the first sector from the disk or drum into locations 0-77 octal and then begins execution at location 77.  This is very different from the boot loader protocol used by the 12565A and the 12557A/13210A.

The data channel (DRD) implements these registers:

name			size		comments

IBUF			16		input buffer

OBUF			16		output buffer

CTL			1		interrupt enable

FLG			1		channel ready

BPTR			6		sector buffer pointer

The command channel implements these registers:

name			size		comments

CW			16		command word

STA			16		status

RUN			1		run flip-flop

TIME			24		inter-word transfer time

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	     		processed as

not attached	drum or disk not ready

12606B/12610B data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

### 13037D Disk Controller (DS) with Eight 7905/7906/7920/7925 Drives

The 13037D disk controller supports up to eight 7905, 7906, 7920, or 7925 multiple-access controller (MAC) disk drives connected via a 13175D disk controller interface:

SET DSn 7905			drive n is a 15MB drive

SET DSn 7906			drive n is a 20MB drive

SET DSn 7920			drive n is a 50MB drive

SET DSn 7925			drive n is a 120MB drive

The drive type also may be set to one of the four supported types automatically, based on the size of the disk image file:

SET DSn AUTOSIZE			drive n type based on file size at ATTACH

Drive types may be intermixed.  The 7905 is selected by default.

Individual drives may be protected against writing.  These commands simulate the Disc Protect/Read Only switches on the drives:

SET DSn LOCKED			set unit n write locked

SET DSn WRITEENABLED		set unit n write enabled

Separate protection for the upper and lower platters of the 7905 and 7906 drives is not supported.  Protecting a 7905 or 7906 drive behaves as though both of the Disc Protect switches were on.

Drives may also have their heads unloaded and loaded:

SET DSn UNLOADED			unload heads on unit n

SET DSn LOADED			load heads on unit n

This provides a convenient method of setting a drive "down" without detaching the associated disk image file.

The setting of the drive Format switch may be changed with:

SET DSn FORMAT			set format enabled

SET DSn NOFORMAT			set format disabled

Drives may also be set DISABLED or ENABLED.

The controller supports the BOOT command.  BOOT DS copies the HP 12992B boot loader ROM into memory and starts it running.  The switch register (S) is modified automatically to the value expected by the loader:

<15:14>	=	11

<13:12>	=	01

<11:6>	=	interface select code

<5:3> 	= 	unchanged

<2> 		=	0

<1:0>		=	unchanged (head number)

The controller may be configured to send debugging information to the previously enabled debug output device:

SET DS DEBUG{=f1;f2;...}	provide debug printouts

SET DS NODEBUG{=f1;f2;...}	inhibit debug printouts

SHOW DS DEBUG			display current debug settings

Five debug flags that control various levels of reporting are supported:

CPU					Words received from and sent to the CPU

CMDS					Interface commands received from the CPU

BUF					Data read from and written to the card FIFO

RWCS					Disk read/write/control/status commands

SERV					Unit service scheduling calls

If individual flags are not specified in the SET DS DEBUG and SET DS NODEBUG commands, then all reporting levels are implied.

The controller implements these registers:

name			size		comments

CMFOL			 1		command follows flag

CMRDY			 1		command ready flag

FCNT			 5		FIFO fill count

FIFO[0:15]		16		data FIFO

STATE			 2		controller state

OPCODE		 6		current operation code

STATUS		 6		last operation status

EOC			 1		end of cylinder flag

EOD			 1		end of data flag

SPDU			16		S/P/D flags and last unit

FLMASK		 4		file mask

CYL			16		controller cylinder

HEAD			 6		controller head

SECTOR		 8		controller sector

VFYCNT		16		verify counter

LASPOL		 3		last unit polled

BUFFER[0:137]	16		sector buffer

INDEX			 8		buffer index

LENGTH		 8		buffer length

STIME			24		per-cylinder seek delay time

ITIME			24		intersector delay time

CTIME			24		command response time

DTIME			24		data transfer response time

WTIME			31		command wait time

CTL			 1		interface control

FLG			 1		interface flag

FBF			 1		interface flag buffer

SRQ			 1		interface DCPC service request

EDT			 1		end of data transfer flag

UCYL[0:7]		10		current cylinder, drives 0-7

UOP[0:7]		 6		operation code, drives 0-7

USTAT[0:7]		 8		drive status, drives 0-7

UPHASE[0:7]		 3		operation phase, drives 0-7

UPOS[0:7]		32		current byte position, drives 0-7

UWAIT[0:7]		32		current wait, drives 0-7

Error handling is as follows:

error	     		processed as

unit disabled	disk not connected to controller

not attached	disk not ready (heads unloaded)

SET DSn UNLOADED	disk not ready (heads unloaded)

end of file		assume rest of disk is zero

OS I/O error	report error and stop; continuing will report an

uncorrectable data error to the CPU

### 12821A Disk Interface (DA) with Four 7906H/7920H/7925H Drives

The 12821A disk interface supports up to four 7906H, 7920H, and 7925H integrated controller disk (ICD) drives connected via HP-IB:

SET DAn 7906H			unit n is a 20MB drive

SET DAn 7920H			unit n is a 50MB drive

SET DAn 7925H			unit n is a 120MB drive

Drive types may be intermixed.  The 7906H is selected by default.

The HP-IB address of each drive may be set:

SET DAn BUS=b			set unit n to HP-IB address b

Bus addresses range from 0-7, and each unit’s address must be unique.  Each unit’s bus address is initially set to its unit number.

Individual drives may be protected against writing.  These commands simulate the Disc Protect/Read Only switches on the drives:

SET DAn LOCKED			set unit n write locked

SET DAn WRITEENABLED		set unit n write enabled

Separate protection for the upper and lower platter of the 7906H drive is not supported.  Protecting a 7906H drive behaves as though both of the Disc Protect switches were on.

Drives may also have their heads unloaded and loaded:

SET DAn UNLOADED			unload heads on unit n

SET DAn LOADED			load heads on unit n

This provides a convenient method of setting a drive "down" without detaching the associated disk image file.

The setting of the drive Format switch may be changed with:

SET DAn FORMAT			set format enabled

SET DAn NOFORMAT			set format disabled

Drives may also be set DISABLED or ENABLED.  A disabled drive is disconnected from the bus.

A diagnostic mode is provided for use with the 12821-16001 ICD Disc Interface diagnostic:

SET DA DIAG				configure cable for diagnostic tests

SET DA HPIB				configure cable for normal bus operation

SHOW DA CABLE			show cable configuration

SET DA ADDRESS=b			set card to HP-IB address b

Setting the diagnostic mode enables a second 12821A device (DC) and simulates the installation of an HP-IB cable between the two 12821A Disk Interface cards.  Setting the HP-IB mode disables the second device and simulates the installation of an HP-IB cable between the first card and the drives.  The bus address used by the diagnostic to test the card’s parallel poll response also may be set.

The 12821A supports the BOOT command.  BOOT DA copies the HP 12992H Boot Loader ROM into memory and starts it running.  The switch register (S) is modified automatically to the value expected by the loader:

<15:14>	=	11

<13:12>	=	01

<11:6>	=	interface select code

<5:3> 	= 	unchanged

<2> 		=	0

<1:0>		=	unchanged (head number)

The 12821A may be configured to send debugging information to the previously enabled debug output device by using these commands:

SET DA DEBUG{=f1;f2;...}	provide debug printouts

SET DA NODEBUG{=f1;f2;...}	inhibit debug printouts

SHOW DA DEBUG			display current debug settings

Six debug flags that control various levels of reporting are supported:

CPU					Words received from and sent to the CPU

CMDS					Interface commands received from the CPU

BUF					Data read from and written to the card FIFO

XFER					Data received and transmitted via HP-IB

RWCS					Disk read/write/control/status commands

SERV					Unit service scheduling calls

If individual flags are not specified in the SET DA DEBUG and SET DA NODEBUG commands, then all reporting levels are implied.

The DA interface and drive controllers implement these registers:

name			size		comments

CWR			16		control word register

SWR			16		status word register

IDR			16		input data register

FCNT			 5		FIFO fill count

FIFO[0:15]		20		data FIFO

ACPT			 4		bus acceptor units bitmap

LSTN			 4		bus listener units bitmap

TALK			 4		bus talker unit bitmap

PPR			 8		parallel poll response bitmap

BUSCTL		 8		HP-IB control lines

CTL			 1		interface control

FLG			 1		interface flag

FBF			 1		interface flag buffer

SRQ			 1		interface DCPC service request

EDT			 1		end of data transfer flag

EOR			 1		end of record flag

BUFFER[0:137]	16		sector buffer

DSJ[0:3]		 2		controller DSJ, drives 0-3

ISTATE[0:3]		 4		interface state, drives 0-3

ICMD[0:3]		 4		interface command, drives 0-3

Error handling is as follows:

error	     		processed as

unit disabled	disk not connected to bus

not attached	disk not connected to bus

SET DSn UNLOADED	disk not ready (heads unloaded)

end of file		assume rest of disk is zero

OS I/O error	report error and stop; continuing will report an

uncorrectable data error to the CPU

## Magnetic Tape

### 12559C Magnetic Tape Controller (MTC, MTD) with One 3030 Drive

The 12559C magnetic tape drive controller has two separate devices, a data channel (MTD) and a command channel (MTC).  The data channel includes a maximum record sized buffer for reads and writes.  The command channel includes the tape unit and supports setting the tape image file format.  Magnetic tape options include the ability to make the unit write enabled or write locked:

SET MTC LOCKED			set unit write locked

SET MTC WRITEENABLED		set unit write enabled

The BOOT command is not supported.  The 12559C is disabled by default.

The data channel (MTD) implements these registers:

name			size		comments

FLG			1		channel ready

FBF			1		channel ready buffer

DBUF[0:65535]	8		record transfer buffer

BPTR			16		buffer pointer (reads and writes)

BMAX			16		buffer size (writes)

The command channel (MTC) implements these registers:

name			size		comments

FNC			8		current function

STA			9		tape status

BUF			8		buffer

CTL			1		interrupt enabled

FLG			1		controller ready

FBF			1		controller ready buffer

DTF			1		data transfer flop

FSVC			1		first service flop

POS			32		tape position

CTIME			24		command delay time

GTIME			24		inter-record gap traversal time

XTIME			24		inter-word transfer delay time

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		parity error

OS I/O error	parity error; if STOP_IOE, stop

### 13181A Magnetic Tape Controller (MSC, MSD) with Four 7970B Drives,
13183A Magnetic Tape Controller (MSC, MSD) with Four 7970E Drives

The 13181A/13183A magnetic tape drive controller has two separate devices, a data channel (MTD) and a command channel (MTC).  The data channel includes a maximum record sized buffer for reads and writes.  The command channel includes the tape units and supports setting the tape image file format.  Magnetic tape options include the ability to select the controller type and timing, drive tape reel size, and the ability to set drives offline, online, write enabled, or write locked:

SET MSC 13181A			provide 7970B (800 BPI) drives

SET MSC 13183A			provide 7970E (1600 BPI) drives

SHOW MSC TYPE			display current controller type

SET MSC REALTIME			set controller to actual timing

SET MSC FASTTIME			set controller to optimized timing (default)

SET MSCn OFFLINE			set unit n offline

SET MSCn ONLINE			set unit n online

SET MSCn LOCKED			set unit n write locked

SET MSCn WRITEENABLED		set unit n write enabled

SET MSCn REEL=length		set unit tape reel size

0 = unlimited (default)

600 = 600 feet

1200 = 1200 feet

2400 = 2400 feet

SET MSCn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW MSCn REEL			display current reel size or capacity

SHOW MSCn CAPAC			display current reel size or capacity

Setting the controller to optimized timing with SET MSC FASTTIME also enables two other optimizations: the initial gap normally written after BOT is omitted, and the Gap and File Mark (GFM) command is executed as a Write File Mark (WFM) command instead.  Note that this does not affect the Write Gap (GAP) command.

MSC may be configured to send debugging information to the previously enabled debug output device using these commands:

SET MSC DEBUG			provide debug printouts

SET MSC NODEBUG			inhibit debug printouts

Three debug flags that control the reporting level are supported:

CMDS					Command initiations and completions

CPU					Words received from and sent to the CPU

RWS					Tape reads, writes, and status returns

The 13181A/13183A supports the BOOT command.  BOOT MSC loads the HP 12992D boot loader ROM into memory and starts it running.  BOOT –S MSC causes the loader to position to the file number specified in the A register before starting to load data.  The switch register (S) is modified automatically to the value expected by the IBL loader:

<15:14>	=	10

<13:12>	=	00

<11:6>	=	data channel select code

<5:3> 	= 	unchanged

<2:0> 	=	00

<0> 		=	1 if the tape is to be positioned before loading

The data channel (MSD) implements these registers:

name			size		comments

BUF			16		data buffer

CTL			1		interrupt enabled

FLG			1		channel ready

FBF			1		channel ready buffer

DBUF[0:65535]	8		record transfer buffer

BPTR			17		buffer pointer (reads and writes)

BMAX			17		buffer size (writes)

The command channel (MSC) implements these registers:

name			size		comments

STA			12		tape status

BUF			16		buffer

USEL			2		currently selected unit

FSVC			1		first service flip-flop

CTL			1		interrupt enabled

FLG			1		controller ready

FBF			1		controller ready buffer

POS[0:3]		32		tape position

BTIME			24		BOT start delay time

CTIME			24		command delay time

GTIME			24		gap traversal time

ITIME			24		IRG traversal time

RTIME			24		rewind initiation time

XTIME			24		inter-word transfer delay time

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		parity error

OS I/O error	parity error; if STOP_IOE, stop

# Symbolic Display and Input

The HP2100 simulator implements symbolic display and input.  These command line switches control the display:

-a			display as ASCII character

-c			display as two packed ASCII characters

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		ASCII character

" or -c		two packed ASCII characters

alphabetic		instruction mnemonic

numeric		octal number

Instruction input uses standard HP 2100 assembler syntax.  There are seven instruction classes: memory reference, I/O group, shift/rotate, alter/skip, extended shift, extended memory reference, extended two-address reference.

Memory reference instructions have the format:

memref {C/Z} address{,I}

where I signifies indirect addressing, C a current-page reference, and Z a zero-page reference.  The address is an octal number in the range 0 - 77777; if C or Z is specified, the address is a page offset in the range 0 - 1777.

C/Z is not needed when entering instructions into CPU memory; the simulator figures out from the target address what mode to use.  However, when entering instructions into device address spaces (e.g., disks), the eventual target address is unknown, and C must be used to specify current-page addressing (omitting C/Z implies zero-page addressing).

IOG instructions have the format:

io device{,C}

where C signifies that the device flag is to be cleared.  The device is an octal number in the range 0 - 77.

Shift/rotate and alter/skip instructions have the format:

sub-op,sub-op,sub-op...

The simulator checks that the combination of sub-opcodes is legal.

Extended shift instructions have the format:

extshift count

where count is a decimal number in the range 1 - 16.

Extended memory reference instructions have the format:

extmemref address{,I}

where I signifies indirect addressing and address is an octal number in the range 0 - 77777.

Extended two-address instructions have the format:

ext2addr addr1{,I},addr2{,I}

where I signifies indirect addressing.  Both addr1 and addr2 are octal numbers in the range 0 - 77777.