VAX-11/780 Simulator Usage

31-MAR-2011

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2011, written by Robert M Supnik

Copyright (c) 1993-2011, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	VAX780 Features	4

2.1	CPU and System Devices	5

2.1.1	CPU	5

2.1.2	Translation Buffer (TLB)	7

2.1.3	SBI Controller (SBI)	7

2.1.4	Memory Controllers (MCTL0, MCTL1)	7

2.1.5	Time-Of-Day Clock (TODR)	8

2.1.6	Interval Timer (TMR)	8

2.1.7	Unibus Adapter (UBA)	8

2.1.8	Massbus Adapters (MBA0, MBA1)	9

2.2	I/O Device Addressing	9

2.3	Programmed I/O Devices	10

2.3.1	Console Input (TTI)	10

2.3.2	Console Output (TTO)	11

2.3.3	RX01 Console Floppy Disk (RX)	11

2.3.4	Line Printer (LPT)	12

2.4	Disks	12

2.4.1	RP04/05/06/07, RM02/03/05/80 Disk Pack Drives (RP)	12

2.4.2	RL11/RL01/RL02 Cartridge Disk (RL)	13

2.4.3	RK611/RK06/RK07 Cartridge Disk (HK)	14

2.4.4	UDA50 MSCP Disk Controllers (RQ, RQB, RQC, RQD)	15

2.5	Tapes	17

2.5.1	TM03/TE16/TU45/TU77 Magnetic Tapes (TU)	17

2.5.2	TS11 Magnetic Tape (TS)	18

2.5.3	TUK50 TMSCP Disk Controller (TQ)	19

2.6	Communications Devices	20

2.6.1	DZ11 Terminal Multiplexer (DZ)	20

2.7	CR11 Card Reader (CR)	21

3	Symbolic Display and Input	23

This memorandum documents the DEC VAX-11/780 simulator.

# Simulator Files

To compile the VAX-11/780, you must define VM_VAX, VAX780, and USE_INT64 as part of the compilation command line.  To enable extended file support (files greater than 2GB), you must define USE_ADDR64 as part of the command line as well.

sim/		scp.h

sim_console.h

sim_defs.h

sim_ether.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_ether.c

sim_fio.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/vax/	vax_defs.h

vax780_defs.h

vax_cis.c

vax_cmode.c

vax_cpu.c

vax_cpu1.c

vax_fpa.c

vax_mmu.c

vax_octa.c

vax_sys.c

vax_syscm.c

vax780_mba.c

vax780_mem.c

vax780_sbi.c

vax780_stddev.c

vax780_syslist.c

vax780_uba.c

sim/pdp11/	pdp11_cr_dat.h

pdp11_mscp.h

pdp11_uqssp.h

pdp11_xu.h

pdp11_cr.c

pdp11_dz.c

pdp11_hk.c

pdp11_lp.c

pdp11_rl.c

pdp11_rp.c

pdp11_rq.c

pdp11_ry.c

pdp11_tq.c

pdp11_ts.c

pdp11_tu.c

pdp11_xu.c

Additional files are:

sim/vax/	vmb.exe		standard boot code

# VAX780 Features

The VAX780 simulator is configured as follows:

device name(s)	simulates

CPU			VAX-11/780 CPU

TLB			translation buffer

SBI			system bus controller

MCTL0,MTCL1		memory controllers, MS780C with 4MB memory each,

or MS780E with 8MB-64MB each

UBA			DW780 Unibus adapter

MBA0,MBA1		RH780 Massbus adapters

TODR			time-of-day clock

TMR			interval timer

TTI,TTO		console terminal

RX			console RX01 floppy disk

DZ			DZ11 8-line terminal multiplexer (up to 4)

CR			CR11 card reader

LPT			LP11 line printer

RP			RP04/05/06/07, RM02/03/05/80 Massbus disks, up to

eight

HK			RK611/RK06(7) cartridge disk controller with eight

drives

RL			RL11/RL01(2) cartridge disk controller with four drives

RQ			UDA50 MSCP controller with four drives

RQB			second UDA50 MSCP controller with four drives

RQC			third UDA50 MSCP controller with four drives

RQD			fourth UDA50 MSCP controller with four drives

RY			RX211 floppy disk controller with two drives

TS			TS11 magnetic tape controller with one drive

TQ			TUK50 TMSCP magnetic tape controller with four drives

TU			TM03 tape formatter with eight TE16/TU45/TU77 drives

XU			DEUNA/DELUA Ethernet controller

XUB			second DEUNA/DELUA Ethernet controller

The DZ, LPT, RP, RL, RQ, RQB, RQC, RQD, RY, TS, TQ, TU, XU, and XUB devices can be set DISABLED.  RQB, RQC, RQD, VH, XU, and XUB are disabled by default.

The VAX780 simulator implements several unique stop conditions:

- Change mode to interrupt stack

- Illegal vector (bits<1:0> = 2 or 3)

- Unexpected exception during interrupt or exception

- Process PTE in P0 or P1 space instead of system space

- Unknown IPL

- Infinite loop (BRB/W to self at IPL 1F)

The LOAD command supports a simple binary format, consisting of a stream of binary bytes without origin or checksum, for loading memory.  The DUMP command is not implemented.

## CPU and System Devices

### CPU

CPU options include the size of main memory and the treatment of the HALT instruction.

SET CPU 8M				set memory size = 8MB

SET CPU 16M				set memory size = 16MB

SET CPU 32M				set memory size = 32MB

SET CPU 48M				set memory size = 48MB

SET CPU 64M				set memory size = 64MB

SET CPU 128M			set memory size = 128MB

The CPU implements a show command to display the I/O address map:

SHOW CPU IOSPACE			show I/O space address map

The CPU also implements a command to display a virtual to physical address translation:

SHOW {-kesu} CPU VIRTUAL=n	show translation for address n

in kernel/exec/supervisor/user mode

The CPU also implements the console AUTORESTART switch:

SET CPU CONHALT			enable AUTORESTART

SET CPU SIMHALT			disable AUTORESTART

Notes on memory size:

- The first version of the VAX-11/780 used MS780C controllers, which supported 1-4MB of memory per controller.  This is the only memory controller recognized by VMS V1.  MS780E controllers supported 4MB-64MB per controller.

- The controller type is set automatically based on memory size.

Initial memory size is 8MB.

Memory can be loaded with a binary byte stream using the LOAD command.  The LOAD command recognizes three switches:

-o			origin argument follows file name

-r			load ROM in memory controller 0

-s			load ROM in memory controller 1

These switches are recognized when examining or depositing in CPU memory:

-b			examine/deposit bytes

-w			examine/deposit words

-l			examine/deposit longwords

-d			data radix is decimal

-o			data radix is octal

-h			data radix is hexadecimal

-m			examine (only) VAX instructions

-p			examine/deposit PDP-11 (compatibility mode) instructions

-r			examine (only) RADIX50 encoded data

-v			interpret address as virtual, current mode

-k			interpret address as virtual, kernel mode

-e			interpret address as virtual, executive mode

-s			interpret address as virtual, supervisor mode

-u			interpret address as virtual, user mode

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

PC			32		program counter

R0 .. R14		32		R0 to R14

AP			32		alias for R12

FP			32		alias for R13

SP			32		alias for R14

PSL			32		processor status longword

CC			4		condition codes, PSL<3:0>

KSP			32		kernel stack pointer

ESP			32		executive stack pointer

SSP			32		supervisor stack pointer

USP			32		user stack pointer

IS			32		interrupt stack pointer

SCBB			32		system control block base

PCBB			32		process controll block base

P0BR			32		P0 base register

P0LR			22		P0 length register

P1BR			32		P1 base register

P1LR			22		P1 length register

SBR			32		system base register

SLR			22		system length register

SISR			16		software interrupt summary register

ASTLVL		4		AST level register

MAPEN			1		memory management enable

PME			1		performance monitor enable

TRPIRQ		8		trap/interrupt pending

CRDERR		1		correctible read data error flag

MEMERR		1		memory error flag

PCQ[0:63]		32		PC prior to last PC change or interrupt;

most recent PC change first

WRU			8		interrupt character

The CPU attempts to detect when the simulator is idle.  When idle, the simulator does not use any resources on the host system.  Idle detection is controlled by the SET IDLE and SET NOIDLE commands:

SET CPU IDLE{=VMS|ULTRIX|NETBSD|FREEBSD|32V|ALL}	enable idle detection

SET CPU NOIDLE			disable idle detection

Idle detection is disabled by default.  Unless ALL is specified, idle detection is operating system specific.  If idle detection is enabled with an incorrect operating system setting, simulator performance could be impacted.  The default operating system setting is VMS.

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU HISTORY=n			enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The maximum length for the history is 65536 entries.

### Translation Buffer (TLB)

The translation buffer consists of two units, representing the system and user translation buffers, respectively.  It has no registers.  Each translation buffer entry consists of two 32b words, as follows:

word n		tag

word n+1		cached PTE

An invalid entry is indicated by a tag of 0xFFFFFFFF.

### SBI Controller (SBI)

The SBI is the VAX-11/780 system bus.  The simulated SBI implements these registers:

name			size		comments

NREQ14		16		Nexus IPL14 interrupt requests

NREQ15		16		Nexus IPL15 interrupt requests

NREQ16		16		Nexus IPL16 interrupt requests

NREQ17		16		Nexus IPL17 interrupt requests

WCSA			16		writeable control store address

WCSD			32		writeable control store data

MBRK			13		microbreak register

SBIFS			32		SBI fault status

SBISC			32		SBI silo compare

SBIMT			32		SBI maintenance register

SBIER			32		SBI error status

SBITMO		32		SBI timeout address

### Memory Controllers (MCTL0, MCTL1)

The memory controllers implement the registers for the MS780C (8MB memory) or MS780E (16MB or greater memory).  Each controller implements these registers:

name			size		comments

CRA			32		control register A

CRB			32		control register B

CRC			32		control register C

CRD			32		control register D (MS780E only)

ROM[0:1023]		32		bootstrap ROM

ROM can be loaded from a file with the commands

LOAD -R <file>			load MCTL0 ROM

LOAD -S <file>			load MCTL1 ROM

### Time-Of-Day Clock (TODR)

The TODR tracks time since an arbitrary start in 1 microsecond intervals.  It has these registers:

name			size		comments

TODR			32		time-of-day register

TIME			24		delay between ticks

The TODR register autocalibrates against real-world time.

### Interval Timer (TMR)

The interval timer implements the VAX architectural timer, with 1 microsecond intervals.  It has these registers:

name			size		comments

ICCS			32		interval timer control and status

ICR			32		interval count register

NICR			32		next interval count register

INT			1		interrupt request

For standard VMS intervals (10 milliseconds), the interval timer autocalibrates against real-world time.

### Unibus Adapter (UBA)

The Unibus adapter (UBA) simulates the DW780.  It recognizes these options:

SET UBA AUTOCONFIGURE		enable autoconfiguration

SET UBA NOAUTOCONFIGURE		disable autoconfiguration

and this SHOW command:

SHOW UBA IOSPACE			display IO address space assignments

The UBA also implements a command to display a Unibus address to physical address translation:

SHOW UBA VIRTUAL=n		show translation for Unibus address n

Finally, the UBA implements main memory examination and modification via the Unibus map.  The data width is always 16b:

EX UBA 0/10				examine main memory words corresponding

to Unibus addresses 0-10

The UBA has these registers:

name			size		comments

IPL14			32		Unibus IPL14 interrupt requests

IPL15			32		Unibus IPL15 interrupt requests

IPL16			32		Unibus IPL16 interrupt requests

IPL17			32		Unibus IPL17 interrupt requests

CNFR			32		configuration register

CR			32		control register

SR			32		status register

DR			32		diagnostic register

INT			1		internal UBA interrupt request

NEXINT		1		UBA Nexus interrupt request

AIIP			1		adapter initialization in progress flag

UIIP			1		Unibus initialization in progress flag

FMER			32		failing memory address

FUBAR			32		failing UBA map register

BRSVR0		32		spare register 0

BRSVR1		32		spare register 1

BRSVR2		32		spare register 2

BRSVR3		32		spare register 3

BRRVR4		32		vector register, IPL 14

BRRVR5		32		vector register, IPL 15

BRRVR6		32		vector register, IPL 16

BRRVR7		32		vector register, IPL 17

DPR[0:15]		32		data path registers 0 to 15

MAP[0:495]		32		map registers 0 to 495

AITIME		24		adapter initialization time

UITIME		24		Unibus initialization time

### Massbus Adapters (MBA0, MBA1)

The Massbus adapters (MBA0, MBA1) simulate RH780's.  MBA0 is assigned to the RP disk drives, MBA1 to the TU tape drives.  Each MBA has these registers:

name			size		comments

CNFR			32		configuration register

CR			32		control register

SR			32		status register

VA			17		virtual address register

BC			32		byte count register

DR			32		diagnostic register

SMR			32		selected map register

MAP[0:255]		32		map registers

NEXINT		1		MBA Nexus interrupt request

## I/O Device Addressing

Unibus I/O space is not large enough to allow all possible devices to be configured simultaneously at fixed addresses.  Instead, many devices have floating addresses; that is, the assigned device address depends on the presence of other devices in the configuration:

DZ11			all instances have floating addresses

RL11			first instance has fixed address, rest floating

RX11/RX211		first instance has fixed address, rest floating

DEUNA/DELUA		first instance has fixed address, rest floating

MSCP disk		first instance has fixed address, rest floating

TMSCP tape		first instance has fixed address, rest floating

To maintain addressing consistency as the configuration changes, the simulator implements DEC's standard I/O address and vector autoconfiguration algorithms for devices DZ, RL, RY, XU, RQ, and TQ.  This allows the user to enable or disable devices without needing to manage I/O addresses and vectors.

Autoconfiguration cannot solve address conflicts between devices with overlapping fixed addresses.  For example, with default I/O page addressing, the PDP-11 can support either a TUK50 or a TS11, but not both, since they use the same I/O addresses.

In addition to autoconfiguration, most devices support the SET <device> ADDRESS command, which allows the I/O page address of the device to be changed, and the SET <device> VECTOR command, which allows the vector of the device to be changed.  Explicitly setting the I/O address of a device that normally uses autoconfiguration DISABLES autoconfiguration for that device and for the entire system.  As a consequence, the user may have to manually configure all other autoconfigured devices, because the autoconfiguration algorithm no longer recognizes the explicitly configured device.  A device can be reset to autoconfigure with the SET <device> AUTOCONFIGURE command.  Autoconfiguration can be restored for the entire system with the SET CPU AUTOCONFIGURE command.

The current I/O map can be displayed with the SHOW CPU IOSPACE command. Addresses that have set by autoconfiguration are marked with an asterisk (*).

All devices support the SHOW <device> ADDRESS and SHOW <device> VECTOR commands, which display the device address and vector, respectively.

## Programmed I/O Devices

### Console Input (TTI)

The terminal interfaces (TTI, TTO) can be set to one of three modes, 7P, 7B or 8B:

mode	input characters		output characters

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is 8B.

When the console terminal is attached to a Telnet session, it recognizes BREAK.  If BREAK is entered, and BDR<7> is set, control returns to the console firmware; otherwise, BREAK is treated as a normal terminal input condition.

The terminal input (TTI) polls the console keyboard for input.  It implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		number of characters input

TIME			24		input polling interval (if 0, the keyboard

is polled synchronously with the TODR)

### Console Output (TTO)

The terminal output (TTO) writes to the simulator console window.  It implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		number of characters input

TIME			24		time from I/O initiation to interrupt

### RX01 Console Floppy Disk (RX)

RX01 options include the ability to set units write enabled or write locked:

SET RXn LOCKED			set unit n write locked

SET RXn WRITEENABLED		set unit n write enabled

The RX01 implements a special command, FLOAD, for loading VAX executables from an RT11-formatted console floppy disk image:

FLOAD <file_name> {<origin>}

FLOAD searches the floppy disk image attached to the RX01 for the named file and then loads it into VAX-11/780 memory starting at the origin.  If no origin is specified, the default origin is 200 (hex).

The RX01 implements these registers:

name			size		comments

FNC			8		function select

ES			8		error status

ECODE			8		error code

TA			8		track address

SA			8		sector address

STATE			4		protocol state

BPTR			7		data buffer pointer

CTIME			24		command initiation delay

STIME			24		seek time delay, per track

XTIME			24		transfer time delay, per byte

STOP_IOE		1		stop on I/O error

DBUF[0:127]		8		data buffer

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

RX01 data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

### Line Printer (LPT)

The line printer (LPT) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

OS I/O error	x		report error and stop

## Disks

All VAX-11/780 disks, and the TUK50 MSCP tape, support a special form of the boot command, with the following syntax:

BOOT <unit>{/R5:<value>}

For example,

BOOT RP0/R5:1

The optional switch, /R5, specifies that R5 is to be loaded with the specified value prior to booting.  If the switch is omitted, R5 is loaded with 0.

### RP04/05/06/07, RM02/03/05/80 Disk Pack Drives (RP)

The RP controller implements the Massbus family of large disk drives.  RP options include the ability to set units write enabled or write locked, to set the drive type to one of six disk types, or autosize, and to write a DEC standard 044 compliant bad block table on the last track:

SET RPn LOCKED			set unit n write locked

SET RPn WRITEENABLED		set unit n write enabled

SET RPn RM03			set type to RM03

SET RPn RM05			set type to RM05

SET RPn RM80			set type to RM80

SET RPn RP04			set type to RP04

SET RPn RP06			set type to RP06

SET RPn RP07			set type to RP07

SET RPn AUTOSIZE			set type based on file size at attach

SET RPn BADBLOCK			write bad block table on last track

The type options can be used only when a unit is not attached to a file.  The bad block option can be used only when a unit is attached to a file.  Units can be set ENABLED or DISABLED.  The RP controller supports the BOOT command.

The RP controller implements the registers listed below.  Registers suffixed with [0:7] are replicated per drive.

name			size		comments

CS1[0:7]		16		current operation

DA[0:7]		16		desired surface, sector

DS[0:7]		16		drive status

ER1[0:7]		16		drive errors

OF[0:7]		16		offset

DC[0:7]		16		desired cylinder

ER2[0:7]		16		error status 2

ER3[0:7]		16		error status 3

EC1[0:7]		16		ECC syndrome 1

EC2[0:7]		16		ECC syndrome 2

MR[0:7]		16		maintenance register

MR2[0:7]		16		maintenance register 2 (RM only)

HR[0:7]		16		holding register (RM only)

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

### RL11/RL01/RL02 Cartridge Disk (RL)

RL11 options include the ability to set units write enabled or write locked, to set the drive type to RL01, RL02, or autosize, and to write a DEC standard 044 compliant bad block table on the last track:

SET RLn LOCKED			set unit n write locked

SET RLn WRITEENABLED		set unit n write enabled

SET RLn RL01			set type to RL01

SET RLn RL02			set type to RL02

SET RLn AUTOSIZE			set type based on file size at attach

SET RLn BADBLOCK			write bad block table on last track

The type options can be used only when a unit is not attached to a file.  The bad block option can be used only when a unit is attached to a file.  Units can be set ENABLED or DISABLED.  The RL11 supports the BOOT command.

The RL11 implements these registers:

name			size		comments

RLCS			16		control/status

RLDA			16		disk address

RLBA			16		memory address

RLBAE			6		memory address extension (RLV12)

RLMP,RLMP1,RLMP2	16		multipurpose register queue

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

### RK611/RK06/RK07 Cartridge Disk (HK)

RK611 options include the ability to set units write enabled or write locked, to set the drive type to RK06, RK07, or autosize, and to write a DEC standard 044 compliant bad block table on the last track:

SET HKn LOCKED			set unit n write locked

SET HKn WRITEENABLED		set unit n write enabled

SET HKn RK06			set type to RK06

SET HKn RK07			set type to RK07

SET HKn AUTOSIZE			set type based on file size at attach

SET HKn BADBLOCK			write bad block table on last track

The type options can be used only when a unit is not attached to a file.  The bad block option can be used only when a unit is attached to a file.  Units can be set ENABLED or DISABLED.  The RK611 supports the BOOT command.

The RK611 implements these registers:

name			size		comments

HKCS1			16		control/status 1

HKWC			16		word count

HKBA			16		bus address

HKDA			16		desired surface, sector

HKCS2			16		control/status 2

HKDS[0:7]		16		drive status, drives 0 to 7

HKER[0:7]		16		drive errors, drives 0 to 7

HKDB[0:2]		16		data buffer silo

HKDC			16		desired cylinder

HKOF			8		offset

HKMR			16		maintenance register

HKSPR			16		spare register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR1<7>)

IE			1		interrupt enable flag (CSR1<6>)

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

### UDA50 MSCP Disk Controllers (RQ, RQB, RQC, RQD)

The simulator implements four MSCP disk controllers, RQ, RQB, RQC, RQD.  Initially, RQB, RQC, and RQD are disabled.  Each RQ controller simulates an UDA50 MSCP disk controller with four drives.  RQ options include the ability to set units write enabled or write locked, and to set the drive type to one of many disk types:

SET RQn LOCKED			set unit n write locked

SET RQn WRITEENABLED		set unit n write enabled

SET RQn RX50			set type to RX50

SET RQn RX33			set type to RX33

SET RQn RD51			set type to RD51

SET RQn RD52			set type to RD52

SET RQn RD53			set type to RD53

SET RQn RD54			set type to RD54

SET RQn RD31			set type to RD31

SET RQn RA81			set type to RA81

SET RQn RA82			set type to RA82

set RQn RA71			set type to RA71

SET RQn RA72			set type to RA72

SET RQn RA90			set type to RA90

SET RQn RA92			set type to RA92

SET RQn RRD40			set type to RRD40 (CD ROM)

SET RQn RAUSER{=n}		set type to RA82 with n MB's

SET -L RQn RAUSER{=n}		set type to RA82 with n LBN's

The type options can be used only when a unit is not attached to a file.  RAUSER is a "user specified" disk; the user can specify the size of the disk in either MB (1000000 bytes) or logical block numbers (LBN's, 512 bytes each).  The minimum size is 5MB; the maximum size is 2GB without extended file support, 1TB with extended file support.

Units can be set ENABLED or DISABLED.  The RQ controllers support the BOOT command.

Each RQ controller implements the following special SHOW commands:

SHOW RQn TYPE			show drive type

SHOW RQ RINGS			show command and response rings

SHOW RQ FREEQ			show packet free queue

SHOW RQ RESPQ			show packet response queue

SHOW RQ UNITQ			show unit queues

SHOW RQ ALL				show all ring and queue state

SHOW RQn UNITQ			show unit queues for unit n

Each RQ controller implements these registers:

name			size		comments

SA			16		status/address register

S1DAT			16		step 1 init host data

CQBA			22		command queue base address

CQLNT			8		command queue length

CQIDX			8		command queue index

RQBA			22		request queue base address

RQLNT			8		request queue length

RQIDX			8		request queue index

FREE			5		head of free packet list

RESP			5		head of response packet list

PBSY			5		number of busy packets

CFLGS			16		controller flags

CSTA			4		controller state

PERR			9		port error number

CRED			5		host credits

HAT			17		host available timer

HTMO			17		host timeout value

CPKT[0:3]		5		current packet, units 0 to 3

PKTQ[0:3]		5		packet queue, units 0 to 3

UFLG[0:3]		16		unit flags, units 0 to 3

INT			1		interrupt request

ITIME			1		response time for initialization steps

(except for step 4)

QTIME			24		response time for 'immediate' packets

XTIME			24		response time for data transfers

PKTS[33*32]		16		packet buffers, 33W each, 32 entries

While VMS is not timing sensitive, most of the BSD-derived operating systems (NetBSD, OpenBSD, etc) are.  The QTIME and XTIME parameters are set to values that allow these operating systems to run correctly.

Error handling is as follows:

error	     		processed as

not attached	disk not ready

end of file		assume rest of disk is zero

OS I/O error	report error and stop

## Tapes

### TM03/TE16/TU45/TU77 Magnetic Tapes (TU)

The TU controller implements the Massbus family of 800/1600bpi magnetic tape drives.  TU options include the ability to set the drive type to one of three drives (TE16, TU45, or TU77), and to set the drives write enabled or write locked.

SET TUn TE16			set unit n drive type to TE16

SET TUn TU45			set unit n drive type to TU45

SET TUn TU77			set unit n drive type to TU77

SET Tun LOCKED			set unit n write locked

SET Tun WRITEENABLED		set unit n write enabled

Magnetic tape units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TUn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW TUn CAPAC			show unit n capacity in MB

Units can be set ENABLED or DISABLED.  The TU controller does not support the BOOT command.

The TU controller implements the following registers:

name			size		comments

CS1			6		current operation

FC			16		frame count

FS			16		formatter status

ER			16		formatter errors

CC			16		check character

MR			16		maintenance register

TC			16		tape control register

TIME			24		operation execution time

UST			17		unit status, drives 0 to 7

POS			32		position, drive 0 to 7

STOP_IOE		1		stop of I/O error

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		bad tape

OS I/O error	parity error; if STOP_IOE, stop

### TS11 Magnetic Tape (TS)

TS options include the ability to make the unit write enabled or write locked.

SET TS LOCKED			set unit write locked

SET TS WRITEENABLED		set unit write enabled

The TS drive can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TS0 CAPAC=m			set capacity to m MB (0 = unlimited)

SHOW TS0 CAPAC			show capacity in MB

The TS11 does not support the BOOT command.

The TS controller implements these registers:

name			size		comments

TSSR			16		status register

TSBA			16		bus address register

TSDBX			16		data buffer extension register

CHDR			16		command packet header

CADL			16		command packet low address or count

CADH			16		command packet high address

CLNT			16		command packet length

MHDR			16		message packet header

MRFC			16		message packet residual frame count

MXS0			16		message packet extended status 0

MXS1			16		message packet extended status 1

MXS2			16		message packet extended status 2

MXS3			16		message packet extended status 3

MXS4			16		message packet extended status 4

WADL			16		write char packet low address

WADH			16		write char packet high address

WLNT			16		write char packet length

WOPT			16		write char packet options

WXOPT			16		write char packet extended options

ATTN			1		attention message pending

BOOT			1		boot request pending

OWNC			1		if set, tape owns command buffer

OWNM			1		if set, tape owns message buffer

TIME			24		delay

POS			32		position

Error handling is as follows:

error			processed as

not attached	tape not ready

end of file		bad tape

OS I/O error	fatal tape error

### TUK50 TMSCP Disk Controller (TQ)

The TQ controller simulates the TUK50 TMSCP disk controller.  TQ options include the ability to set units write enabled or write locked, and to specify the controller type and tape length:

SET TQn LOCKED			set unit n write locked

SET TQn WRITEENABLED		set unit n write enabled

SET TQ TK50				set controller type to TK50

SET TQ TK70				set controller type to TK70

SET TQ TU81				set controller type to TU81

SET TQ TKUSER{=n}			set controller type to TK50 with tape

capacity of n MB

User-specified capacity must be between 50 and 2000 MB.  The TUK50 supports the BOOT command.

Regardless of the controller type, individual units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TQn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW TQn CAPAC			show unit n capacity in MB

The TQ controller implements the following special SHOW commands:

SHOW TQ TYPE			show controller type

SHOW TQ RINGS			show command and response rings

SHOW TQ FREEQ			show packet free queue

SHOW TQ RESPQ			show packet response queue

SHOW TQ UNITQ			show unit queues

SHOW TQ ALL				show all ring and queue state

SHOW TQn UNITQ			show unit queues for unit n

The TQ controller implements these registers:

name			size		comments

SA			16		status/address register

S1DAT			16		step 1 init host data

CQBA			22		command queue base address

CQLNT			8		command queue length

CQIDX			8		command queue index

RQBA			22		request queue base address

RQLNT			8		request queue length

RQIDX			8		request queue index

FREE			5		head of free packet list

RESP			5		head of response packet list

PBSY			5		number of busy packets

CFLGS			16		controller flags

CSTA			4		controller state

PERR			9		port error number

CRED			5		host credits

HAT			17		host available timer

HTMO			17		host timeout value

CPKT[0:3]		5		current packet, units 0 to 3

PKTQ[0:3]		5		packet queue, units 0 to 3

UFLG[0:3]		16		unit flags, units 0 to 3

POS[0:3]		32		tape position, units 0 to 3

OBJP[0:3]		32		object position, units 0 to 3

INT			1		interrupt request

ITIME			1		response time for initialization steps

(except for step 4)

QTIME			24		response time for 'immediate' packets

XTIME			24		response time for data transfers

PKTS[33*32]		16		packet buffers, 33W each, 32 entries

Error handling is as follows:

error	     		processed as

not attached	tape not ready

end of file		end of medium

OS I/O error	fatal tape error

## Communications Devices

### DZ11 Terminal Multiplexer (DZ)

The DZ11 is an 8-line terminal multiplexer.  Up to 4 DZ11's (32 lines) are supported.  The number of lines can be changed with the command

SET DZ LINES=n			set line count to n

The line count must be a multiple of 8, with a maximum of 32.

The DZ11 supports three character processing modes, 7P, 7B, and 8B:

mode	input characters		output characters

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default is 8B.

The DZ11 supports logging on a per-line basis.  The command

SET DZ LOG=line=filename

enables logging for the specified line to the indicated file.  The command

SET DZ NOLOG=line

disables logging for the specified line and closes any open log file.  Finally, the command

SHOW DZ LOG

displays logging information for all DZ lines.

The terminal lines perform input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH {-am} DZ <port>		set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  The optional switch -m turns on the DZ11's modem controls; the optional switch -a turns on active disconnects (disconnect session if computer clears Data Terminal Ready).  Without modem control, the DZ behaves as though terminals were directly connected; disconnecting the Telnet session does not cause any operating system-visible change in line status.

Once the DZ is attached and the simulator is running, the DZ will listen for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connection remains open until disconnected by the simulated program, the Telnet client, a SET DZ DISCONNECT command, or a DETACH DZ command.

Other special DZ commands:

SHOW DZ CONNECTIONS 		show current connections

SHOW DZ STATISTICS 		show statistics for active connections

SET DZ DISCONNECT=linenumber	disconnects the specified line.

The DZ11 implements these registers:

name			size		comments

CSR[0:3]		16		control/status register, boards 0 to 3

RBUF[0:3]		16		receive buffer, boards 0 to 3

LPR[0:3]		16		line parameter register, boards 0 to 3

TCR[0:3]		16		transmission control register, boards 0 to 3

MSR[0:3]		16		modem status register, boards 0 to 3

TDR[0:3]		16		transmit data register, boards 0 to 3

SAENB[0:3]		1		silo alarm enabled, boards 0 to 3

RXINT			4		receive interrupts, boards 3 to 0

TXINT			4		transmit interrupts, boards 3 to 0

MDMTCL		1		modem control enabled

AUTODS		1		autodisconnect enabled

The DZ11 does not support save and restore.  All open connections are lost when the simulator shuts down or the DZ is detached.

## CR11 Card Reader (CR)

The card reader (CR) implements a single controller (the CR11) and card reader (e.g., Documation M200, GDI Model 100) by reading a file and presenting lines or cards to the simulator.  Card decks may be represented by plain text ASCII files, card image files, or column binary files.  The CR11 controller is also compatible with the CM11-F, CME11, and CMS11.

Card image files are a file format designed by Douglas W. Jones at the University of Iowa to support the interchange of card deck data.  These files have a much richer information carrying capacity than plain ASCII files.  Card Image files can contain such interchange information as card-stock color, corner cuts, special artwork, as well as the binary punch data representing all 12 columns.  Complete details on the format, as well as sample code, are available at Prof. Jones's site: http://www.cs.uiowa.edu/~jones/cards/.

Examples of the CR11 include the M8290 and M8291 (CMS11).  All card readers use a common vector at 0230 and CSR at 177160.  Even though the CR11 is normally configured as a BR6 device, it is configured for BR4 in this simulation.

The card reader supports ASCII, card image, and column binary format card “decks.”  When reading plain ASCII files, lines longer than 80 characters are silently truncated.  Card image support is included for 80 column Hollerith, 82 column Hollerith (silently ignoring columns 0 and 81), and 40 column Hollerith (mark-sense) cards.  Column binary supports 80 column card images only.  All files are attached read-only (as if the -R switch were given).

ATTACH –A CR <file>			file is ASCII text

ATTACH –B CR <file>			file is column binary

ATTACH –I CR <file>			file is card image format

If no flags are given, the file extension is evaluated.  If the filename ends in .TXT, the file is treated as ASCII text.  If the filename ends in .CBN, the file is treated as column binary.  Otherwise, the CR driver looks for a card image header.  If a correct header is found the file is treated as card image format, otherwise it is treated as ASCII text.

The correct character translation MUST be set if a plain text file is to be used for card deck input.  The correct translation SHOULD be set to allow correct ASCII debugging of a card image or column binary input deck.  Depending upon the operating system in use, how it was generated, and how the card data will be read and used, the translation must be set correctly so that the proper character set is used by the driver.  Use the following command to explicitly set the correct translation:

SET TRANSLATION={DEFAULT|026|026FTN|029|EBCDIC}

This command should be given after a deck is attached to the simulator.  The mappings above are completely described at http://www.cs.uiowa.edu/~jones/cards/codes.html.  Note that DEC typically used 029 or 026FTN mappings.

DEC operating systems used a variety of methods to determine the end of a deck (recognizing that 'hopper empty' does not necessarily mean the end of a deck.  Below is a summary of the various operating system conventions for signaling end of deck:

RT-11:	12-11-0-1-6-7-8-9 punch in column 1

RSTS/E:	12-11-0-1 or 12-11-0-1-6-7-8-9 punch in column 1

RSX:		12-11-0-1-6-7-8-9 punch

VMS:		12-11-0-1-6-7-8-9 punch in first 8 columns

TOPS:		12-11-0-1 or 12-11-0-1-6-7-8-9 punch in column 1

Using the AUTOEOF setting, the card reader can be set to automatically generate an EOF card consisting of the 12-11-0-1-6-7-8-9 punch in columns 1-8.  When set to CD11 mode, this switch also enables automatic setting of the EOF bit in the controller after the EOF card has been processed.  [The CR11 does not have a similar capability.]  By default AUTOEOF is enabled.

SET CR AUTOEOF

SET CR NOAUTOEOF

The default card reader rate for the CR11 is 285 cpm.  The reader rate can be set to its default value or to anywhere in the range 200 to 1200 cpm.  This rate may be changed while the unit is attached.

SET CR RATE={DEFAULT|200 to 1200}

It is standard operating procedure for operators to load a card deck and press the momentary action RESET button to clear any error conditions and alert the processor that a deck is available to read.  Use the following command to simulate pressing the card reader RESET button,

SET CR RESET

Another common control of physical card readers is the STOP button.  An operator could use this button to finish the read operation for the current card and terminate reading a deck early.  Use the following command to simulate pressing the card reader STOP button.

SET CR STOP

The simulator does not support the BOOT command.  The simulator does not stop on file I/O errors.  Instead the controller signals a reader check to the CPU.

The CR controller implements these registers:

name			size		comments

BUF			8		ASCII value of last column processed

CRS			16		CR11 status register

CRB1			16		CR11 12-bit Hollerith character

CRB2			16		CR11 8-bit compressed character

CRM			16		CR11 maintenance register

CDST			16		CD11 control/status register

CDCC			16		CD11 column count

CDBA			16		CD11 current bus address

CDDB			16		CD11 data buffer, 2nd status

BLOWER		2		blower state value

INT			1		interrupt pending flag

ERR			1		error flag (CRS<15>)

IE			1		interrupt enable flag (CRS<6>)

POS			32		file position - do not alter

TIME			24		delay time between columns

# Symbolic Display and Input

The VAX simulator implements symbolic display and input.  Display is controlled by command line switches:

-a,-c			display as ASCII data

-m			display instruction mnemonics

-p			display compatibility mode mnemonics

-r			display RADIX50 encoding

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		ASCII characters (determined by length)

" or -c		ASCII string (maximum 60 characters)

-p			compatibility mode instruction mnemonic

alphabetic		instruction mnemonic

numeric		octal number

VAX instruction input uses standard VAX assembler syntax.  Compatibility mode instruction input uses standard PDP-11 assembler syntax.

The syntax for VAX specifiers is as follows:

syntax		specifier	displacement	comments

#s^n, #n		0n		-		short literal, integer only

[Rn]			4n		-		indexed, second specifier follows

Rn			5n		-		PC illegal

(Rn)			6n		-		PC illegal

-(Rn)			7n          -		PC illegal

(Rn)+			8n		-

#i^n, #n		8F		n		immediate

@(Rn)+		9n		-

@#addr		9F		addr		absolute

{+/-}b^d(Rn)	An		{+/-}d	byte displacement

b^d			AF		d - PC	byte PC relative

@{+/-}b^d(Rn)	Bn		{+/-}d	byte displacement deferred

@b^d			BF		d - PC	byte PC relative deferred

{+/-}w^d(Rn)	Cn		{+/-}d	word displacement

w^d			CF		d - PC	word PC relative

@{+/-}w^d(Rn)	Dn		{+/-}d	word displacement deferred

@w^d			DF		d - PC	word PC relative deferred

{+/-}l^d(Rn)	En		{+/-}d	long displacement

l^d			EF		d - PC	long PC relative

@{+/-}l^d(Rn)	Fn		{+/-}d	long displacement deferred

@l^d			FF		d - PC	long PC relative deferred

If no override is given for a literal (s^ or i^) or for a displacement or PC relative address (b^, w^, or l^), the simulator chooses the mode automatically.